# 3GPP TSG-RAN WG4 Meeting #113                                 R4-2418488

Orlando, USA, 18th – 22rd November, 2024

Source:	ZTE Corporation

Title:	Discussion on RRM requirements for R18 maintenance NTN

Agenda item:	5.21.3

Document for:	Approval

Introduction

At RAN 98-e meeting the revised WI “NR NTN (Non-Terrestrial Networks) enhancements” [1] was approved.  In the last meeting, we discussed the location-based cell re-selection in earth moving cell NTN deployments and in #108 meeting we will specify the requirements for NTN-NTN cell re-selection enhancements and the NTN-TN cell re-selection enhancements, what we discussed before were captured in [2].

Discussion

In previous meeting, RAN4 has already reached the following agreement on Idle/Inactive/Connected mode mobility enhancements.

2.1 Idle/Inactive mode mobility enhancements

### Issue 4-1: TN to NTN cell reselection Agreement (online): - For NTN incapable UE, if both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode, the existing TN-to-TN cell reselection requirements apply. FFS: - For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied. ### Issue 4-2: NTN to TN cell reselection FFS: - Clarify that the requirements for NR NTN to LTE TN inter-RAT cell re-selection is defined as NEUTRA_carrier * Tdetect/measure/evaluate,EUTRAN, where - The parameter NEUTRA_carrier is the number of configured E-UTRA carriers indicated in the neighbour frequency list by serving cell, except for the frequency carrier where there is no coverage of that frequency based on the provide TN cell coverage information and UE GNSS position information. - Tdetect/measure/evaluate,EUTRAN is the inter-RAT E-UTRAN TN cell re-selection requirement defined in Table 4.2.2.5-1 in TS 38.133

TN to NTN cell re-selection

For this part, we have already introduced GNSS ON when NTN capable UE which stays in TN cell and would like to reselect to NTN cell. We find that if the UE is NTN incapable, even if both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode, the existing TN-to-TN cell reselection requirements apply, then GNSS will always be OFF. However, one thing is raised when UE turn on the GNSS switch from OFF to ON. Since it will cause time consuming and the whole delay for cell re-selection is increased if the mobility/cell re-selection is triggered for first time . Then we consider to select some conditions to help UE decide whether the GNSS ON shall be kept or not, even the UE stays in TN coverage. One condition is “For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied”. From my perspective, this can be applied. We can see the following spec:

TS 38.133 4.2C.2.4 If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the distance between UE and serving cell reference location or serving cell moving reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2C.2.9. If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, or the distance between UE and serving cell reference location or serving cell moving reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 50 m when referenceLocation is configured by the network or [80]m when movingReferenceLocation is configured by the network. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

We find whatever the quality of current serving cell is good or not, if the network configures high priority frequency to UE, the UE will search for high priority at least every Thigher_priority_search. We also know that the high priority carrier which contains NTN cells, then UE shall keep GNSS ON even if UE stays in the TN coverage. Some companies think this is up to UE implementation, but we do not think this can reduce the time consuming on switching GNSS OFF to ON for the first time.

Proposal 1: If UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied.

NTN to LTE TN cell re-selection

Actually, we have already defined the related cell re-selection requirements for this scenario in TS38.133 4.2C.2.11 and the definition of each parameters has been clarified in current spec. Thus, this is not an issue for us.

Observation 1: RAN4 has already defined the related cell re-selection requirements for this scenario in TS38.133 4.2C.2.11 and the definition of each parameters has been clarified in current spec.

2.2 Connected mode mobility enhancements

In previous meeting, we reached the agreements as below:

### Issue 5-2-S: Soft’ Satellite switch FFS: - RAN4 to not consider PDD reporting between serving and target satellites involved in the satellite switching without PCI change. FFS: - To enhance scheduling restriction in soft satellite switching, consider modifying the capability rule such that: if UE indicates to support soft satellite switching without PCI change, it also means UE supports both simultaneousRxDataSSB-DiffNumerology and parallelMeasurementWithoutRestriction-r17.

Actually, in previous meeting, we deem that we have already reached the agreements on whether to consider PDD reporting between serving and target satellites. RAN2 has already reached the agreements as below:

RAN2 understands that the NW is not expected to configure PDD reporting between serving and target satellites involved in the satellite switch

Therefore, RAN4 shall not consider PDD reporting between serving and target satellites involved in the satellite switch.

Proposal 2: RAN4 shall not consider PDD reporting between serving and target satellites involved in the satellite switch.

For the second issue, we deem that only parallelMeasurmentWithoutRestriction-r17 is enough for no scheduling restriction. We can see the following table from TS38.306:

| parallelMeasurementWithoutRestriction-r17 Indicates whether the UE supports measurements on cells belonging to different satellites as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release. If not reported, for measurements in parallel with normal operation of serving cell scheduling restrictions shall apply. | Band | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |

This IE has already indicates parallel measurement without scheduling restrictions, no need to also add simultaneousRxDataSSB-DiffNumerology.

Proposal 3: No need to add simultaneousRxDataSSB-DiffNumerology.

Conclusion

In this paper we provided our views on RRM requirements on R18 NR NTN enhancement.

Proposal 1: If UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied.

Observation 1: RAN4 has already defined the related cell re-selection requirements for this scenario in TS38.133 4.2C.2.11 and the definition of each parameters has been clarified in current spec.

Proposal 2: RAN4 shall not consider PDD reporting between serving and target satellites involved in the satellite switch.

Proposal 3: No need to add simultaneousRxDataSSB-DiffNumerology.

- References

[1] RP-223534, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales.

[2] R4-2414100, WF on NR NTN enhancement RRM requirements, Qualcomm.