3GPP TSG-RAN WG4 Meeting #113	R4-2418519

# Orlando, FL, USA, 18 – 22 November 2024

Source: 	KDDI

Title: 	Discussion on UE RF requirement for non-collocated Type 4a4b

Agenda Item:	7.5.2.1

Document for:	Discussion

- 1. Introduction

In RAN#105 meeting, the revised WID for intra-band non-collocated EN-DC/NR-CA is approved [1]. The WI’s core part is focusing on specifying UE RF/RRM requirements for Type-4 UE capability for FWA and clarifying the UE behaviour for Type-4 supported UE. Some discussions were organized in the last RAN4#112bis meeting and the agreements were captured in [2]. We try to solve remaining issues in this contribution.

- 2. Discussion

Regarding UE RF requirements, RAN4 conclude the WF[2] for Rel-19 Type 4 UE in the last meeting as follows.

< Issue 1-1: How to capture the power imbalance and REFSENS requirements > Way forward: - In the last meeting RAN4 agreed no rush to make decision on Rel-19 CR. Discuss it as Rel-19 big CR. - The CR leader take companies’ inputs of R4#112-bis meeting into account when drafting actually.

On the other hand, RAN4 conclude the other WF[3] for Rel-18 Type 2 UE in the last meeting as follows.

- Topic #3: Other aspects (incl. clarification of contiguous LTE CCs) - <Issue 3-1: UE Type definition for EN-DC> Way Forward: - The following table is a starting point, and continue further discussion the following table in the next meeting. - The table will be added into TS 38.101-3: 7.10B power imbalance for DC in FR1. - Table 1: EN-DC UE Capability types [UE Capability Type]; Type description; Dedicated UE capability. Type 1; It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133.; N/A. Type 2; The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133; interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18. Type 4; The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133; TBD. - <Issue 3-2: UE Type definition for NR-CA> Way Forward: - The following table is a starting point, and continue further discussion the following table in the next meeting. - The table will be added into TS 38.101-1: 7.10A Power imbalance for CA. - Table 2: NR-CA UE Capability types [UE capability types]; Type description; dedicated UE capability. Type 1; It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133; N/A. Type 2; The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133; intraBandNR-CA-non-collocated-r18. Type 4; The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133; TBD.

Firstly, RAN4 need to conclude the discussion on WF[3] for Rel-18. And then, RAN4 can discuss UE RF requirement for Rel-19.

Proposal 1: Conclude the discussion on WF R4-2417114 on Rel-18 clarifications of RF requirements for different UE types.

Proposal 2: No rush to make decision on Rel-19 CR. Discuss it as Rel-19 big CR.

- 3. Conclusion

Proposal 1: Conclude the discussion on WF R4-2417114 on Rel-18 clarifications of RF requirements for different UE types.

Proposal 2: No rush to make decision on Rel-19 CR. Discuss it as Rel-19 big CR.

- 4. Reference

[1] RP-230838 Revised WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), KDDI, RAN#105

[2] R4-2417115 WF on rest issues for intra-band non-collocated EN-DC/NR-CA, KDDI, RAN4#112bis

[3] R4-2417114 WF on Rel-19 non-collocated scenario, KDDI, RAN4#112bis