3GPP TSG-RAN WG4 Meeting #113		R4-2418554

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.19.3.1

Source:	Apple

Title:	On RRM Requirements for UE initiated beam management

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#112bis the RRM requirements for UE initiated beam management was discussed and WF [1] was approved. In this contribution we present our views on RRM core requirements for UE initiated beam management.

# 2. Discussion

In [1] the following agreements were made for RRM core requirements with UE initiated beam management:

Issue 1-1: How to define Event triggered measurement reporting delay? < Agreement> - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air. - Ending point is the point when the UE transmits a [first PUCCH] triggered by the event. - “[first PUCCH]” can be updated based on further RAN1/2 agreement. Issue 1-3: Applicability of Event triggered measurement reporting delay requirements? < Agreement> - Applicable for all events agreed in RAN1/2 - Applicable to both Mode A and B - Note: applicable to the event/mode supported by UE Issue 1-7: Whether to support CSI-RS based L1-RSRP for aperiodic? <Agreement > - RAN4 to define CSI-RS based UEIBM L1-RSRP requirements for periodic CSI-RS. CSI-RS based UEIBM L1-RSRP requirements for semi-persistent CSI-RS are FFS pending on RAN1 further agreement to support semi-persistent CSI-RS.

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay?

<Way forward >

- Option 1:

- If multiple beams are measured with different measurement period, RAN4 to discuss how to consider the total measurement time.

- Option 2:

- It is proposed to take measurement period defend for Rel-18 LTM as baseline

- Option 3:

- RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirements for UE-initiated/event-driven beam management

- Option 4:

- Use legacy measurement period for TL1-RSRP_Measurement_Period_SSB for SSB based intra-cell , inter-cell, TL1-RSRP_Measurement_Period_CSI-RS for CSI RS based.

- Option 4a:

    - Remove Treport or set to 0 in legacy requirements.

- Option 4b:

    - Remove considerations for HST enhancements/ scenarios

    - From legacy measurement period, for new beam report further discuss if M is needed or can be removed/assumed 1.

    - From legacy measurement period, for new beam report further discuss if P can be the same or modified for simplification.

- Option 4c:

    - For FR2, the T_L1_measurement_period is scaled by N.

- Other options are not precluded.

The measurement delay can be defined for L1-RSRP based new beam measurements for the following:

- Intra-cell SSB

- Inter-cell SSB

- CSI-RS (inter-cell or intra-cell is not distinguishable)

The delay requirements can be based on existing requirements for L1-RSRP measurement as a baseline and modified for new beam measurement. Taking the L1-RSRP measurement delay for SSB as an example, the following need to be removed or further discussed

Treport: removed since there is no reporting period configured for UE triggered report

M: Number of samples – FFS if this should be included as configuration for new beams for measurement is not known yet.

P: Measurement sharing factor – FFS if existing can be used or modified for simplification

Considerations for HST scenarios/ enhancements: removed

Use legacy measurement period for TL1-RSRP_Measurement_Period_SSB for SSB based intra-cell , TL1-RSRP_Measurement_Period_SSB_CDP for SSB inter-cell, TL1-RSRP_Measurement_Period_CSI-RS for CSI RS based with following modifications:
Remove Treport
Remove considerations for HST enhancements/ scenarios
M = 1 (further discuss if M is needed)
further discuss if P can be the same or modified for simplification

Issue 1-4: How to consider the time window of triggering event determination in Event triggered measurement reporting?

<Way forward >

- Option 1:

- RAN4 needs to consider the impact of triggering window defined in RAN1.

- Option 2:

- It is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M).

- It is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M)

- Option 3:

- No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements

- Option 4:

- Discuss the necessity of filtering based on the two trigger-event detection solutions approved in RAN1.

  - For the basic solution, one-shot measurement is defined by RAN1. No room to consider filtering.

  - For the optional solution, multi-shot measurement with at least M instance is defined by RAN1. Filtering is feasible.

- Option 5:

- RAN4 to discuss how to define one Event-2 instance for following Event-2 case:

  - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

- Option 6:

- Wait further RAN1 progress.

- Other options are not precluded.

RAN4 should define requirements based on 1 shot measurement without additional filtering. If RAN1 defines the trigger window is defined based on multiple events, it would be based on additional signaling/ capability. Hence requirements without considering time to trigger should be defined as baseline or minimum requirements.

Event triggered measurement delay is defined without any filtering (as single shot measurement) and without time window to trigger as baseline.

Issue 1-5: How to consider UL resource in Event triggered measurement reporting?

<Way forward >

- Option 1:

- RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting.

- Option 2:

- The beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

- No need to differentiate mode A and mode B

- Option 3:

- Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B

- Option 4:

- To determine how to count for the uncertainty in acquiring UL resource for UEIBR, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not (for Mode-A).

- Other options are not precluded.

Figure 1: Event triggered Reporting delay timeline

After the trigger condition is met, the reporting delay consist of the following:

- Time to RS/ new beam – Tfirst-RS-NB

- L1-RSRP measurement delay – TL1-RSRP-NB

- Time to first PUCCH – Tfirst-PUCCH or TPUCCH

The total reporting delay for event triggered beam reporting is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH.

The total reporting delay for event triggered beam reporting is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH.

Issue 1-6: Whether to define requirements if end of measurement period is before the UL resource?

<Way forward >

- Option 1:

- RAN4 to discuss whether and how to define requirement when measurement period is shorter than the UL resource waiting time

- Other options are not precluded.

The NW is expected to configure PUCCH resources for resource request or indication periodically. The L1-RSRP measurement time is known, and the least possible measurement time is the resource periodicity. It is reasonable expectation that NW will ensure that the UL resources for beam report indication or request are configured with shorter periodicity that the measurement period.

It is reasonable expectation that NW will ensure that the UL resources for beam report indication or request are configured with shorter periodicity that the measurement period.

Issue 1-8: CSI-RS based L1-RSRP for inter-cell BM?

<Way forward >

- Option 1:

- RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case

- Option 2:

- For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary:

  - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

  - The prerequisite on L3 measurement on NSC.

- Option 3:

- RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario in FR2.

- RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Other options are not precluded.

The CSI_RS resources configured for new beam measurement cannot be identified as from serving cell or cell with different PCI. For event triggered reporting based on CSI-RS, the requirements are defined for CSI-RS without specifying inter-cell or intra-cell. The measurement delay can be based on the legacy CSI-RS based L1-RSRP measurement as proposed in proposal #1.

- CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell.

CSI-RS based L1-RSRP measurement for new beam is defined based on legacy L1-RSRP measurement without distinguishing inter-cell or intra-cell. The requirements are applicable to both.

Issue 1-9: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- Option 1:

- RAN4 should investigate whether any new required TCI state switch delays need to be defined or existing requirements are sufficient

- Option 2:

- Wait further RAN1/RAN2 progress.

- Option 3:

- No need to define new TCI state switching delay requirements. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.This can be revisited if the RS in the RS resource set can be updated by MAC-CE in further RAN1 conclusion.

- Other options are not precluded.

After the UE sends event triggered beam report, the network could send a TCI state switch command to the new beam. The existing TCI state switching delay requirements for unified TCI state or legacy DL active TCI state would still be appliable.

No need to define new TCI state switching delay requirements. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM?

<Way forward >

- Option 1:

- Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM.

- Option 2:

- No impact of UE triggered beam report to known/unknown conditions for TCI state.

- The known/unknown condition would be determined by the same criteria as legacy.

- Other options are not precluded.

The TCI state known/unknown condition is relevant only when TCI state switch is activated or indicated for PDCCH/ PDSCH ; PUSCH/PUCCH/SRS in case of joint TCI state. When the UE receives TCI state switch (MAC-CE or DCI based) to a TCI state, the known/unknown condition would be determined by the same criteria as legacy. The L1-RSRP report should have been received within 1280 ms of the TCI state switch command, and it doesn’t matter if the L1-RSRP report was based on UE triggered beam report or legacy measurement report.

- No impact of UE triggered beam report to known/unknown conditions for TCI state.

- The known/unknown condition would be determined by the same criteria as legacy.

No impact of UE triggered beam report to known/unknown conditions for TCI state. The known/unknown condition would be determined by the same criteria as legacy.

Issue 1-11: Whether to enhance beam sweeping in event triggered L1 beam reporting?

<Way forward >

- Option 1:

- Discussions on beam sweeping enhancements should be postponed till the baseline requirement for delay and measurement period have taken place

- Option 2:

- Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting. No enhancement.

- Option 3:

- Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

- Other options are not precluded.

Faster Tx and Rx beam refinement is not in the scope of this WI. Keeping in mind the objectives of this WI, RAN4 shall define requirements associated with UE/ event triggered beam reporting with legacy assumptions for beam sweeping.

- Faster RX, TX beam refinement is not in scope of this WI.

Use legacy assumptions for beam sweeping for requirements for UE/ event triggered beam reporting.

Issue 1-12: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

<Way forward >

- Option 1:

- The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements.

- Option 2:

- For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

- For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements.

- Option 3:

- Discuss the accuracy for the two trigger-event solutions respectively.

  - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement;

  - For the optional trigger-event detection, reusethe existing L1-RSRP accuracy requirement.

- Other options are not precluded.

The L1-RSRP measurement is the same as legacy whether it is for beam report configured or based on UE/event triggered beam report. With the assumption that same one-shot measurement for RSRP is employed for new beams as legacy L1-RSRP measurement, the accuracy requirements should remain the same. Hence there is no need to define new accuracy requirements for L1-RSRP measurements for UEIBM.

Legacy accuracy requirements for L1-RSRP shall be applicable for UEIBM.  No new accuracy requirements are necessary.

Issue 1-13: Capabilities for Support of Event Triggering and Reporting Criteria?

<Way forward >

- Option 1:

- Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

- Other options are not precluded.

For RAN1 led WIs and features developed RAN1 would typically define associated capabilities. It would be premature to define any UE capabilities related to supported number of UE-initiated/event-driven beam reporting in RAN4.

Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

Issue 1-14: Compatibility of capabilities with LTM

<Way forward >

- Option 1:

- Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

- Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

- Other options are not precluded.

RAN4 has defined several L1 based measurements, and some are UE capability based. But we have not defined joint capability across different measurements. There doesn’t seem to be a strong necessity to combine the capability across different L1 measurements.

Do not combine capabilities across different L1 measurements.

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

- Use legacy measurement period for TL1-RSRP_Measurement_Period_SSB for SSB based intra-cell , TL1-RSRP_Measurement_Period_SSB_CDP for SSB inter-cell, TL1-RSRP_Measurement_Period_CSI-RS for CSI RS based with following modifications:
Remove Treport
Remove considerations for HST enhancements/ scenarios
M = 1 (further discuss if M is needed)
further discuss if P can be the same or modified for simplification

Event triggered measurement delay is defined without any filtering (as single shot measurement) and without time window to trigger as baseline.

The total reporting delay for event triggered beam reporting is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH.

It is reasonable expectation that NW will ensure that the UL resources for beam report indication or request are configured with shorter periodicity that the measurement period.

- CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell.

CSI-RS based L1-RSRP measurement for new beam is defined based on legacy L1-RSRP measurement without distinguishing inter-cell or intra-cell. The requirements are applicable to both.

No need to define new TCI state switching delay requirements. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

- No impact of UE triggered beam report to known/unknown conditions for TCI state.

- The known/unknown condition would be determined by the same criteria as legacy.

No impact of UE triggered beam report to known/unknown conditions for TCI state. The known/unknown condition would be determined by the same criteria as legacy.

- Faster RX, TX beam refinement is not in scope of this WI.

Use legacy assumptions for beam sweeping for requirements for UE/ event triggered beam reporting.

Legacy accuracy requirements for L1-RSRP shall be applicable for UEIBM.  No new accuracy requirements are necessary.

Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

Do not combine capabilities across different L1 measurements.

# Reference

- R4-2416856, “WF on RRM requirements for NR_MIMO_Ph5_Part1”, Samsung.