# 3GPP TSG-RAN WG4 Meeting #113	R4-2418566

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.4.3

Source:	Apple

Title:	On RRM core for less than 5MHz Phase 2

Document for:	Discussion

# Introduction

The WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2 [1] has been agreed in RAN4#112bis, and the open issues have been summarized in the topic summary for [112bis][205] NR_FR1_lessthan_5MHz_BW_Ph2 [2].

In this contribution, we continue the discussion on the RRM requirement for the WI of less than 5MHz Phase 2.

# RRM for CA/DC in less than 5MHz

Issue 1-1-2: Measurement requirements for deactivated SCC:

The time index detection extension of deactivated SCC could be same as the intra-frequency case since the baseline sample number is also same between intra-frequency (Table 9.2.5.1-3) and deactivated SCell (Table 9.2.5.1-6). We agree with the option 1 in the last WF that,

Proposals 1:

  - Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table.

| DRX cycle | TSSB_time_index_Scell |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

Issue 1-1-3: Whether intra-band CA is in the scope:

As described in WID, we didn’t see any scenario required CA/DC have two or more serving cells with less than 5MHz, that is, RAN4 only needs to consider following scenarios:

- CA with 3MHz PCell + 5MHz or 10MHz SCell(s)

- CA with 5MHz or 10MHz PCell + one 3MHz SCell

- DC with 3MHz PCell + 5MHz or 10MHz PSCell

- DC with 5MHz or 10MHz PCell + 3MHz PSCell

Since the detailed band combination of CA/DC needs to wait RF conclusions, it’s better to decide the corresponding RRM requirement after RF session has concrete conclusions, e.g., whether or not we need to consider intra-band CA case. So far we didn’t see intra-band scenario agreed in RF session.

Proposal 2: RAN4 to decide the corresponding RRM requirement for intra-band CA after RF session has concrete conclusions.

Issue 1-3-1a: Allowing 12PRB SSB bandwidth off sync raster:

Issue 1-3-1b: NW indication on PBCH BW in MO configuration:

Issue 1-3-1c: NW indication on PBCH BW in HO command/SCell addition command/SIB4 message:

In R18 less than 5MHz, we had some agreements in RAN4#109 (R4-2321629) on whether network needs to provide the bandwidth information to UE during MO configuration or HO, and the main reason is because CD-SSB PCell has to be on sync raster and R18 only considered single carrier case for less than 5MHz.

## Sub-topic 1-2 Assistance information for PBCH is 12 or 20 PRBs Agreement: There is no need not include information regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command.

However, if now SCell can also use 12 PRB SSB bandwidth, as CD-SSB of SCell can be off sync raster, then it would be necessary to provide assistance information regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command (HO with SCell or PSCell), otherwise, UE may assume wrong PBCH DMRS bandwidth and fail the SSB index reading.

In last RAN4#112bis meeting, RF session had agreements as following that,

Agreement - Update RAN4 #112 agreement as follows: - SCell with 15 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency position same as defined in TS 38.101-1 Table 5.4.3.1-2 - For band n100 - SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3 - SCell with 20 PRB transmission bandwidth configuration within 5 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3 - Capture corresponding restrictions as notes in TS 38.101-1 TS 38.101-1 Table 5.4.3.1-2 and Table 5.4.3.1-3

For all the transmission BW cases, the SSBs are designed only on the sync raster based on the above agreements. Thus, there is no need to introduce an indication from network to UE regarding whether the PBCH is 12 or 20 PRBs.

Proposal 3: No need to introduce network indication on PRB number for PBCH transmission.

# Conclusion

In this contribution, we discuss the RRM requirement for co-located and non-co-located inter-band NR CA/DC UE with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band.

Proposals 1:

  - Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table.

| DRX cycle | TSSB_time_index_Scell |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

Proposal 2: RAN4 to decide the corresponding RRM requirement for intra-band CA after RF session has concrete conclusions.

Proposal 3: No need to introduce network indication on PRB number for PBCH transmission.

# References

[1] R4-2416849, WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2, Intel, RAN4 #112bis

[2] R4-2415652, Topic summary for [112bis][205] NR_FR1_lessthan_5MHz_BW_Ph2, Intel, RAN4#112bis