3GPP TSG-RAN WG4 Meeting #113	 R4-2418584

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	7.15.3

Source: 	Apple

Title:	Discussion on fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

Fast SCell activation for UE supporting Rel-18 EMR has been widely discussed in previous RAN4 meetings. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

Issue 1-1-1: Applicability of fast SCell activation delay requirements

Proposals:

- Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK)

  - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured.

- Option 2: (OPPO, Nokia, Samsung, Ericsson)

  - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

- Option 2a: (OPPO)

  - Consider the following two approaches to support the case:

    - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or

    - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration.

- Option 2b: (Samsung)

  - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured

  - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

We continue supporting option 1. Because 1) network is expected to configure the timer so that UE can know clearly NW would like to enable this feature. 2) if a measurement result was obtained long time ago, it may not reliably provide accurate time and frequency info. 3) a R18 UE may choose to send report even if the timer is not configured. The report can be used to help network to select which carrier(s) to add as candidate SCell for this UE. However, the measurement may be performed quite a long time ago so that target cell cannot be considered as known from cell search and tracking point of view. In other words, fast SCell activation requirements cannot always be guaranteed. If we request this UE to meet fast SCell activation requirements, UE may choose to not report anything.

Observation 1: including the case where timer for validity check is not configured into RAN4 scope has impact on UE reporting criteria, i.e. UE which chooses to report in R18 may choose not to report in R19 since fast SCell activation requirements cannot always be guaranteed.

Proposal 1: not include the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured.

The most important issue is how to update RRM requirements:

Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting

Agreement:

- RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception.

  - For FR1,

    - [Y] equals to [5s] for direct SCell activation

    - FFS: Y for other SCell activation case.

  - For FR2,

    - Reuse the existing value of Y in direct SCell activation

    - FFS: Y for other SCell activation case.

Agreement:

- the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting).

  - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

  - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

For continued discussion in next meeting:

- For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured:

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with [2] SSB samples.

  - For FR2:

    - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X]

      - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW)

    - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion.

    - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1

      - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured)

    - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT)

      - K = 0 or 1 if  when the “validityduration” is smaller than X

      - K = 1 or 2 otherwise

    - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung)

      - K = 0 or 1 if  when the “validityduration” is applied with smaller than X

      - K = 1 or 2 otherwise

      - FFS how to test

For progress, we would like to confirm that for FR1 RAN4 can “use existing SSB based known SCell activation delay requirement with [2] SSB samples”. For FR2, the most promising option is option 3. We continue supporting option 3 in general and the condition that the validity timer is configured. However, there are still some details that we need to pay attention, e.g. beam level measurement shall be included, we still need to distinguish periodic CSI-RS and semi-persistent CSI-RS.

Note that in this sense we probably don’t need to update definition of known and unknown. We can add this as a new scenario in SCell activation requirements. Besides, we would like to point out that this applies for both RRC based direct SCell activation at SCell addition and MAC-CE based single SCell activation.

Proposal 2: For single direct SCell activation at SCell addition and MAC-CE based single SCell activation, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured:

- For FR1:

  - Use existing SSB based known SCell activation delay requirement with [2] SSB samples, i.e. Tactivation_time is TFirstSSB_MAX + Trs + 5ms

- For FR2:

  - Assuming beam level measurement is included in the report, Tactivation_time is:

    - If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

      - 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

    - If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

      - 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + max{(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

The last issue is about new indication from UE to network:

Issue 1-2-4: Whether the indication to network is needed

Proposals

- Option 1: (CATT, Apple, Samsung)

  - Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation.

- Option 2: (Ericsson, Nokia)

  - RAN4 to introduce a UE indication to the network. The fast or not definition can refer to the known/unknown condition that will be defined for fast Scell activation via EMR.

We don’t quite understand why additional indication is necessary on top of valid EMR report in R18. The objective of this work item is just to study and, if feasible, to reduce the SCell activation delay with valid EMR reporting, not any new indication.

Proposal 3: Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast SCell activation.

# Conclusion

In this contribution, we provide discussion on fast SCell activation for UE supporting Rel-18 EMR. After discussion, the following conclusions are provided:

Observation 1: including the case where timer for validity check is not configured into RAN4 scope has impact on UE reporting criteria, i.e. UE which chooses to report in R18 may choose not to report in R19 since fast SCell activation requirements cannot always be guaranteed.

Proposal 1: not include the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured.

Proposal 2: For single direct SCell activation at SCell addition and MAC-CE based single SCell activation, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured:

- For FR1:

  - Use existing SSB based known SCell activation delay requirement with [2] SSB samples, i.e. Tactivation_time is TFirstSSB_MAX + Trs + 5ms

- For FR2:

  - Assuming beam level measurement is included in the report, Tactivation_time is:

    - If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

      - 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

    - If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

      - 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + max{(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

Proposal 3: Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast SCell activation.

# Reference

- R4-2416854, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT