3GPP TSG-RAN WG4 Meeting #113	 R4-2418594

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	5.23.1

Source: 	Apple

Title:	Discussion on RAN2 LS on R18 LTM

Document for:	Discussion

# Introduction

RAN4 received two LSes [1~2] from RAN2 on R18 LTM, respectively one for fast RRC processing for LTM and the other one for UE capabilities for inter-frequency L1 measurement for LTM. In this contribution, we provide our views on these two issues.

# Discussion

## Fast RRC processing for LTM

Informatively, RAN2 LS is duplicated here:

# 1 Overall description According to the RAN4 feature list, two components are included in the LTM fast RRC processing capability ltm-FastProcessingConfig-r18: - maxNumberStoredConfigCells, indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18, represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133. Currently, only the number of LTM candidate configurations is visible in LTM-config, while the number of SpCell/SCell(s) in LTM candidate configurations is not visible until the UE decodes LTM candidate configurations. RAN2 would like to know the answers for the following three questions: - As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations? - Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration? - Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

There are three questions that RAN4 is expected to answer.

Q1: As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations?

The condition listed in Q1 is also one of the conditions to apply fast RRC processing as such defined in RAN4 spec.

TLTM-RRC-processing = 0, if the UE supports ltm-FastProcessingConfig-r18 capability, and: - The number of candidate cells in the LTM candidate cell configuration does not exceed maxNumberConfigs-r18 and the number of the configured serving cells and candidate cells does not exceed maxNumberStoredConfigCells-r18, or - The following conditions are fulfilled for the target cell: - UE has received LTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before the LTM cell switch command, and/or - UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before the LTM cell switch command, where NT,2 is defined in section 8.1 of TS 38.213 [3], and - The number of candidate cells for which TCI state(s) were activated or PDCCH order was received before the cell switch command does not exceed maxNumberConfigs-r18, and the number of serving cells and candidate cells for which TCI state(s) were activated or PDCCH order was received before the cell switch does not exceed maxNumberStoredConfigCells-r18. - [FFS further conditions]- [FFS: SCell is not part of the cell switch.] Otherwise TLTM-RRC-processing = 10 ms.

Therefore, answer to Q1 is YES.

Proposal 1: answer to Q1 is YES, i.e. UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations under this condition.

Q2: Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration?

From UE implementation point of view, it is feasible to perform early ASN.1 decoding of an LTM candidate configuration without knowing the number of SpCells and SCells within the configured LTM candidate configurations. However, if the actual number of cells within the configured LTM candidate configuration exceeds the corresponding UE capability, UE may fail to do it for all cells. As minimum requirement, we have to assume UE still needs time to perform ASN.1 decoding when LTM cell switch command is received. In fact, that is also aligned with existing RAN4 requirements, i.e. TLTM-RRC-processing = 10ms if number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the corresponding UE capability.

Proposal 2: answer to Q2: it is feasible for UE to perform early ASN.1 decoding of an LTM candidate configuration without knowing the number of SpCells and SCells within the configured LTM candidate configurations. However, if the actual number of cells within the configured LTM candidate configuration exceeds the corresponding UE capability, NW has to assume UE may not have done early decoding, i.e. TLTM-RRC-processing during cell switch cannot be 0 as minimum requirements.

Q3: Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

This is a general issue. Once UE reports a capability for a functionality, UE can guarantee the functionality can work well (meeting corresponding requirements) as long as network configuration does NOT exceed the UE capability. Back this specific issue, if NW expects UE can successfully perform early ASN.1 decoding, number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations is expected NOT to exceed the UE reported maxNumberStoredConfigCells.

Proposal 3: if NW expects UE can successfully perform early ASN.1 decoding, number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations is expected NOT to exceed the UE reported maxNumberStoredConfigCells.

We also provide a drafted LS to RAN2 in Annex.A.

## UE capabilities for inter-frequency L1 measurements for LTM

Key message from the LS is duplicated here for information:

The UE capability for inter-frequency L1 measurements for LTM (FG 45-1a) is specified as: interFreqL1-MeasConfig-r18 Indicates whether UE supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s). This capability signalling comprises of the following parameters: - supportedMaxIntraInterFreqCellsConfig-r18 indicates the maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqCellsPerReport-r18 indicates maximum number of candidate cells in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqBeamsPerCellReports-r18 indicates maximum number of candidate beams per candidate cell in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; - supportedMaxIntraInterFreqBeamsReports-r18 indicates maximum number of candidate cells beams in total across all cells in one report where a SSBRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18.; BC; No; N/A; N/A. RAN2 understands that, when the UE indicates support of this capability for a BC and the UE is currently operating in this BC (or a subset of it), the SSBs of candidate cells on which the UE can perform inter-frequency L1 measurements according to this UE capabilities can be in any band on which the UE supports LTM. For example, if the UE has serving cells in bands A and B, indicates support of inter-frequency L1 measurements in a BC (A, B, C) and supports LTM in bands A, B, C, D, E, F and G, the UE can perform inter-frequency L1 measurements of SSBs of candidate cells in bands A, B, C, D, E, F and G. RAN2 agreed to introduce a new UE capability to allow the UE to indicate, per UE, that the SSBs of LTM candidate cells on which the UE can perform inter-frequency L1 measurements are limited to the BCs in which the UE indicates support of inter-frequency L1 measurements. RAN2 did not agree yet the final name and description but it could be for example: interFreqL1-OnlyInBC-r18 When included, indicates that, for each BC in which the UE indicates support of interFreqL1-MeasConfig-r18, the UE only supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) in the BC i.e., the UE does not support L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) out of the BC.; UE; No; No; No. In the above example, if the UE also signals interFreqL1-OnlyBC, the UE can perform inter-frequency L1 measurements of SSBs of candidate cells in bands A, B and C, but cannot perform inter-frequency L1 measurements of SSBs of candidate cells in bands D, E, F or G.

Before we discuss the issues raised in RAN2 LS, we would like review RAN4 design of inter-frequency L1 measurement in R18. According to TS38.133 section 9.15, both inter-frequency L1 measurement with and without gap are supported. Regarding inter-frequency L1 measurement within gap, it was agreed as a per UE capability, which can be found in the agreed RAN4 feature list and TS38.306:

| ltm-InterFreqMeasGap-r18 Indicates whether the UE supports SSB based inter-frequency L1-RSRP measurements with measurement gaps for LTM. A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Observation 1: in RAN4 design inter-frequency L1 measurement with gap is a per UE feature.

As for inter-frequency L1 measurement without gap, it is limited to the serving band according to the applicability defined in RAN4 spec:

When configured by the network and the inter-frequency cell’s SSB is completely contained in the DL active BWP, the UE shall be able to perform L1-RSRP measurements without gaps and without interruptions based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap.

Observation 2: in RAN4 design inter-frequency L1 measurement without gap can only happen on serving bands.

Back to the issues in RAN2 LS. Actually, RAN2 design of interFreqL1-MeasConfig-r18 is effectively aligned with RAN4 design for both inter-frequency L1 measurement with and without gap.

For inter-frequency L1 measurement with gap, the following RAN2’s understanding is aligned with RAN4 design: RAN2 understands that, when the UE indicates support of this capability for a BC and the UE is currently operating in this BC (or a subset of it), the SSBs of candidate cells on which the UE can perform inter-frequency L1 measurements according to this UE capabilities can be in any band on which the UE supports LTM. It is also reflected in RAN2’s example: if the UE has serving cells in bands A and B, indicates support of inter-frequency L1 measurements in a BC (A, B, C) and supports LTM in bands A, B, C, D, E, F and G, the UE can perform inter-frequency L1 measurements of SSBs of candidate cells in bands A, B, C, D, E, F and G.

For inter-frequency L1 measurement without gap, according to RAN4 design it can only happen on serving bands. In the above RAN2’s example, it can only happen on bands A and B.

Therefore, having interFreqL1-MeasConfig-r18 seems enough based on RAN4 measurement design. It seems unnecessary at least from RAN4 point of view to have the new UE capability - interFreqL1-OnlyInBC-r18. The consequence of the new UE capability is given by RAN2’s example:  In the above example (UE has serving cells in bands A and B, indicates support of inter-frequency L1 measurements in a BC (A, B, C) and supports LTM in bands A, B, C, D, E, F and G), if the UE also signals interFreqL1-OnlyBC, the UE can perform inter-frequency L1 measurements of SSBs of candidate cells in bands A, B and C, but cannot perform inter-frequency L1 measurements of SSBs of candidate cells in bands D, E, F or G.

To make it clear, we can summarize the examples below:

- Serving bands: A and B

- Support of interFreqL1-MeasConfig-r18: A, B and C

- Support of LTM: A, B, C, D, E, F and G

- Assuming UE supports interFreqSSB-L1-MeasWithoutGaps-r18 in BC of A and B, then:

  - Support of inter-frequency L1 measurement without gap: A and B regardless of interFreqL1-OnlyBC.

  - Support of inter-frequency L1 measurement with gap:

    - If interFreqL1-OnlyBC is false: A, B, C, D, E, F and G

    - If interFreqL1-OnlyBC is true: A, B and C

Based on above analysis, the case where UE only supports inter-frequency L1 measurement on bands A, B and C but not on bands D, E, F and G does NOT exist.

Observation 3: the case, where SSBs of LTM candidate cells on which the UE can perform inter-frequency L1 measurements are limited to the BCs in which the UE indicates support of inter-frequency L1 measurements, does NOT exist from RAN4 measurement design point of view.

Proposal 4: the new UE capability interFreqL1-OnlyBC is unnecessary from RAN4 measurement design point of view.

We also provide a draft LS to RAN2 in Annex.B.

# Conclusion

In this contribution, we provide discussion on RAN2 LS on R18 LTM. After discussion, the following conclusions are provided:

Proposal 1: answer to Q1 is YES, i.e. UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations under this condition.

Proposal 2: answer to Q2: it is feasible for UE to perform early ASN.1 decoding of an LTM candidate configuration without knowing the number of SpCells and SCells within the configured LTM candidate configurations. However, if the actual number of cells within the configured LTM candidate configuration exceeds the corresponding UE capability, NW has to assume UE may not have done early decoding, i.e. TLTM-RRC-processing during cell switch cannot be 0 as minimum requirements.

Proposal 3: if NW expects UE can successfully perform early ASN.1 decoding, number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations is expected NOT to exceed the UE reported maxNumberStoredConfigCells.

Observation 1: in RAN4 design inter-frequency L1 measurement with gap is a per UE feature.

Observation 2: in RAN4 design inter-frequency L1 measurement without gap can only happen on serving bands.

Observation 3: the case, where SSBs of LTM candidate cells on which the UE can perform inter-frequency L1 measurements are limited to the BCs in which the UE indicates support of inter-frequency L1 measurements, does NOT exist from RAN4 measurement design point of view.

Proposal 4: the new UE capability interFreqL1-OnlyBC is unnecessary from RAN4 measurement design point of view.

# Reference

- R4-2417509, LS on the fast RRC processing for LTM, RAN2

- R4-2417508, LS on UE capabilities for inter-frequency L1 measurements for LTM, RAN2

# Annex.A

3GPP TSG-RAN WG4 Meeting #113						     R4-24xxxxx

Orlando, USA, 18th – 22nd November, 2024

Title:	LS to RAN2 on the fast RRC processing for LTM

Response to:	R2-2409377

Release:	Release 18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	Qiming Li

E-mail Address:	< li_qiming@apple.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN4 thanks RAN2 for the LS on fast RRC processing for LTM. RAN4 discussed the questions from RAN2 and reached consensus on the following answers.

Q1: As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations?

Answer: YES, i.e. UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations under this condition

Q2: Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration?

Answer: it is feasible for UE to perform early ASN.1 decoding of an LTM candidate configuration without knowing the number of SpCells and SCells within the configured LTM candidate configurations. However, if the actual number of cells within the configured LTM candidate configuration exceeds the corresponding UE capability, NW has to assume UE may not have done early decoding, i.e. TLTM-RRC-processing during cell switch cannot be 0 as minimum requirements.

Q3: Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

Answer: if NW expects UE can successfully perform early ASN.1 decoding, number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations is expected NOT to exceed the UE reported maxNumberStoredConfigCells.

2. Actions:

To RAN2.

ACTION: 	RAN4 kindly asks RAN2 to take above information into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #114		              17 - 21 February 2025 			Athens, Greece

TSG RAN WG4 Meeting #114bis		              7 - 1 April 2025				China

# Annex.B

3GPP TSG-RAN WG4 Meeting #113						     R4-24xxxxx

Orlando, USA, 18th – 22nd November, 2024

Title:	LS to RAN2 on UE capabilities for inter-frequency L1 measurements for LTM

Response to:	R2-2409360

Release:	Release 18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	Qiming Li

E-mail Address:	< li_qiming@apple.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN4 thanks RAN2 for the LS on UE capabilities for inter-frequency L1 measurements for LTM. RAN4 had extensive discussion on the issue in RAN4#133 and reached the following agreement:

Agreements:

- The new UE capability interFreqL1-OnlyBC is unnecessary from RAN4 measurement design point of view.

2. Actions:

To RAN2.

ACTION: 	RAN4 kindly asks RAN2 to take above information into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #114		              17 - 21 February 2025 			Athens, Greece

TSG RAN WG4 Meeting #114bis		              7 - 1 April 2025				China