3GPP TSG-RAN WG4 Meeting #113                                       	                       R4-2418595

# Orlando FL, USA, November 18 – November 22, 2024

Agenda item:	7.1.1.3.2

Source:	Apple Inc.

Title:	Views on Maximum Number of MIMO Layers for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

In this discussion paper, we share our views on the maximum number of MIMO layers for 6Rx handheld UEs. Enabling 6Rx for large form-factor handheld UEs has the potential of improving DL coverage and throughput. However, it is important to understand if supporting 6 layers vs. 4 layers can produce practical throughput gains. We continue discussing our views below, including simulation results supporting our conclusions.

# Scope

During the previous RAN4#112bis meeting in Hefei, China, discussion continued on the general considerations for the MIMO layers evaluation for 6Rx UEs, and in addition, specific outcomes on 6 layers support. As captured in the previous WF [2], we highlight the discussion outcome below:

Sub-topic 3-2: 6-layer Support

Issue 3-1-1: 6-layer Support

Way Forward: RAN4 to further discuss the following options (options are not necessarily mutually exclusive).

- Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios.

- Option 2: 6-layer support is feasible for FWA devices.

- Option 3: Companies to further discuss and align on the feasibility criteria and simulation assumptions by RAN4#113 to evaluate 6-Layer feasibility for handhelds using the following as a starting point.

- Feasibility criteria

- Consider 4-layer vs 6-layer throughput performance.

- Simulation assumptions

- Only consider link-level simulation for performance evaluations.

- Updated simulation assumptions are provided in Annex A for discussions on alignment at RAN4#113 and can be used by companies to provide simulation results at RAN4#113 for discussion. Companies to clarify and justify (e.g., where the number comes from) the settings used in the simulation assumptions for BS correlation matrix and Tx EVM.

- Option 4: 6-layer support is not feasible for handheld UEs.

- Option 5: Companies to further discuss the following aspect of 6-layer support.

- If the handheld feasibility is confirmed, then the handheld and FWA devices will share the same 6-layer requirements.

- If the handheld feasibility is not confirmed, 6-layer requirements will be specified for FWA device only.

Issue 3-1-2: 6-layer Support as optional feature

Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature per the following.

- Introduce 6 MIMO layers support as an optional feature, if 6-layer performance requirements are introduced for 6Rx.

Issue 3-1-3: TxEVM for feasibility study

Way Forward: FFS on Tx EVM values to be used for feasibility evaluation. RAN4 to further discuss the following:

Option 1: Use Tx EVM = 1%

Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation

Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM

Option 4: use the existing BS Tx EVM core requirement

Option 5: The representative BS Tx EVM values provided by the BS vendor.

In addition to this, Annex A in [2] corresponds to the progress on agreed simulation assumptions, which will be discussed below.

# Discussion

One of the primary objectives of this WID is twofold: 1) to establish 4 MIMO layers as the baseline configuration, and 2) to evaluate the potential performance gains and feasibility of supporting 6 MIMO layers with a 6-receive antenna (6Rx) user equipment (UE). During the previous meeting, we reached several foundational agreements to quantitatively assess whether supporting up to 6 MIMO layers is viable. Specifically, we agreed that: 1) feasibility criteria should be based on throughput performance comparisons between 4-layer and 6-layer configurations, 2) only link-level simulations are relevant for this evaluation, and 3) a preliminary set of detailed simulation assumptions was documented in Annex A of the previous WF [2]. While this represents significant progress, we propose narrowing the parameter space further to streamline the decision-making process. Additionally, based on our own simulation results, we conclude that supporting 6 MIMO layers on a handheld UE is not feasible, a position that is shared by several other companies.

Simulation Parameters:

In Annex A, one of the assumptions in discussion is the transmit antenna configuration. During RAN4#112bis it was agreed to 1) Consider {8Tx, 6Rx} as baseline, and 2) Interested companies can submit results with {32Tx, 6Rx} and specify the precoding type.

Observation #1: There is insufficient justification to increase the number of transmit antennas beyond 8 when considering only 6 MIMO layers.

Proposal #1: Limit the transmit/receive antenna configuration to 8Tx/6Rx.

Proposal #2: Exclude 16Tx/6Rx and 32Tx/6Rx antenna configurations from the feasibility analysis.

Regarding rank, the previous RAN4#112bis meeting concluded that: 1) Rank 4 and Rank 6 should be included in the feasibility assessment, and 2) companies expressing interest may submit results for Rank 5. In this context, we maintain that the primary intent of the WID is to assess whether six MIMO layers provide a performance advantage over four MIMO layers, despite other potential interpretations based on the specific wording of the WID. For instance, if a scenario arises where five MIMO layers outperform four layers, rather than six, the key question remains whether this feature would offer sufficient benefit to justify deployment. Our position is that, in such a case, it would not.

Observation #2: The primary objective of the WID is to assess whether 6 MIMO layers can outperform 4 MIMO layers.

Proposal #3: Include only Rank 4 and Rank 6 configurations in the feasibility evaluation.

Regarding the selection of MCS values for the analysis, it was agreed to use Adaptive MCS for performance comparison. As per the agreement in the previous meeting, the focus will be on the 64QAM and 256QAM modulation schemes. For this analysis, Table 2 provides all necessary data points.

Observation #3: MCS Table 2 already considers the full range of code rates and spectral efficiency to be evaluated for 64QAM.

Proposal #4: Consider only MCS Table 2 for the feasibility evaluation.

Regarding the DMRS configuration, four options were still under consideration during the previous meeting, one of which includes Rel-18 DMRS eType-1. We believe that the most equitable comparison is to focus on Option 3: [1+1 (4L), 2+2 (6L)], as these configurations represent the typical deployment scenarios. It is anticipated that the majority of field deployments will use 1+1 for 4 layers, as this is sufficient for Rel-15 DMRS Type-1.

Observation #4: The typical DMRS configuration used by operators is 1+1 for 4 layers, while 2+2 is typically used for 6 layers due to RAN1 design requirements.

Proposal #5: Limit the analysis to Rel-15 DMRS Type-1 for this study.

Proposal #6: Use only Option 3: [1+1 (4L), 2+2 (6L)] as the DMRS configuration.

Regarding the assumption on TxEVM impairments, we propose a typical value based on prior experience with demodulation performance requirements. Specifically, we suggest 6% for 64QAM and 3% for 256QAM, which already represent lower values than the minimum base station (BS) requirements. We do not support custom tightening of these values for the purposes of this specific Work Item, but rather advocate for a general tightening of the requirements if multiple companies can demonstrate that TxEVM performance in the field consistently exceeds the minimum specifications. Furthermore, no tightening was agreed upon during the Rel-18 8Rx evaluation, and we believe the same approach should be applied here.

Observation #5: If multiple companies support the claim that TxEVM in the field exceeds the minimum BS RF requirements, then the requirements should be tightened. However, this would fall outside the scope of this Work Item.

Proposal #7: Do not relax TxEVM for the purposes of this feasibility study. Consider TxEVM values of 6% for 64QAM and 3% for 256QAM.

MIMO Correlation Models:

3GPP ULA Models: We have expanded the use of the ULA 3GPP model based on the α and β parameters, a proposal that has also been put forward by other companies. In one of our previous contributions, we considered the MediumB model, motivated by its status as the 3GPP model with the least correlation. For α=0.3 and β=0.005154, the resulting correlation matrices for both the transmission (Tx) and reception (Rx) sides are shown in the figure below. Using this correlation matrix, we observed no performance gain.

Although the correlation coefficients in the Rx matrix are relatively reasonable, we acknowledge that the Tx correlation may be too high. Therefore, moving forward, we propose relaxing α to 0.005154 to generate a new pair of correlation matrices. For our simulation results, we have chosen to consider two custom models, as described below.

- Custom_1: \alpha = 0.005154 and \beta = 0.005154

- Custom_2: \alpha = 0, \beta = 0.005154

For illustration purposes, the obtained correlation matrices for the Custom_1 model are given below:

3GPP XPL Models: For completeness, we also consider the use of the XPL model. For our simulation results, we choose to consider two custom models described as

- Custom_3: \alpha = 0.005154, \beta = 0.005154, \gamma = 0.2

- Custom_4: \alpha = 0, \beta = 0.005154, \gamma = 0.2

In the case of the XPL model, the corresponding matrices are given below:

Overall, performance will be superior with XPL models.

Observation #6: The ULA and XPL 3GPP correlation models with parameters such as α=0.005154, β=0.005154, and γ=0.2 are relatively optimistic, yet they remain reasonable for this study.

Proposal #8: Include at least the XPL 3GPP correlation model structure in the evaluation.

Custom ECC Models: Given that some companies are willing to share ECC measurements, it was agreed during RAN4#112bis to at least consider custom MIMO correlation models as part of the evaluation options. From our perspective, however, we remain concerned about the methodology used to derive these correlation models, including the types of devices they represent and other related factors, as they may present an overly optimistic scenario.

Observation #7: The exact conditions under which ECC-based correlation models were derived remain unclear.

Proposal #9: RAN4 should clarify the nature of the proposed ECC-based custom correlation models and specify the types of devices they represent.

In addition, during RAN4#112bis has been agreed that since custom correlation models are considered, also antenna efficiency should be part of the analysis. Antenna efficiency, plus other practical factors, increase the SNR

Observation #8: During RAN4#112bis, it was agreed that antenna efficiency could be considered in the feasibility analysis, and companies are encouraged to provide antenna efficiency data.

Observation #9: Results submitted by companies during RAN4#112bis and earlier are based on baseband simulations and do not account for antenna efficiency or impairment factors.

Proposal #10: Apply at least a 3 dB correction to the SNR values reported by companies in baseband simulation results.

# Simulation Results

As the starting point for our analysis, it is important to note that a comprehensive calibration exercise involves comparing the performance of 4 MIMO layers versus 6 MIMO layers under ideal conditions. Specifically, this assumes no correlation at both the Tx and Rx sides (α=0, β=0), and no impact from TxEVM. This approach allows us to establish the best-case scenario, after which we can proceed with analyzing how factors like correlation and TxEVM may influence our conclusions.

In Figure 1 below, we present a comparison between Rank 4 and Rank 6 using MCS adaptation across the full range of 64QAM and 256QAM (MCS Table 2).

Figure 1: Absolute throughput comparison between Rank 4 and Rank 6 under the assumptions of no antenna correlation and no impact of TxEVM. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2. Baseband results.

At this stage, it is clear that any potential performance gain between 6 MIMO layers and 4 MIMO layers would only be observable at exceptionally high SNR values, which could be considered impractical or overly optimistic relative to the SNR levels achievable in real-world deployments.

Observation #10: Performance gains between 6 MIMO layers and 4 MIMO layers are only noticeable at very high SNR (>> 30 dB) under idealized conditions with no correlation and no TxEVM.

The next step is to assess the impact of antenna correlation on this comparison. In Figure 2 below, we present the performance results using the ULA Custom models described earlier.

Figure 2: Absolute throughput comparison between Rank 4 and Rank 6 considering antenna correlation based on custom ULA model coefficients but no impact of TxEVM. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2. Baseband results.

It is immediately clear that the correlation at both Tx and Rx sides play an important role in the performance. Unfortunately, Rank 6 this time performs worse than Rank 4, which is quite obvious.

Observation #11: Rank 4 outperforms Rank 6 under optimistic MIMO antenna correlations and with no TxEVM.

In the the next step we now include the effect of TxEVM in the results. See Figure 3.

Figure 3: Absolute throughput comparison between Rank 4 and Rank 6 considering antenna correlation based on custom ULA model coefficients and a TxEVM impairment of 6% for 64QAM modulation and 3% for 256QAM modulation. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2. Baseband results.

Once again, including this additional impairment significantly degrades the performance one more time. Here, Rank 6 show basically no throughput.

Observation #12: Under the current simulation assumptions, the effect of TxEVM, when applied to the optimistic MIMO correlation models, severely degrades throughput for 4 MIMO layers and essentially reduces throughput to zero for the 6-layer case.

Finally, the last set of simulation results is about the use of customized XPL model results, for which we show the same analysis used in the case of ULA models. See Figure 4 and Figure 5.

Figure 4: Absolute throughput comparison between Rank 4 and Rank 6 considering antenna correlation based on custom XPL model coefficients but no TxEVM impact. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2. Baseband results.

Figure 5: Absolute throughput comparison between Rank 4 and Rank 6 considering antenna correlation based on custom XPL model coefficients and a TxEVM impairment of 6% for 64QAM modulation and 3% for 256QAM modulation. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2. Baseband results.

Observation #13: Under custom XPL models, performance improves for both 4 MIMO and 6 MIMO layers. However, when TxEVM is taken into account, 4 MIMO layers still outperform 6 MIMO layers.

At this point, and based on the simulations results above, the conclusion is quite clear.  However, in addition to the results above, we will present results based on agreed options from RAN4#112bis.

First, we will analyze simulation results based on custom correlation models based on ECC measurements. As shown in Figure 6 below, the inclusion of these models in comparison to no antenna correlation show that they are likely very optimistic. If we pay attention to Oppo’s correlation matrix and its very optimistic coefficients, it’s basically saying that there’s virtually no correlation in their model.

Figure 6: Absolute throughput comparison between ideal zero correlation (3GPP Low) vs Custom ECC-based correlation matrices, for both Rank 4 and Rank 6. No TxEVM is considered for both 64QAM and 256QAM modulations. Baseband results.

In any case, even if these models are fully realistic, the conclusion that small gains are only observed at very high SNRs does not change.

Observation #14: Under custom ECC-based models and ideal conditions (no TxEVM impairments), 6 MIMO layers would only outperform 4 MIMO layers at very high SNR (>> 30 dB).

In addition, we also agreed to explore what would be the hypothetical consequences of assuming the same DMRS overhead between Rank 4 and Rank 6. Even though we do not believe this is a valid and relevant scenario to consider, we show some of these results in Figure 7 below.

Figure 7: Absolute throughput comparison between ideal zero correlation (3GPP Low) vs Custom ECC-based correlation matrices, for Rank 6. In addition, Rank 4 with no correlation is added as a reference, both as 1+1 and the 2+2 hypothetical DMRS configuration. No TxEVM is considered for both 64QAM and 256QAM modulations. Baseband results.

In Figure 7 above we can still observe that if we hinder Rank 4 to the same overhead as Rank 6, Rank 6 outperforms Rank 4 still at a very high SNR condition. In addition to this, we need to consider both TxEVM impairments and implementation aspects into the equation.

Observation #15: Under custom ECC-based models, assuming no TxEVM impairments and hypothetical equal DMRS overhead between Rank 4 and Rank 6, 6 MIMO layers would still only outperform 4 MIMO layers at very high SNR.

Finally, as the last pending topic, we want to address the influence of TxEVM under the performance of the custom ECC-based models. In Figure 8 below we use Samsung’s proposal as the ECC correlation model.

Figure 7: Absolute throughput comparison between ideal 0% TxEVM, 1% TxEVM and 6%/3% TxEVM for 64QAM and 256QAM modulations, for Rank 4 and Rank 6. Baseband results obtained using Samsung’s ECC-based proposal.

Observation #16: The proposal to use a TxEVM of 1% is overly optimistic, as this value is one-eighth of the BS minimum requirement for 64QAM and less than one-third of the minimum BS requirement for 256QAM.

Observation #17: Under realistic TxEVM values, the performance of both 64QAM and 256QAM modulations is significantly affected, particularly at high SNR.

Hence, after all the numerical results presented, we believe that the conclusion remains the same with respect to the evaluation of 6 MIMO layers for 6Rx handheld UEs.

Proposal #9: RAN4 to confirm that 6 MIMO layers with 6Rx is not feasible for handheld scenarios.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: There is insufficient justification to increase the number of transmit antennas beyond 8 when considering only 6 MIMO layers.

Proposal #1: Limit the transmit/receive antenna configuration to 8Tx/6Rx.

Proposal #2: Exclude 16Tx/6Rx and 32Tx/6Rx antenna configurations from the feasibility analysis.

Observation #2: The primary objective of the WID is to assess whether 6 MIMO layers can outperform 4 MIMO layers.

Proposal #3: Include only Rank 4 and Rank 6 configurations in the feasibility evaluation.

Observation #3: MCS Table 2 already considers the full range of code rates and spectral efficiency to be evaluated for 64QAM.

Proposal #4: Consider only MCS Table 2 for the feasibility evaluation.

Observation #4: The typical DMRS configuration used by operators is 1+1 for 4 layers, while 2+2 is typically used for 6 layers due to RAN1 design requirements.

Proposal #5: Limit the analysis to Rel-15 DMRS Type-1 for this study.

Proposal #6: Use only Option 3: [1+1 (4L), 2+2 (6L)] as the DMRS configuration.

Observation #5: If multiple companies support the claim that TxEVM in the field exceeds the minimum BS RF requirements, then the requirements should be tightened. However, this would fall outside the scope of this Work Item.

Proposal #7: Do not relax TxEVM for the purposes of this feasibility study. Consider TxEVM values of 6% for 64QAM and 3% for 256QAM.

Observation #6: The ULA and XPL 3GPP correlation models with parameters such as α=0.005154, β=0.005154, and γ=0.2 are relatively optimistic, yet they remain reasonable for this study.

Proposal #8: Include at least the XPL 3GPP correlation model structure in the evaluation.

Observation #7: The exact conditions under which ECC-based correlation models were derived remain unclear.

Proposal #9: RAN4 should clarify the nature of the proposed ECC-based custom correlation models and specify the types of devices they represent.

Observation #8: During RAN4#112bis, it was agreed that antenna efficiency could be considered in the feasibility analysis, and companies are encouraged to provide antenna efficiency data.

Observation #9: Results submitted by companies during RAN4#112bis and earlier are based on baseband simulations and do not account for antenna efficiency or impairment factors.

Proposal #10: Apply at least a 3 dB correction to the SNR values reported by companies in baseband simulation results.

Observation #10: Performance gains between 6 MIMO layers and 4 MIMO layers are only noticeable at very high SNR (>> 30 dB) under idealized conditions with no correlation and no TxEVM.

Observation #11: Rank 4 outperforms Rank 6 under optimistic MIMO antenna correlations and with no TxEVM.

Observation #12: Under the current simulation assumptions, the effect of TxEVM, when applied to the optimistic MIMO correlation models, severely degrades throughput for 4 MIMO layers and essentially reduces throughput to zero for the 6-layer case.

Observation #13: Under custom XPL models, performance improves for both 4 MIMO and 6 MIMO layers. However, when TxEVM is taken into account, 4 MIMO layers still outperform 6 MIMO layers.

Observation #14: Under custom ECC-based models and ideal conditions (no TxEVM impairments), 6 MIMO layers would only outperform 4 MIMO layers at very high SNR (>> 30 dB).

Observation #15: Under custom ECC-based models, assuming no TxEVM impairments and hypothetical equal DMRS overhead between Rank 4 and Rank 6, 6 MIMO layers would still only outperform 4 MIMO layers at very high SNR.

Observation #16: The proposal to use a TxEVM of 1% is overly optimistic, as this value is one-eighth of the BS minimum requirement for 64QAM and less than one-third of the minimum BS requirement for 256QAM.

Observation #17: Under realistic TxEVM values, the performance of both 64QAM and 256QAM modulations is significantly affected, particularly at high SNR.

Proposal #9: RAN4 to confirm that 6 MIMO layers with 6Rx is not feasible for handheld scenarios.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

- R4-2417209 “WF on 6Rx UE requirements”, AT&T, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, October 14-18, 2024