3GPP TSG-RAN WG4 Meeting #113	R4-2418619

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:	Discussion and simulation results on MIMO layer evaluation for 6Rx UE

Agenda item:	7.1.1.3.2

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements (which are specified for fixed wireless access (FWA) types of devices). The 6Rx in Rel-19 is targeting for handheld UE considering 6Rx would be possible and useful to improve DL coverage and throughputs for the large size handheld UEs, e.g., the foldable smartphone. In the RAN#104 meeting, the WID has been updated [2]. The latest objectives of 6Rx are as below:

Objective of Core part WI 6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. Objective of Performance part WI 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - Support 4 MIMO layers at least, and follow the conclusion in core part for whether to support 6 MIMO layers - Specify the SDR requirements to support 6Rx

At least three pieces of information could be obtained from above objective descriptions of 6Rx. Firstly, it is clearly mentioned the objective is targeting at the support of handheld UE or smartphone. Secondly, 6 MIMO layers can be supported only when both the gain and feasibility are verified. At last, once the Rx requirements for 6 MIMO layers are specified for handheld UE, these requirements can also be applicable for FWA devices, herein the FWA device is in a subordinate position to the handheld UE. Hence, it seems the logic of the WID should be “if the Rx requirements for 6 MIMO layers are specified for handheld UE, then these requirements can also be applicable for FWA device; on the other hand, if the Rx requirements for 6 MIMO layers are not specified for handheld UE, then no related requirements could be used for FWA device as well.”

Observation 1: It is clearly mentioned in the WID, the objective of 6Rx is targeting at the support of handheld UE or smartphone.

Observation 2: For the objective of 6Rx described in the WID, 6 MIMO layers can be supported only when both the gain and feasibility are verified.

Observation 3: For the objective of 6Rx described in the WID, once the Rx requirements for 6 MIMO layers are specified for handheld UE, these requirements can also be applicable for FWA devices, herein the FWA device is in a subordinate position to the handheld UE.

Observation 4: The logic for the objective of 6Rx described in the WID should be “if the Rx requirements for 6 MIMO layers are specified for handheld UE, then these requirements can also be applicable for FWA device; on the other hand, if the Rx requirements for 6 MIMO layers are not specified for handheld UE, then no related requirements could be used for FWA device as well.”

According above observations, we think these is no need to discuss the 6 MIMO layers feasibility for FWA devices at all. Once the gain and feasibility for handheld UE with 6 MIMO layers are verified, then 6 MIMO layers related requirements defined for handheld UE could be applicable for FWA devices. Otherwise, no 6 MIMO layers related requirements should be defined for neither handheld UE nor FWA devices.

Proposal 1: Once the gain and feasibility for handheld UE with 6 MIMO layers are verified, then 6 MIMO layers related requirements defined for handheld UE could be applicable for FWA devices. Otherwise, no 6 MIMO layers related requirements should be defined for neither handheld UE nor FWA devices.

In last RAN4#112bis meeting, further link level simulation assumptions alignment for the 6 MIMO layers performance gain validation of 6Rx receiver has been achieved [3], although there are still several unreasonable options proposed for some parameters. Our initial simulation results [5] which based on XP low/medium (even the correlation factor downgrades from 0.6 to 0.1) and further simulation results [6] which based on the measured UE ECC correlation matrixes show that the performance of 6 MIMO layers case is hard to outperform 4 MIMO layers. In this contribution, we provide our further analysis and simulation results for 6Rx receiver based on the agreed simulation assumptions in last RAN4#112bis meeting.

- Discussion and simulation results

Therotially, rank is used to describe the characteristics of the channel quantitatively for spatial mulitiplexing, which is the number of uncorrelation/independent virtual channels. For the scenarios with the same number of Rx antennas, it’s no doubt that the performance of 6 MIMO layers could outperform 4 MIMO layers if the channel uncorrelation promised. However, once correlation between antennas are introduced, the performance of 6 MIMO layers can decrease compared with 4 MIMO layers. The correlation not only come from UE receive antennas but also may come from BS transmit antennas and wireless channels. Therefore, both UE antenna correlation and BS antenna correlation matrix should be considered for the 6 MIMO layers performance evaluation. In addition, since 4 MIMO layers and 6 MIMO layers corresponding to different number of DMRS ports, which may impact the overhead and final throughput performance. Hence the configuration of DMRS ports need further discussion.  The last point is about the TxEVM, as the value of TxEVM can impact the throughput performance, especially at the high SNR scenarios the impct is non negligible.

  - UE antenna correlation matrix

UE antenna correlation matrix should be highly related with the product design, different product design may correspond to different antenna correlation matrix. In our view, this is the reason why RAN4 has recieved some contributions focusing on foldable devices which may understate the antenna correlation with a larger form factor than normal handheld UE. However, on top of the larger space of the foldable UEs, it also should be noted that there would be a lot of other restrictions to support the advanced form factor. Even for the the foldable devices on the market, according to our experiences, it’s not easy to make RF path between two sides due to the hinge and both side full screen for example. Hence we take two antenna deployments as reference in Figure 2-1, which are surely practical layouts of 6Rx for foldable Ues. The ECC antenna correlation matrix A and B for foldable devices corresponding to Figure 2-1 are used for our simulation.

Observation 5: For realistic antenna correlation factors of the foldable devices, it also should be noted that there would be a lot of other restrictions to support the advanced form factor on top of the larger space of the foldable UEs.

UE antenna correlation matrix A

UE measured ECC (Envelop Correlation Coefficient) matrix A for N41 based on antenna deployment in Figure 2-1 (a).

1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00

UE antenna correlation matrix B

UE measured ECC matrix B for N77 based on antenna deployment in Figure 2-1 (b).

1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00

(a)                                           (b)

Figure 2-1. Antenna deployment examples for foldable UE

  - BS antenna correlation matrix

As mentioned above, the final channel correlation not only come from UE receive antennas but also may come from BS transmit antennas and wireless channels. Therefore, the BS antenna correlation matrix should be considered for the 6 MIMO layers performance evaluation as well. Ideal BS antenna correlation matrix (no correlation between different BS transmit antennas) could be used for simualtion results alignment, but it may be hard to promise. In addition, BS antenna correlation matrix with some correlations between different BS transmit antennas could be defined since it’s more realistic.

BS antenna correlation matrix A

Identity matrix assuming no correlation between different BS transmit antennas.

1001000000001001000000000000000000000000000000001001000000001001

BS antenna correlation matrix B

Refer to the channel spatial correlation matrix defined in 38.101-4 B.2.3.1, the BS antenna correlation matrix could be described as below.

1α149α449α949α1649α2549α3649αα149*1α149α449α949α1649α2549α3649α449*α149*1α149α449α949α1649α2549α949*α449*α149*1α149α449α949α1649α1649*α949*α449*α149*1α149α449α949α2549*α1649*α949*α449*α149*1α149α449α3649*α2549*α1649*α949*α449*α149*1α149α*α3649*α2549*α1649*α949*α449*α149*1

RAN4 defines α as 0.3 for ULA Medium correlation and 0.9 for ULA high correlation. Here we set α as 0.05 in order to introduce a really low correlation coefficient between BS transmit antennas.

  - DMRS configurations

For the DMRS configurations, currently four options are proposed as below.

Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) Option 2: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) Option 4: Rel-15 2+2(4L), Rel-15 2+2(6L)

Per our understanding and the description of DMRS configurations described in 38.211 and 38.331 as below, DMRS type 1 with one additional OFDM symbol for DMRS ports and maxLength=1 is the typical DMRS configuration (Rel-15 1+1 DMRS symbols) for 4 MIMO layers as Figure 2-2 (a), it is straightforward to configure maxLength=2 in order to support 6 MIMO layers as Figure 2-2 (b), hence we think Option 3 is the typical configuration for real network.  For Option 1 with Rel-18 1+1(6L), it only could be support when the feature of Rel-18 enhanced DMRS is supported. For Option 2, since only two adjacent OFDM symbols are configured for 6 MIMO layers case, it is only suitable for low velocity UEs, which could not work for medium and high velocity UEs. For option 4, It looks strange and unreasonable to configure maxLength=2 (Rel-15 2+2 DMRS symbols) for 4 MIMO layers, which is just a waste of resources.

Observation 6: For DMRS configurations, Option 3 (Rel-15 1+1(4L), Rel-15 2+2(6L)) is the typical configuration for real network.

Observation 7: For DMRS configurations, Option 1 (Rel-18 1+1(6L), Rel-15 1+1(4L)) only could be support when the feature of Rel-18 enhanced DMRS is supported; Option 2 is suitable for low velocity UEs, which could not work for medium and high velocity UEs. Option 4 (Rel-15 2+2(4L), Rel-15 2+2(6L)) is strange and unreasonable to configure maxLength=2 (Rel-15 2+2) for 4 MIMO layers, which is just a waste of resources and unrealistic.

Proposal 2: For DMRS configurations, use the typical configuration Option 3 (Rel-15 1+1(4L), Rel-15 2+2(6L)).

3GPP 38.211

3GPP 38.331

(a)                                                                             (b)

Figure 2-2. Typical DMRS ports configurations for Rel-15 DMRS

  - TxEVM assumption

About the TxEVM for simulation alignment, several options are proposed as below in last RAN4#112bis meeting

-	Option 1: Use Tx EVM = 1%

-	Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation

-	Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM

-	Option 4: use the existing BS Tx EVM core requirement

-	Option 5: The representative BS Tx EVM values provided by the BS vendor.

Considering the TxEVM requirements defined in 38.104 as below are minimum requirements, and some BS vendors may be able to achieve better TxEVM values, hence RAN4 simulation alignment usually use TxEVM 6% for 64QAM and TxEVM 3% for 256QAM. Since companies already agreed to do the feasibility study based on adaptive MCS with MCS Table 2 [3], we think it is a good selection to use TxEVM=3% for all of modulation order in MCS Table 2 for performance evaluation.

3GPP 38.104

Proposal 3: For the TxEVM used for simulation assumption, use TxEVM=3% for all of modulation order in MCS Table 2 for performance evaluation.

  - Simulation assumptions

As there are many options for different parameters in the feasibility study link level simulation assumption [3], it’s necessary to narrow down some options. Our simulations are based on Table 2-1.

Table 2-1 test parameters for 6Rx

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz |
| SCS |  | 30kHz |
| Antenna configuration |  | { 8Tx, 6Rx} |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed, rank 4 and 6 |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | Specific to each Reference channel |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
|  | Resource allocation type | Type 0 |
|  | RBG size | Config2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Position of the first DMRS for PDSCH mapping type A | 2 |
|  | Antenna ports indexes | {1000-1003} for Rank 4 tests {1000-1005} for Rank 6 tests |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 for rank <= 4 2 for rank > 4 1 for case “2plus0” of rank > 4 |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx |
| Codebook configuration | CodebookType | typeI-SinglePanel |
|  | CodebookMode | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) | (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| UE correlation matrix |  | Matrix A: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Matrix B: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 |
| BS correlation matrix |  | Matrix A: 1001000000001001000000000000000000000000000000001001000000001001 Matrix B: α as 0.05 1α149α449α949α1649α2549α3649αα149*1α149α449α949α1649α2549α3649α449*α149*1α149α449α949α1649α2549α949*α449*α149*1α149α449α949α1649α1649*α949*α449*α149*1α149α449α949α2549*α1649*α949*α449*α149*1α149α449α3649*α2549*α1649*α949*α449*α149*1α149α*α3649*α2549*α1649*α949*α449*α149*1 |

  - Simulation results

Based on the test parameters in Table 2-1, our simulation results are summarized in Figure 2-1 - Figure 2-2 for ideal BS correlation and limited BS correlation separately.

Figure 2-1. Performance comparison for 6Rx with ideal BS correlation matrix A with TxEVM=0%

Figure 2-2. Performance comparison for 6Rx with BS correlation matrix B with TxEVM=0%

From our simulation results, for both ideal BS antenna correlation matrix A (zero BS antenna correlation considered) and non-ideal BS antenna correlation matrix B, for both UE antenna correlation martix A and B, for 6Rx with 6 MIMO layers cases, there is no performance gain could be observed compared with 6Rx with 4 MIMO layers cases.

Observations from our simulation results could be summarized as:

Observation 8: For both ideal BS antenna correlation matrix A (zero BS antenna correlation considered) and non-ideal BS antenna correlation matrix B, there is no performance gain could be observed for 6Rx with 6 MIMO layers cases compared with 6Rx with 4 MIMO layers cases.

Observation 9: For both UE antenna correlation martix A and B, there is no performance gain could be observed for 6Rx with 6 MIMO layers cases compared with 6Rx with 4 MIMO layers cases.

From above observations, it seems it’s hard to get the benefit using 6Rx with 6 MIMO layers.

Simulation results analysis

According to our simulation results and observations above, 6 MIMO layers could not get benefit even at high SNR value range, even with zero BS antenna correlation assumed and even with 0% TxEVM applied. In addition, for the practical deployment, we can easily assume that it is hard to promise zero BS antenna correlation factor and 0% TxEVM. As a consequence of this performance evaluation, for both handheld UE and FWA device with 6Rx receiver antennas, we propose not to introduce requirements for 6 MIMO layers.

Proposal 4: For both handheld UEs and FWA device with 6Rx antennas, there is no need to introduce requirements for 6 MIMO layers since no benefit could be observed from our simulation results based on realistic antenna correlation matrixes.

- Conclusion

In this contribution, we provide our observations from WID and specifications, simulation assumptions, simulation results analysis, and views on “6Rx for handheld and FWA” topic.

The observations and proposals could be summarized as:

Observation 1: It is clearly mentioned in the WID, the objective of 6Rx is targeting at the support of handheld UE or smartphone.

Observation 2: For the objective of 6Rx described in the WID, 6 MIMO layers can be supported only when both the gain and feasibility are verified.

Observation 3: For the objective of 6Rx described in the WID, once the Rx requirements for 6 MIMO layers are specified for handheld UE, these requirements can also be applicable for FWA devices, herein the FWA device is in a subordinate position to the handheld UE.

Observation 4: The logic for the objective of 6Rx described in the WID should be “if the Rx requirements for 6 MIMO layers are specified for handheld UE, then these requirements can also be applicable for FWA device; on the other hand, if the Rx requirements for 6 MIMO layers are not specified for handheld UE, then no related requirements could be used for FWA device as well.”

Proposal 1: Once the gain and feasibility for handheld UE with 6 MIMO layers are verified, then 6 MIMO layers related requirements defined for handheld UE could be applicable for FWA devices. Otherwise, no 6 MIMO layers related requirements should be defined for neither handheld UE nor FWA devices.

Observation 5: For realistic antenna correlation factors of the foldable devices, it also should be noted that there would be a lot of other restrictions to support the advanced form factor on top of the larger space of the foldable UEs.

Observation 6: For DMRS configurations, Option 3 (Rel-15 1+1(4L), Rel-15 2+2(6L)) is the typical configuration for real network.

Observation 7: For DMRS configurations, Option 1 (Rel-18 1+1(6L), Rel-15 1+1(4L)) only could be support when the feature of Rel-18 enhanced DMRS is supported; Option 2 is suitable for low velocity UEs, which could not work for medium and high velocity UEs. Option 4 (Rel-15 2+2(4L), Rel-15 2+2(6L)) is strange and unreasonable to configure maxLength=2 (Rel-15 2+2) for 4 MIMO layers, which is just a waste of resources and unrealistic.

Proposal 2: For DMRS configurations, use the typical configuration Option 3 (Rel-15 1+1(4L), Rel-15 2+2(6L)).

Proposal 3: For the TxEVM used for simulation assumption, use TxEVM=3% for all of modulation order in MCS Table 2 for performance evaluation.

Observation 8: For both ideal BS antenna correlation matrix A (zero BS antenna correlation considered) and non-ideal BS antenna correlation matrix B, there is no performance gain could be observed for 6Rx with 6 MIMO layers cases compared with 6Rx with 4 MIMO layers cases.

Observation 9: For both UE antenna correlation martix A and B, there is no performance gain could be observed for 6Rx with 6 MIMO layers cases compared with 6Rx with 4 MIMO layers cases.

Proposal 4: For both handheld UEs and FWA device with 6Rx antennas, there is no need to introduce requirements for 6 MIMO layers since no benefit could be observed from our simulation results based on realistic antenna correlation matrixes.

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#104 meeting, Huawei

[3] R4-2417209, WF on 6Rx UE requirements, RAN4#112bis meeting, AT&T

[4] R4-2409459, Views on MIMO layer for 6Rx UE, RAN4#111 meeting, Samsung

[5] R4-2412877, Discussion and initial simulation results on MIMO layer evaluation for 6Rx UE, RAN4#112 meeting, Samsung

[6] R4-2415681, Discussion and simulation results on MIMO layer evaluation for 6Rx UE, RAN4#112bis meeting, Samsung