3GPP TSG-RAN4 Meeting #113	R4-2418632

# Orlando, US, Nov.18th – 22nd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5106 | rev | - | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) Correction to RLM scheduling restriction test cases_R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2024-11-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CAT A CR for R4-2420171 - In FR2 RLM scheduling restriction test cases. TE schedules PDCCH on the symbols right before the RLM-RS/BFR-RS. Then the DUT is required to correctly receive the scheduled PDSCH and sends back corresponding ACK/NACKs. However, UE is also required to perform intra-frequency measurements on SpCell in this TCs (Note that RMC SMTC.1 is also configured) hence needs to perform RSRP/RSRQ on serving cells as specified in 38.331. According 38.133 cl. 9.2.5.3.3, L3 measurement also can causes scheduling restriction on SSB symbols and RSSI symbols (the entire SMTC window when ss-RSSI-measurement is not configured) and 1 data symbol before and after SSB/RSSI symbols The time domain position of SSB, SMTC, PDCCH, scheduling restriction caused by RLM/BFR and scheduling restriction caused by L3 measurements are depicted as following figure. It can observed that RLM/BFR-RS fully overlapps with SMTC. Then the UE shares the measurement opportunities among L3 meaurement and RLM/BFR in a ratio of 2:1 according to RAN4 common understanding. However, the TE doesn’t know whether the UE performs L3 measurement or RLM/BFR on a specific SSB occasion because it’s up to UE impementation. Therefore, it is impossible to determine whether the ACK/NACK DTX is caused by the UE's failure to comply with the RLM scheduling restriction or just by the UE's L3 measurement. The entire test is no longer testable. There are two possible ways to solve this issue: Option 1: Don’t config L3 measurement in RLM/BFR scheduling restriction TCs; Option 2: using SMTC.3 (SMTC period = 160ms) instead. Then the UE is expected to only perform RLM/BFR on SSBs outside SMTC according to RAN4 understanding. Correspondingly, the TE only needs to schedule DL before SSBs outside SMTC depicted as follows. We suggest option 2 because the UE still needs L3 measurement for SpCell tracking/DL sync/etc. Disabling L3 measurement may have unexpected effects. Same issue also exists in RedCap version RLM/BFR scheduling restriction test cases. It’s captured in CR R4-242418643. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SMTC.1 is changed to SMTC.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.5.1.9, A.7.5.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2

A.5.5.1.9.1	Test Purpose and Environment

The purpose is to verify that the NR UE correctly follows the RLM scheduling restrictions requirements defined in clause 8.1.7. This test verifies that the UE correctly receive the PDCCH scheduled on the symbols right before the RLM SSB symbols without overlap so that it sends ACK/NACK correctly. The test case is only applicable to UE which supports pdcch-MonitoringAnyOccasions or pdcch-MonitoringAnyOccasionsWithSpanGap.

Two cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and NR FR2 PSCell (Cell 2). The test parameters for NR PSCell are given in table A.5.5.1.9.1-1, table A.5.5.1.9.1-2 and table A.5.5.1.9.1-3 below and the parameters and applicability for the E-UTRAN cell are defined in A.3.7.2. The UE is required during time period T1 to transmit ACK/NACK correctly upon scheduling of PDSCH.

Table A.5.5.1.9.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD LTE, 120 kHz SSB SCS, 120 kHz RMC SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | TDD LTE, 120 kHz SSB SCS, 120 kHz RMC SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.5.5.1.9.1-2: General test parameters for RLM scheduling restriction test case in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | 1 and 2 | 1 for NR PSCell and 2 for LTE PCell |
| SSB configuration |  | 1, 2 | SSB.1 FR2 |  |
| SMTC configuration |  | 1, 2 | SMTC.3 |  |
| DRX cycle length | s | 1, 2 | OFF |  |
| T1 | s | 1, 2 | 5 | During T1 the UE is required to correctly transmit ACK/NACK |

Table A.5.5.1.9.1-3: Cell specific test parameters for RLM scheduling restriction test case in FR2

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
| AoA setup |  | 1, 2 | Setup 3 defined in A.3.15.3 |  |
|  |  |  | AoA1 | AoA2 |
| Assumption for UE beamsNote 1 |  |  | Rough | Rough |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1, 2 | 24 |  |
| PDSCH Reference measurement channel |  | 1, 2 | SR.3.2 TDD | Not sent |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 TDD | Not sent |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.2 TDD | Not sent |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD | TRS.2.2 TDD |
| PDCCH/PDSCH TCI state |  | 1, 2 | TCI.State.2 | Not sent |
| OCNG Pattern |  | 1, 2 | OP.5 defined in A.3.2.1 | Not sent |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2 | SSB with index 0 | SSB with index 1 |
|  | dBm/15kHz | 1, 2 | -92.1 | -92.1 |
| Note2 | dBm/SCS | 1, 2 | -83.1 | -83.1 |
|  | dB | 1, 2 | 2 | 2 |
| BB Note 4 | dB | 1, 2 | 1 | 1 |
| SSB_RP Note3 | dBm/SCS | 1, 2 | -81.1 | -81.1 |
| Io | dBm/95.04 MHz | 1, 2 | -54.35 | -54.35 |
| Time multiplexing of the downlink transmissions from each AoA |  | 1, 2 | Defined in Figure A.5.5.1.9.1-1 |  |
| Propagation Condition |  | 1, 2 | AWGN | AWGN |
| Note 1: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |

Figure A.5.5.1.9.1-1: Time multiplexed downlink transmissions

A.5.5.1.9.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.1.7.3.

The UE shall be continuously scheduled by PDCCH on the symbols right before each SSB which is not covered by SMTC during the entire length of T1. The UE shall transmit ACK/NACK for every scheduled PDCCH during the time duration T1.

<End of Change 1>

<Start of Change 2>

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2

A.7.5.1.9.1	Test Purpose and Environment

The purpose is to verify that the NR UE correctly follows the RLM scheduling restrictions requirements defined in clause 8.1.7. This test verifies that the UE correctly receive the PDCCH scheduled on the symbols right before the RLM SSB symbols without overlap so that it sends ACK/NACK correctly. The test case is only applicable to UE which supports pdcch-MonitoringAnyOccasions or pdcch-MonitoringAnyOccasionsWithSpanGap.

The test parameters are given in table A.7.5.1.9.1-1, table A.7.5.1.9.1-2 and table A.7.5.1.9.1-3 below. The UE is required during time period T1 to transmit ACK/NACK correctly upon scheduling of PDSCH.

Table A.7.5.1.9.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 120 kHz RMC SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.1.9.1-2: General test parameters for NR RLM scheduling restriction test case in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | 1 | 1 |  |
| SSB configuration |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  | 1 | SMTC.3 |  |
| DRX cycle length | s | 1 | OFF |  |
| T1 | s | 1 | 5 | During T1 the UE is required to correctly transmit ACK/NACK |

Table A.7.5.1.9.1-3: Cell specific test parameters for NR RLM scheduling restriction test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
| AoA setup |  | 1 | Setup 3 defined in A.3.15.3 |  |
|  |  |  | AoA1 | AoA2 |
| Assumption for UE beams Note 1 |  |  | Rough | Rough |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1 | 24 |  |
| PDSCH Reference measurement channel |  | 1 | SR.3.2 TDD | Not sent |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD | Not sent |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.2 TDD | Not sent |
| TRS configuration |  | 1 | TRS.2.1 TDD | TRS.2.2 TDD |
| PDCCH/PDSCH TCI state |  | 1 | TCI.State.2 | N/A |
| OCNG Pattern |  | 1 | OP.5 defined in A.3.2.1 | Not sent |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |
| RLM-RS |  | 1 | SSB with index 0 | SSB with index 1 |
|  | dBm/15kHz | 1 | -92.1 | -92.1 |
| Note2 | dBm/SCS | 1 | -83.1 | -83.1 |
|  | dB | 1 | 2 | 2 |
| BB Note 4 | dB | 1 | 1 | 1 |
| SSB_RP Note3 | dBm/SCS | 1 | -81.1 | -81.1 |
| Io | dBm/95.04 MHz | 1 | -54.35 | -54.35 |
| Time multiplexing of the downlink transmissions from each AoA |  | 1 | Defined in Figure A.7.5.1.9.1-1 |  |
| Propagation Condition |  | 1 | AWGN | AWGN |
| Note 1: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |

Figure A.7.5.1.9.1-1: Time multiplexed downlink transmissions

A.7.5.1.9.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.1.7.3.

The UE shall be continuously scheduled by PDCCH on the symbols right before each SSB which is not covered by SMTC during the entire length of T1. The UE shall transmit ACK/NACK for every scheduled PDCCH during the time duration T1.

<End of Change 2>