3GPP TSG-RAN WG4 Meeting # 113	R4-2418652

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.15.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

The new Work Item about RRM phase 5 was approved in [1]. One of the objectives which is Fast SCell activation for UE supporting Rel-18 EMR, which was discussed in last RAN4 meeting. In this contribution, we further provide our views on the remaining issues.

# Discussion

Based on the discussion in last meeting, RAN4 achieved following agreement [2]:

- Issue 1-1-2: Scope of fast SCell activation for UE supporting Rel-18 eEMR Agreement: - On top of Single Direct SCell Activation at SCell addition, MAC CE based Single SCell Activation is also supported. - Note: MAC CE based PUCCH SCell activation is included. - Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting Agreement: - RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. - For FR1, - [Y] equals to [5s] for direct SCell activation - FFS: Y for other SCell activation case. - For FR2, - Reuse the existing value of Y in direct SCell activation - FFS: Y for other SCell activation case. Agreement: - the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting). - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. For continued discussion in next meeting: - For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured: - For FR1: - use existing SSB based known SCell activation delay requirement with [2] SSB samples. - For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW) - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured) - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT) - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test Other proposals: - Proposal 6: For a UE supporting measValidationReportEMR or measValidationReportReselectionMeasurements, the legacy known conditions with deactivated SCell L3 measurement still apply. - Proposal 7: - In case the SCell has more than one actually transmitted SSB, the report should be based on beam level measurement result, i.e. MeasIdleConfig > MeasIdleCarrierNR > BeamMeasConfigIdle-NR > includeBeamMeasurements = true. - Proposal 8: It is feasible to introduce a new dedicated eEMR based fast SCell activation indication - The corresponding MAC-CE command could be provided together with MAC-CE for SCell activation - [Rel-18 EMR based SCell fast activation command] is received [after L3-RSRP reporting via Rel-18 EMR] and no later than the time when UE receives MAC-CE command for TCI activation - Issue 1-2-2: Consideration on SINR condition during the whole procedure for known SCell activation Agreement: - Not to update the SINR condition (i.e., Ês/Iot≥ -2dB) for SCell activation delay requirements.

The controversial and most important issue is how to modify the known conditions and corresponding UE requirements. Based on the discussion in last meeting, there are two direction for enhancement.

Direction 1: The first understanding is to have a small value for validation duration (e.g. 5 second), and UE can meet the exact the same delay as known case. The benefit is obvious that the SCell activation latency is as short as legacy known case. The drawback is that the applicable scenario is limited.

Direction 2: Another understanding which is proposed during the meeting that no need to limit the value of validation duration as long as it is configured, which could be up to 100 seconds. The enhanced SCell activation delay is not as short as known case, but it is slightly shorter than that of unknown case. Companies proposed that some SSB samples can be saved since UE can used the historical information to adjust the AGC as long as the conditions remains detectable. The benefit of the direction is that the applicable scenario is more extensive but the delay reduction gain is not that significant than direction 1.

Observation 1: There are two different direction on how to modify the known and condition and corresponding SCell activation delay.

- Direction 1: The first understanding is to have a small value for validation duration (e.g. 5 second), and UE can meet the exact the same delay as known case.

- Direction 2: Another understanding which is proposed during the meeting that no need to limit the value of validation duration as long as it is configured, which could be up to 100 seconds. The enhanced SCell activation delay is not as short as known case, but it is slightly shorter than that of unknown case

As emphasized from the first meeting, this is a three-meetings objective, which means there is no chance for complicated or over enhanced solutions. Based on above considerations, we propose following conditions with corresponding requirements:

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with [2] SSB samples when measValidationReportEMR or measValidationReportReselectionMeasurements is configured without limitation on values.

  - For FR2:

    - The SCell activation delay is define as: [2] SSB samples * beam sweeping +1 SSB sample when measValidationReportEMR or measValidationReportReselectionMeasurements is configured without limitation on values.

Proposal 1: Define SCell activation based on Rel-18 EMR with following conditions and corresponding requirements:

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with [2] SSB samples when measValidationReportEMR or measValidationReportReselectionMeasurements is configured without limitation on values.

  - For FR2:

    - The SCell activation delay is define as: [2] SSB samples * beam sweeping +1 SSB sample when measValidationReportEMR or measValidationReportReselectionMeasurements is configured without limitation on values.

Another issue is whether to have indication to network. The status is summarized as follows:

- Issue 1-2-4: Whether the indication to network is needed Proposals - Option 1: (CATT, Apple, Samsung) - Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation. - Option 2: (Ericsson, Nokia) - RAN4 to introduce a UE indication to the network. The fast or not definition can refer to the known/unknown condition that will be defined for fast Scell activation via EMR.

The necessity of having the additional indication is not well justified during the discussion in last meeting. Proponent companies want to have additional indication about whether it is a fast activation. First, the motivation is not clear. Even in Rel-15 SCell activation, the requirements defined in the spec is the minimum requirements that all UE shall met. Apparently, there could be some UE can achieve fast SCell activation even the known condition is not met. From Network perspective, there is no means/necessity to be aware of this. Besides, as clearly stated in the WI, the enhancement is based on UE supporting Rel-18 EMR, the SCell activation enhancement can leverage Rel-18 EMR reporting or as condition. But we should not have new EMR procedure/indications which is no in the scope of the WID.

Proposal 2： No need to introduce additional indication to the network for fast or slow SCell activation.

# Conclusions

Observation 1: There are two different direction on how to modify the known and condition and corresponding SCell activation delay.

- Direction 1: The first understanding is to have a small value for validation duration (e.g. 5 second), and UE can meet the exact the same delay as known case.

- Direction 2: Another understanding which is proposed during the meeting that no need to limit the value of validation duration as long as it is configured, which could be up to 100 seconds. The enhanced SCell activation delay is not as short as known case, but it is slightly shorter than that of unknown case

Proposal 1: Define SCell activation based on Rel-18 EMR with following conditions and corresponding requirements:

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with [2] SSB samples when measValidationReportEMR or measValidationReportReselectionMeasurements is configured without limitation on values.

  - For FR2:

    - The SCell activation delay is define as: [2] SSB samples * beam sweeping +1 SSB sample when measValidationReportEMR or measValidationReportReselectionMeasurements is configured without limitation on values.

Proposal 2： No need to introduce additional indication to the network for fast or slow SCell activation.

# References

[1] RP-240830 New WID: WI resulting from discussion on RRM enhancements

[2] R4-2416854 WF on NR_RRM_Ph5_Part2 - fast SCell activation