3GPP TSG-RAN WG4 Meeting # 113	R4-2418658

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.28.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impacts for IoT NTN TDD mode

Document for:	Discussion

# Introduction

The new WID of NTN IoT TDD mode was approved in [1]. RAN4 is scheduled to study the impact due to periodic pattern from RAN4#113 meeting. In this paper, we provide our views on the potential impacts on RRM impacts.

# Discussion

The objective of the WID is shown as follows:

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The study and work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band. This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24 - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

Based the WI, RAN4 shall study and evaluate the impact due to the periodic pattern for at least UE downlink synchronization.

RAN1 has started the evaluation since last meeting, within following progress:

Agreement The target operating DL CNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz: - For LEO-600, the operating DL CNR is 4.91dB (i.e. with 0dBi UE antenna gain) - For LEO-1200, the operating DL CNR is 5.51dB (i.e. with 0dBi UE antenna gain) - When reporting link level simulation results, companies are encouraged to report the CNR margin with respect to the above operating DL CNRs. - Companies may report values for other scenarios (e.g. UE antenna gain of -5.5 dBi, LEO-800) with the corresponding assumptions. Agreement For the study phase, RAN1 assumes as a baseline for evaluations (which includes DL synchronization as per the WID) that downlink PHY channels and signals are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD. - For the purpose of study, the DL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and signals and eNB will not transmit the corresponding instances. - FFS: What is the TDD pattern Agreement RAN1 evaluates via link level simulations the following: - NPSS/NSSS detection - NPBCH decoding Agreement The impact of the TDD pattern at least on the following channels / signals are evaluated at least qualitatively (e.g. specification impact, number of available instances, etc.): - SIB1-NB - Other SIBs needed for basic NB-IoT NTN operation (e.g. SIB2, SIB31) - Paging - NPRACH / RAR - NPSS/NSSS/NPBCH - NPDCCH, NPUSCH, NPDSCH - Note: Study for the above channels / signals may continue during the normative phase. - It is up to companies to perform link level evaluations for some of the aspects above Agreement For link level simulations of NPSS / NSSS / NPBCH, RAN1 uses the following assumptions: Parameter; Value. Carrier frequency; 1.6GHz. Channel model; NTN TDL-C rural, 30 degrees. Frequency error / timing drift (including XO error); 34 ppm for NPSS / NSSS (24ppm Doppler + 10ppm XO error) 0.1ppm for NPBCH. Variation of frequency error / variation of timing drift; 0.27ppm/s for NPSS / NSSS 0 for NPBCH. UE velocity; 3km/h. Target performance; For NPSS/NSSS: - 99% detection probability with 0.1% false alarm rate - FFS: How to define correct detection, including successful timing / frequency estimation / cell ID detection. For NPBCH: - 1% BLER For the channels / signals above, acquisition delay shall be reported for the corresponding detection probability. Other assumptions (e.g. combining length, frequency error hypothesis, etc.); To be reported and justified by companies. Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes): - N = 9: - D = 8 - D = 20 - D = 30 - N = 8: - D = 20 - Companies to report which subframe indices are included in the set of consecutive downlink subframes. - Other combinations are not precluded to be reported by companies NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase. Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and U (U = number of consecutive uplink subframes): - N = 9: - U = 8 - U = 20 - U = 30 - N = 8: - U = 20 - Companies to report which subframe indices are included in the set of consecutive uplink subframes. - Other combinations are not precluded to be reported by companies - For evaluations including both uplink and downlink (if any), companies are encouraged to prioritize the case of D=U NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase. Agreement For the study phase, RAN1 assumes as a baseline for evaluations that uplink PHY channels are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD. - For the purpose of study, the UL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and eNB will not receive the corresponding instances. - FFS: What is the TDD pattern

Per SID/WID description, RAN4 is tasked to evaluate the impacts due to the periodic pattern at least on UE downlink synchronization and other aspects. For downlink synchronization, following requirements may be impacted, and we provide our analysis in case by case manners:

Observation 1: Following RRM requirements shall be evaluated about the impact due to periodic pattern on DL synchronization:

- Cell reselection

- RRC Re-establishment

- Radio link monitoring

- Connected mode serving Cell measurement

- Connected mode neighbour Cell measurement

## Impacts on DL synchronization

The may concern is about the Cell detection/searching impacts due to periodic pattern. Per WID description, no blind detection is assumed at the UE side. It means there should be a SMTC like indication to locate the location of transmitted DL subframe not only for serving cell but also for neighbour cell. For Cell reselection, UE may need to search inter-frequency, UE shall be able to known the periodic pattern of target inter-frequency; Otherwise, UE needs to perform blind detection, whether the performance could be very poor.

Observation 2: For Cell Re-selection, UE shall be aware of the periodic pattern of frequency layers to be measured.

Besides, due to power saving consideration, though NPSS/NSSS are available very frame/even frame, UE is only required to search several instances per DRX [2]. As defined in [2], the number of instances to acquire per DRX is as follows:

- Number of instances (NPSS or NSSS) to acquire per DRX cycle: - 1.28s: 5 - 2.56s: 10 - 5.12s: 10 - 10.24s: 20

Thus, considering the periodic pattern, as discussed in RAN1, the available NPSS/NSSS could be 1 or 2 instance per N frames, which depends on the discussion about the D. It means, the available NPSS/NSSS instance is greatly reduced per DRX. Another aspect is that UE may needs to wake up early for synchronization for paging reception, for instance N frames before next paging occasions. Thus, it means the available time for UE to switch to other carriers for NPSS/NSSS measurement is reduced.

Observation 3: For Cell reselection requirements, RAN4 shall considered relaxed requirements considering following aspects:

- The number of available NPSS/NSSS instance are reduced per DRX

- UE may need to wake up early for synchronization for paging reception

For Cell searching requirements in RRC Re-establishment, the requirements are defined in absolute time, e..g 1400ms per frequency layer for NC and 14800ms per frequency layer for EC. It is straight forward that the requirements shall be scaled. For instance, if there is one NPSS/NSSS available every N frame, the requirements shall be scaled by at least N. Due to non-contiguous frames, the cell detection performance may be further impacted.

Observation 4: For RRC Re-establishment requirements, the search time per frequency layer shall be scaled. (e.g. if there is one NPSS/NSSS available every N frame, the requirements shall be scaled by at least N).

For RLM requirements, it is the same story when DRX is not configured. It is assumed 400/200 ms are need for OOS/INS evaluation when Rmax is smaller than 64 and 4000/2000 ms when Rmax is larger than 64. Thus, it is straightforward that the evaluation period when DRX is not configured shall be scaled.

Observation 5: For RLM, the evaluation period when DRX is not configured shall be scaled.

For measurement requirements in connected mode, which are further divided into serving cell measurement and neighbour cell measurement. For serving cell measurement and intra-frequency measurement, same situation as RLM, when DRX is not configured, the delay requirement shall be scaled.

However, for inter-frequency measurement, the delay requirements are defined in number of available samples. RAN4 defined minimum length of occasion for detection and measurement which is 200/50ms. For TDD with periodic pattern, obviously that the minimum length shall be reconsidered.

Observation 6: For connected mode serving cell measurement and intra-frequency measurement, the delay requirements shall be scaled. For inter-frequency measurement, the minimum length of measurement occasions shall be reconsidered pending on the periodic pattern defined in RAN1.

## Impacts on other RRM requirements

Apart from the requirements, for which the synchronization performance maybe impacted. There are other RRM requirements may be impacted.

For RRC Re-direction, which is for UE to be redirected from anchor carrier to non-anchor carrier, when anchor carrier and non-anchor carrier are from the same SAN node. In current requirements, there is on component about downlink subframe bitmaps, which could be 10 ms or 40 ms. For periodic pattern, the value should be extended accordingly.

Observation 7: For RRC re-direction, the periodicity of DL subframe maybe extended accordingly due to periodic pattern.

There is also RRM requirements related to demodulation performance, e.g. WUS reception and Channel quality report. Whether there is impact on above requirements depends on the evaluation/discussion in RAN1 about NPDCCH/NPDSCH, which maybe continuously discussed in normative stage.

Observation 8: Whether there is impacts on channel quality reporting depends on RAN1 conclusion on NPDCCH/NPDSCH evaluation.

Based on analysis above, the impact on RRM requirements due to periodic pattern is summarized as following tables.

Proposal 1: RAN4 to consider impacts on RRM requirements due to periodic pattern as summarized in Table I.

Table I: RRM requirements due to periodic pattern

| Requirements | Impacts |
| --- | --- |
| Cell Reselection | UE shall be aware of the periodic pattern of frequency layers to be measured. For Cell reselection requirements, RAN4 shall considered relaxed requirements considering following aspects: - The number of available NPSS/NSSS instance are reduced per DRX - UE may need to wake up early for synchronization for paging reception |
| Random Access | Whether there is impacts on channel quality reporting depends on RAN1 conclusion on NPDCCH/NPDSCH evaluation. |
| RRC Re-establishment | The search time per frequency layer shall be scaled. (e.g. if there is one NPSS/NSSS available every N frame, the requirements shall be scaled by at least N). |
| RRC Re-direction | The periodicity of DL subframe maybe extended accordingly due to periodic pattern. |
| Radio link monitoring | For RLM, the evaluation period when DRX is not configured shall be scaled. |
| Connected mode serving cell measurement | When DRX is not configured, the delay requirements shall be scaled |
| Connected mode neighbour cell measurement | For Intra-frequency, when DRX is not configured, the delay requirements shall be scaled. For inter-frequency: The length of measurement occasion should be reconsidered. |
| Channel quality report | Whether there is impacts on channel quality reporting depends on RAN1 conclusion on NPDCCH/NPDSCH evaluation. |

# Conclusions

Observation 1: Following RRM requirements shall be evaluated about the impact due to periodic pattern on DL synchronization:

- Cell reselection

- RRC Re-establishment

- Radio link monitoring

- Connected mode serving Cell measurement

- Connected mode neighbour Cell measurement

Observation 2: For Cell Re-selection, UE shall be aware of the periodic pattern of frequency layers to be measured.

Observation 3: For Cell reselection requirements, RAN4 shall considered relaxed requirements considering following aspects:

- The number of available NPSS/NSSS instance are reduced per DRX

- UE may need to wake up early for synchronization for paging reception

Observation 4: For RRC Re-establishment requirements, the search time per frequency layer shall be scaled. (e.g. if there is one NPSS/NSSS available every N frame, the requirements shall be scaled by at least N).

Observation 5: For RLM, the evaluation period when DRX is not configured shall be scaled.

Observation 6: For connected mode serving cell measurement and intra-frequency measurement, the delay requirements shall be scaled. For inter-frequency measurement, the minimum length of measurement occasions shall be reconsidered pending on the periodic pattern defined in RAN1.

Observation 7: For RRC re-direction, the periodicity of DL subframe maybe extended accordingly due to periodic pattern.

Observation 8: Whether there is impacts on channel quality reporting depends on RAN1 conclusion on NPDCCH/NPDSCH evaluation.

Proposal 1: RAN4 to consider impacts on RRM requirements due to periodic pattern as summarized in Table I.

# References

[1] RP-242415 New WID on introduction of IoT-NTN TDD mode

[2] R4-164550 WF on NB-IoT cell identification