3GPP TSG-RAN WG4 Meeting # 113	R4-2418663

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on test for Improvement on SCell/SCG setup delay

Source: 	Huawei, HiSilicon

Agenda item:	5.23.1

Document for:	Discussion

Introduction

The performance maintenance was discussed in last meeting [1]. This contribution provides further analysis on test for improvement on SCell/SCG setup delay.

Discussion

For R18 idle mode CA/DC measurement test, the below three tests are defined [2]:

- For EMR, TC verified UE does not send measurement report which is measured exceeds X second before paging reception.

- For non-EMR,

  - TC verified that UE send measurement report which is measured within X second before [paging reception].

  - TC verified that UE does not send measurement report which is measured exceeds X second before [paging reception] if it is feasible for the testing.

In RAN4#112 meeting, companies are discussing whether more test is introduced for idle mode CA/DC measurement [1].

Issue 3-2-1: More test for Improvement on SCell/SCG setup delay < Way Forward > FFS the following option: - Proposal 1 (Nokia): - Introduce test cases with valid reporting for FR1 and FR2 also for the case when the UE is configured with EMR measurements.

One proposal is to add a test case for EMR-based measurements, where the UE reports valid measurements from the duration of X, when T331 is still running. We know in this case the correct UE behaviour is that UE shall report valid result which is obtained with X window. However the question is how to differentiate good UE who is performing validity check function and wrong UE who would never perform validity check. In other words, the suggested test could not verify UE have correctly executed validity check.

Observation: The new test for EMR based UE reporting valid measurement within X could not verify whether UE correctly executed validity check correctly.

Proposal 1: Suggest not to introduce new test for Idle mode fast CA/DC eEMR measurement.

Conclusions

This contribution provides further analysis on test for improvement on SCell/SCG setup delay. The following proposals are provided:

Observation: The new test for EMR based UE reporting valid measurement within X could not verify whether UE correctly executed validity check correctly.

Proposal 1: Suggest not to introduce new test for Idle mode fast CA/DC eEMR measurement.

Reference

- R4-2413960, WF on Further NR mobility enhancements, MediaTek

- R4-2410280, WF on R18 Further NR mobility enhancement - part 2, Apple