3GPP TSG-RAN WG4 Meeting # 113	R4-2418671

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on CSI-RS based L1 measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.24.2.2

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

### 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on CSI-RS based L1 measurement.

Discussion

How CSI-RS based L1 measurement of candidate cells works

Before discussing the detailed CSI-RS based L1 measurement requirements, RAN4 shall first have an unified understanding on how CSI-RS based L1 measurement of candidate cells works. The following two alternatives on CSI-RS L1 RSRP measurement procedure were proposed in last RAN4#112bis meeting.

- Issue 2-4: whether SSB based L3 measurement is a pre-requisite of CSI-RS based L1 measurement Tentative Agreement during online session: further discuss the two alternatives Alternative 1: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1. Alternative 2: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

UE first performs SSB based L3 measurement on the neighbour cells. Through SSB based L3 measurement, the coarse timing is obtained, and then UE can correctly receive configured CSI-RS resource of candidate neighbour cells. If UE can report the measurement result with SSB index to network, it would further help network to configure proper CSI-RS resources to UE.

Proposal 1: UE performs SSB based L3 measurement and reports measurement result with SSB index of candidate neighbour cells.

For FR1, after UE correctly received configured CSI-RS resource of candidate neighbour cells, UE can perform L1-RSRP measurement on the configured CSI-RS resources with omni-directional RX antenna. Therefore alternative 1 and alternative 2 are the same for FR1.

Alternative 1

For FR2, it is a bit complicated. Alternative 1 is inherited from the principle of R15 CSI-RS based L1 measurement on serving cell. In R15, CSI-RS resource is QCLed-D with SSB for L1-RSRP measurement. It means that UE RX beam for CSI-RS based L1-RSRP measurement uses the fine SSB beam for L1-RSRP measurement. Rx beam sweeping is not applied to CSI-RS based L1-RSRP measurement when repetition is OFF.

TS38.133 section 9.5.4.2 For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for Table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set.

For R19 CSI-RS L1 RSRP measurement on candidate cell, to align with the principle in R15, UE would not sweep the RX beam for CSI-RS resources on candidate cell. The procedure of CSI-RS based L1 measurement on candidate neighbour cell shall be,

- Step 1 (gNB TX wide SSB beam, UE RX wide SSB beam): UE performs SSB based L3 measurement and acquires the SSB index information. UE reports the L3 SSB measurement results with SSB index to network.

- Step 2 (gNB TX wide SSB beam, UE RX fine SSB beam): UE performs SSB based L1-RSRP measurement on the candidate cell to refine the SSB based RX beam. Step 2 is what we specified for R18 LTM where UE sweeps RX beam based on SSB.

- Step 3 (gNB TX narrow CSI-RS beam, UE RX fine SSB beam): UE perform measurements on the configured CSI-RS resources of candidate cell where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement in step 2. Herein UE performs L1 CSI-RS based measurement without RX beam sweeping. Step 3 is similar as R15 CSI-RS based L1-RSRP measurement, the difference is step3 is on candidate cell and R15 CSI-RS based L1-RSRP measurement is on serving cell.

In last meeting, companies were discussing CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or in step 2. SSB beam in step 1 is coarse beam for L3 measurement, while SSB beam in step 2 is fine beam for L1 measurement. From RX beam reception gain perspective, it shall be QCL-typeD with SSB in step 2. In addition, from mobility perspective, as the CSI-RS L1-RSRP result on candidate cell would be compared with CSI-RS L1-RSRP on serving cell, it is supposed that the gain derived from RX reception for both measurements are the same. So QCL-typeD with SSB in step 2 is also preferred.

It is noted that if CSI-RS resource repetition is set to ON, network would fix Tx beam and UE is supposed to refine Rx beam. This procedure only happens in training RX beam for beam management in serving cell. We don’t think CSI-RS resources with repetition ON are applied for CSI-RS L1 RSRP measurement on candidate cell.

Proposal 2: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

Alternative 2

Alternative 2 skipped step 2 which is in alternative 1. However the whole procedure of alternative 2 is not very clear. For step 3, there are two understandings:

- Step 3- understanding 1 (gNB TX narrow CSI-RS beam, UE RX fine CSI-RS beam): After step 1(UE performs SSB based L3 measurements and acquire SSB index information on the candidate cells), UE performs L1 CSI-RS measurement with RX beam sweeping.

- Step 3-understanding 2 (gNB TX narrow CSI-RS beam, UE RX coarse SSB beam): After step 1(UE performs SSB based L3 measurements and acquire SSB index information on the candidate cells), UE uses the coarse SSB beam for L3 measurement in step 1, and performs L1 CSI-RS measurement without RX beam sweeping.

If it is understanding 1, as typically CSI-RS resources for measurement are associated with SSB, UE can sweep Rx beam with CSI-RS resources within the associated SSB beam range. Through CSI-RS based RX beam sweeping, the CSI-RS level finer Rx beam can be obtained in UE reception side.  Although the implementation of alternative 2-understanding 1 can get the higher RX beam reception gain, some outstanding drawbacks are observed:

1. With RX beam sweeping on CSI-RS resource of neighbour cell, the measurement delay of neighbour cell would be quite longer (N times) than that of serving cell where typical L1 measurement periodicity is 20ms for serving cell. As we know, in FR2 environment, the channel changes quickly. If UE is moving to the cell edge, the RSRP on serving cell would react channel degradation faster than that on neighbour cell. It is most likely that UE will switch to neighbour cell. However when “neighbour cell” becomes serving cell, RSRP fluctuation of the current serving cell will be intense, and UE may quickly switch back. In a word, different measurement periodicity between neighbour cell and serving cell may result in “ping-pong” handover.

2. As we know, there is no RX beam sweeping for L1 CSI-RS measurement on serving cell when repetition is OFF. UE uses SSB level RX beam to perform CSI-RS measurement (The QCL-D in qcl-InfoPeriodicCSI-RS is SSB). If L1 CSI-RS measurement on neighbour cell is executed with RX beam sweeping, the RX beam gain on serving cell and on neighbour cell would be different. Therefore the RSRP comparison between serving cell and neighbor cell will be somewhat unfair.

If it is understanding 2, after step 1, there is no RX beam sweeping and UE use the coarse SSB beam for L3 measurement in step 1. The RX beam reception gain from alternative 2-unstanding 2 would be lower than that in alternative 1. The measurement result with understanding will be worse.

Based on the above analysis, whether it is understanding 1 or understanding 2, alternative 2 has some outstanding drawbacks. Therefore we think the measurement procedure for CSI-RS based LTM candidate cell measurements should be alternative 1.

Proposal 3: RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

- Step 1(gNB TX wide SSB beam, UE RX wide SSB beam): SSB based L3 measurements and reports with SSB index

- Step 2(gNB TX wide SSB beam, UE RX fine SSB beam): For FR2-1, SSB-based L1 measurements with beam sweeping and reports on the same candidate cell

Note 1: Step 2 is the procedure of R18 SSB based L1 RSRP measurement on candidate cell

Note 2: Step 2 is removed for FR1.

- Step 3(gNB TX narrow CSI-RS beam, UE RX fine SSB beam): CSI-RS based L1 measurements on the same candidate cell (without RX beam sweeping)

Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 2.

Know cell definition

It is agreed in last meeting that RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells.

- Agreement: - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells

The open issue is how to define known cell in CSI-RS based L1 measurement for candidate cells [2].

- Issue 2-2: known cell definition in CSI-RS based L1 measurement For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

The cell known definition is highly depends on the framework of CSI-RS based L1 measurement on candidate cells. Based on Proposal 6, the following definition of known cell is proposed.

Proposal 4: For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

- The UE has performed L3 measurement on the target cell during the last 5 seconds, and

- The SSB from the target cell configured for L3 and L1 measurement remains detectable according to the cell identification requirements in clause 9.2.

Note: L1 measurement in the 2nd bullet is not needed for FR1.

- The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable].

Intra-frequency CSI-RS based L1 RSRP measurement definition

In last meeting, the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement was discussed and unfortunately no consensus was reached.

The legacy L3 CSI-RS resources are included in MeasObjectNR, and the CSI-RS resources are defined as cell-specific. In details, in one MO, there are multiple cells which have the same SSB frequency and the same SCS. In each cell, the L3 CSI-RS resources have the same CSI-RS center frequency, bandwidth and density.

Observation 1: For L3 CSI-RS resources which are included in MeasObjectNR, in each cell, the L3 CSI-RS resources have the same CSI-RS center frequency, bandwidth and density.

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

<omit unrelated>

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

CSI-RS-ResourceConfigMobility ::=   SEQUENCE {

subcarrierSpacing                   SubcarrierSpacing,

csi-RS-CellList-Mobility            SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility,

...,

[[

refServCellIndex                    ServCellIndex                                                           OPTIONAL    -- Need S

]]

}

CSI-RS-CellMobility ::=             SEQUENCE {

cellId                              PhysCellId,

csi-rs-MeasurementBW                SEQUENCE {

nrofPRBs                            ENUMERATED { size24, size48, size96, size192, size264},

startPRB                            INTEGER(0..2169)

},

density                             ENUMERATED {d1,d3}                                                      OPTIONAL,   -- Need R

csi-rs-ResourceList-Mobility        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility

}

Based on L3 CSI-RS resource signalling design, the L3 CSI-RS based intra-frequency measurement is specified as below, considering the same center frequency, SCS and CP type between serving cell and neighbour cell.

< TS38.133> A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement

For R19 L1 CSI-RS resource measurement configuration on neighbor cell, the below agreements were achieved in the past August RAN2 meeting.

5.   R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

In RAN2#127bis meeting, RAN2 further agreed that R18 LTM CSI resource configuration is reused for CSI-RS based L1 measurement resource configuration on candidate cell.

RAN2 agreements in RAN2#127bis

For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

As per above RAN2 signalling design philosophy, the R19 L1 CSI-RS measurement resources of candidate cells is to reuse existing NZP-CSI-RS-Resource structure in LTM-config rather than in MeasObjectNR. The NZP-CSI-RS-Resource is duplicated as below.

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]]

}

CSI-RS-ResourceMapping ::=          SEQUENCE {

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12)),

row4                                BIT STRING (SIZE (3)),

other                               BIT STRING (SIZE (6))

},

nrofPorts                           ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

firstOFDMSymbolInTimeDomain2        INTEGER (2..12)                                                         OPTIONAL,   -- Need R

cdm-Type                            ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

density                             CHOICE {

dot5                                ENUMERATED {evenPRBs, oddPRBs},

one                                 NULL,

three                               NULL,

spare                               NULL

},

freqBand                            CSI-FrequencyOccupation,

...

}

CSI-FrequencyOccupation ::=         SEQUENCE {

startingRB                          INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs                             INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

...

}

It is observed that one candidate neighbour cell can have multiple L1 CSI-RS resources and the center frequency and bandwidth of these CSI-RS resources can be different.

Observation 2: There may have more than one L1 CSI-RS resources configured in each candidate neighbour cell, and the center frequency and bandwidth of these L1 CSI-RS resources can be different.

The L1 CSI-RS resource configuration in serving cell uses NZP-CSI-RS-Resource. It can be observed that there may have also more than one NZP-CSI-RS resources configured in the serving cell. One thing needs to be pointed out the SCS of CSI-RS resources is not present in NZP-CSI-RS-Resource. The SCS of CSI-RS resource of serving cell refers to the SCS of the active BWP. It means that the center frequency of CSI-RS resources configured in the serving cell may change with the active BWP.

Observation 3: In serving cell, there may have more than one L1 CSI-RS resources configured and the center frequency, bandwidth and SCS of a L1 CSI-RS resource would change with the active BWP switching.

According to L3 CSI-RS intra/inter -frequency definition, the center frequency of CSI-RS resource of neigbhour cell is compared with the center frequency of CSI-RS resource of serving cell where the center frequency of each CSI-RS resource in serving cell is fixed. The advantage of the L3 CSI-RS intra/inter -frequency definition is that a measurement is always regarded as an intra-f or inter-f measurement, and does not change with active BWP switching.

The situation seems somewhat different for R19 L1 CSI-RS measurement. According to observation 3, the center frequency of CSI-RS resource of serving cell (which is the reference point in intra-f definition) is changing along with active BWP switching. Therefore the advantage of defining intra-f measurement based on center frequency is lost. Then we think alternative based on active BWP can be considered.

If BW of NZP-CSI-RS resource(s) of a candidate cell is within DL active BWP of the serving cell (e.g., case 1 and case 2), no gap is needed as well. If BW of NZP-CSI-RS resource(s) of a candidate cell is not within DL active BWP of the serving cell (e.g., case 3 and case 4), gap is needed. Regarding case 5, although BW of NZP-CSI-RS resource(s) of a candidate cell exceeds active BWP of serving cell, UE may can intercept parts of CSI-RS resource of candidate cell. RAN4 can further discuss the definition of case 5.

Figure 1. Intra-f/inter-f definition for L1 CSI-RS measurement on candidate cells

Proposal 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell.

Further discuss on the case when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

Measurement with/without gap

Another open issue is whether to support CSI-RS based L1 measurement outside active BWP or with measurement gap.

- Issue 2-3: CSI-RS based L1 measurement outside active BWP or with measurement gap - Candidate solutions - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia) - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi) - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia) - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT) - Without gap - With gap - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC) - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT) - Option 2c: CSI-RS based L1 measurements for candidate cells shall support - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE)

After RAN4 reach conclusion on the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, the supported measurement types can be discussed. If the intra-f definition follows proposal 8, it is straight forward that intra-frequency measure is performed without gap and inter-frequency is performed with gap.

Proposal 6: Prioritize intra-frequency without gap and inter-frequency with gap for CSI-RS based L1 measurement on neighbour cell.

Applicability

When CSI-RS based L1-RSRP measurements are outside active BWP, measurement gap is needed. During measurement gap occasion, UE would retune to the centre frequency of the target CSI-RS resources to execute measurements. Within one measurement gap occasion, only the CSI-RS resources with the same center frequency, the same SCS and the same CP type are to be measured. We don’t think all CSI-RS resources outside BWP with different center frequency can be measured within one measurement gap occasion.

Proposal 7: Only the CSI-RS resources with the same center frequency, SCS and CP type can be measured within one measurement gap occasion.

The CSI-RS resources from serving cell and different candidate cells which have the same centre frequency, but different SCS or CP length, UE is not able to measure them within one gap occasion. To simplify measurement requirements, RAN4 can specify measurement applicability that the requirements apply all CSI-RS resources on candidate cells on the same layer are configured with the SCS and CP type.

Proposal 8: The requirements apply all CSI-RS resources on candidate cells on the same layer are configured with the SCS and CP type.

Imitating R18 LTM CSI resource configuration structure, each CSI-RS resource on candidate cell can have flexible periodicity and time offset. From measurement perspective, it is expected that the measurement opportunity of the CSI-RS resource outside active BWP are covered in measurement gap. So there are some implicit restriction on CSI-RS resources configuration. e.g., on time domain the CSI-RS resources outside active BWP shall be aligned with measurement gap duration.

As we know, active BWP may be dynamically switched by DCI. While CSI-RS resource configuration is configured by RRC. So the same CSI-RS resource would be measured dynamically with measurement gap and without gap. So the restriction on CSI-RS resources outside active BWP shall be extended for within active BWP scenario. It shall be noted that such kind of restriction has no specification impact as these are up to network configuration.

Proposal 9: There are some implicit restrictions on CSI-RS resources configuration, e.g., the CSI-RS resources on time domain are supposed to be aligned with measurement gap. However this is network configuration, and there is no specification impact.

Conclusion

The contribution provides analysis on CSI-RS based L1 measurement. The following proposals are proposed:

Proposal 1: UE performs SSB based L3 measurement and reports measurement result with SSB index of candidate neighbour cells.

Proposal 2: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

Proposal 3: RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

- Step 1(gNB TX wide SSB beam, UE RX wide SSB beam): SSB based L3 measurements and reports with SSB index

- Step 2(gNB TX wide SSB beam, UE RX fine SSB beam): For FR2-1, SSB-based L1 measurements with beam sweeping and reports on the same candidate cell

Note 1: Step 2 is the procedure of R18 SSB based L1 RSRP measurement on candidate cell

Note 2: Step 2 is removed for FR1.

- Step 3(gNB TX narrow CSI-RS beam, UE RX fine SSB beam): CSI-RS based L1 measurements on the same candidate cell (without RX beam sweeping)

Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 2.

Proposal 4: For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

- The UE has performed L3 measurement on the target cell during the last 5 seconds, and

- The SSB from the target cell configured for L3 and L1 measurement remains detectable according to the cell identification requirements in clause 9.2.

Note: L1 measurement in the 2nd bullet is not needed for FR1.

- The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable].

Observation 1: For L3 CSI-RS resources which are included in MeasObjectNR, in each cell, the L3 CSI-RS resources have the same CSI-RS center frequency, bandwidth and density.

Observation 2: There may have more than one L1 CSI-RS resources configured in each candidate neighbour cell, and the center frequency and bandwidth of these L1 CSI-RS resources can be different.

Observation 3: In serving cell, there may have more than one L1 CSI-RS resources configured and the center frequency, bandwidth and SCS of a L1 CSI-RS resource would change with the active BWP switching.

Proposal 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell.

Further discuss on the case when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

Proposal 6: Prioritize intra-frequency without gap and inter-frequency with gap for CSI-RS based L1 measurement on neighbour cell.

Proposal 7: Only the CSI-RS resources with the same center frequency, SCS and CP type can be measured within one measurement gap occasion.

Proposal 8: The requirements apply all CSI-RS resources on candidate cells on the same layer are configured with the SCS and CP type.

Proposal 9: There are some implicit restrictions on CSI-RS resources configuration, e.g., the CSI-RS resources on time domain are supposed to be aligned with measurement gap. However this is network configuration, and there is no specification impact.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1), Apple