3GPP TSG-RAN WG4 Meeting # 113	R4-2418674

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on impacts of RAN4 requirements in AI/ML mobility

Source: 	Huawei, HiSilicon

Agenda item:	7.18.2

Document for:	Discussion

Introduction

The revised SID on AI/ML for mobility in NR agreed in [1]. The objectives are duplicated as below.

### 4.1 Objective of SI or Core part WI or Testing part WI The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In RAN4#112bis meeting, RAN4 started measurement prediction study, and will carry out the discussion on measurement event prediction and HO failure/RLF prediction till 2025 [2].

Issue 1-1-1: Priorities of different use cases Agreement: - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting. Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

This contribution focuses on RRM measurement prediction in AI/ML mobility.

Discussion

In R18 study item phase on “AI/ML for NR air interface”, the general functional framework for AI/ML for NR air interface is illustrated in below figure (duplicated from TR38.843[4]). The functional architecture and high-level procedure can be regarded as a baseline for AI/ML aided mobility.

Figure 4.4-1: Functional framework for AI/ML for NR Air Interface [TR 38.843]

There is a parallel R19 WI on “AI/ML for NR air interface” which focus on use cases of CSI compression, beam management and positioning. As per the revised SID on AI/ML mobility [1], the work from “AI/ML for NR air interface” shall be leveraged as much as possible.  In RAN4 progress on R19 AI/ML for NR air interface, the following agreements are achieved in [3]. These agreements provide a good guidance on RRM core requirements definition. In the following, the RRM requirements for R19 AI/ML mobility are provided.

Issue 1-4: Inference latency handling Agreement: - Use the following as the starting point - Inference latency can be implicitly captured in the delay of corresponding procedures (e.g. reporting delay) and/or performance requirements. - Inference latency is part of delay of the corresponding procedure and no separate requirement will be specified for inference latency. - Check the above bullets based on per use case, and if other WGs define the inference latency, RAN4 can revisit the above bullets. Issue 1-5: LCM Requirements Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded Issue 1-6: Legacy RRM requirements handling Agreement: - UE shall meet all legacy RRM requirements (non-AI/ML) even during the AI/ML operation mode.

Data collection requirements

Data collection function provides input data to model training, management and inference function. If new measurement quantity is introduced in AI/ML mobility, new requirements would be specified accordingly, otherwise no RRM requirements on data collection is observed.

Proposal 1: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed.

Inference requirements

Measurement requirements for RRM measurement prediction

In RAN2, the evaluation scenarios for measurement prediction with prioritization are listed in [TR38.744].

Table 5.2.1-1 prioritization of evaluation scenarios [TR38.744]

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

There are two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption) [5]. Case A and case B in intra-frequency temporal domain are roughly defined as below in RAN2.

Agreements in RAN2#126 Chairman notes 7 To agree on methodology of Intra_F_C_T_Case A as following: Intra-frequency intra-cell temporal domain case A prediction is done by predicting measurement result(s) in prediction window based on measurement results in observation window of the same cell for both FR1_to_FR1 and FR2_to_FR2. FFS aligning the prediction window 8 To agree on methodology of Intra_F_C_T_Case B: Intra-frequency intra-cell temporal domain prediction is done by predicting sub set measurement (case B) instances in temporal domain of the same cell for both FR1_to_FR1 and FR2_to_FR2. Several measurement reduction rates should be aligned among companies. The detail values for measurement reduction rate are FFS.

In RAN4#112bis, RAN4 agreed to align with RAN2 and prioritize the study on scenario 2, 3 and 4.

Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Agreement: - Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

We know temporal domain case A and case B have different working mechanism. For both case A and case B, OW (observation window) and PW (prediction window) are introduced [5].

Figure 5.2.1-1 Example of intra-frequency temporal domain case A [TR38.744]

In intra-cell temporal domain prediction case A, observation window is a duration UE performs actual measurement. Within an observation window, the measurement period shall follow the legacy measurement period. The prediction window is a duration UE predicts measurement. Observation window and prediction window will slide. It means that the instances in previous prediction window would become part of the observation window when UE perform measurement in predicted instances. It shall be noted the observation window length depends on model performance, channel environment and observation length, etc. The sliding window in case A is similar as the evaluation period in legacy RLM requirement. From defining requirement perspective, a prediction window length for predication (or called as prediction evaluation period) may be needed.

It is straight forward that the prediction would result in measurement result difference between the actual measured RSRP and predicted RSRP. The prediction accuracy would be evaluated and defined in RAN4 RRM.

Proposal 2: For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified.

Proposal 3: For intra-frequency temporal domain prediction case A, the RSRP prediction accuracy would be specified.

In intra-cell temporal domain prediction case B, the measurements in PW are predicted and these time instances are completely skipped by UE. RAN2 provide two examples duplicated as below, and example 2 is recommended as baseline for evaluation.

Figure 5.2.1-2 Example 1 of intra-frequency temporal domain case B [TR38.744]

Figure 5.2.1-3 Example 2 of intra-frequency temporal domain case B [TR38.744]

Measurement Reduction Rate in temporal domain (MRRT) is used for case B in RAN2 evaluation work.

MRRT = skipped measurement time instances / total measurement time instances

It is observed that,

1. Under different MRRT (e.g., 1:1, 2:3), the prediction accuracy may be different. RAN4 may need to specify prediction accuracy under the side condition of MRRT.

2. From the two above examples of the illustration, MRRT for both examples are the same, that is 0.5. However the prediction error may be not the same, even with the same MRRT value. Taking an extreme example, following the predication strategy like example 1, using100 measurements in OW predict 100 measurements in PW, MRRT is still 0.5. However the prediction accuracy of the sample in the farthest instance in PW would be degraded. Therefore only MRRT is not able to cover all kinds of skipping patterns in case B. The accuracy requirements are to be specified for certain MRRT with some limitations, e.g., PW length.

Proposal 4: For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT with limited PW length may need to be specified.

For inter-frequency inter-cell frequency domain measurement, the idea is to leverage the RSRP result on one cell to infer the RSRP on another cell in another frequency. The prediction accuracy in frequency domain would be also evaluated and defined in RAN4 RRM. Even with frequency prediction model, it is not possible that the measurement on any frequency layer can be inferred. Therefore side conditions, e.g., EPRE difference, need to be specified.

Proposal 5: For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy may need to be specified.

The legacy measurement reporting delay is defined as a delay for UE transmitting the measurement report. In AI/ML mobility, the inference function used for prediction may need some times. Therefore the processing time of inference function may need to be considered in measurement reporting delay.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

Proposal 6: The processing time of inference function may need to be considered in measurement reporting delay.

Metrics for measurement prediction accuracy

In TR38.744, 3 sub-use cases for cell-level measurement prediction and 3 sub-use cases for beam level measurement prediction are considered, where L3 filtered beam level prediction sub-use cases are lower priority. To align with RAN2, RAN4 can focus on the first 3 cell level sub-use cases with high priority.

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) 3 sub-use cases are considered for beam-level RRM measurement prediction: Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: L3 filtered beam level prediction sub-use cases are lower priority and should focus on FR2 intra-frequency temporal domain case A

To facilitate discussion, the high-level measurement model in TS38.300 without AI is duplicated as below. In RAN4 RRM, the legacy measurement accuracy is defined at point A1, where A1 is measurements (i.e. beam specific measurements) reported by layer 1 to layer 3 after layer 1 filtering.

Figure 9.2.4-1: Measurement Model [in TS38.300]

For AI mobility, in RAN2 evaluation prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all sub cases. From RAN4 perspective, as the final measurement results of the 3 sub-use cases for cell-level RRM measurement prediction are L3 cell-level, the measurement prediction accuracy can be defined at point C.

Proposal 7: For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C.

In legacy measurement accuracy, the predication accuracy is derived from the RSRP difference between measured measurement and the ideal RSRP measurement. The ideal RSRP does not include any channel estimation noise.For AI mobility, the similar accuracy definition can be reused. The predication accuracy shall be the RSRP difference between predicted L3 cell level measurement and the ideal L3 cell level RSRP measurement. The ideal L3 cell level RSRP measurement does not include any channel estimation noise.

Proposal 8: Predication accuracy is the RSRP difference between predicted L3 cell level measurement and the ideal L3 cell level RSRP measurement. The ideal L3 cell level RSRP measurement does not include any channel estimation noise.

In sub-use case 1, the prediction error is generated from actual L1 beam level predicting L1 beam level measurement (prediction is executed at point A1).

In sub-use case 2, the prediction error is generated from actual L3 beam level predicting L3 beam level measurement (prediction is executed at point C).

In sub-use case 3, the prediction error is generated from actual L1 beam level predicting L3 beam level measurement (measurement result at point A1 predicting measurement result at Point C).

In the three cases, the inference model is used at different position in the measurement process. The prediction model algorithm would be various due to different input and output. In theory the prediction accuracy evaluated at point C may be different for 3 sub-use cases. Then the question is whether RAN4 needs to define prediction accuracy for each sub-use case or RAN4 only define one cell level prediction accuracy requirement to cover 3 sub-use cases.

If use-case specific prediction accuracy is defined, UE needs to know which prediction accuracy is to be satisfied. Then network shall indicate which the sub-use case is. The motivation of such indication needs to be justified. RAN4 can wait for RAN2 progress.

If it is expected to define one unified prediction accuracy, based on simulation results RAN4 can select one sub-use case which has proper or acceptable prediction accuracy, or RAN4 can determine one prediction accuracy to cover all sub-use cases. The prerequisite of both ways shall be based on simulation results.

Proposal 9: RAN4 to discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

Impacts of Measurement errors on the prediction accuracy

It is observed that the RAN2 performed simulations for verifying the RSRP difference in AI RRM measurement prediction use case. Both the training dataset and inference dataset are generated from system level simulations. In all the cases, ideal RSRP is used as the model input, without considering the measurement impairment. Since in practical, only the non-ideal RSRP can be obtained, the impact of the measurement impairment needs to be captured when we verify the actual prediction accuracy enabled by AI.

Observation 1: Measurement impairment may impact the prediction accuracy.

Observation 2: The RSRP with measurement impairment can be obtained in reality, rather than the ideal RSRP.

Observation 3: In RAN2 assumption, the ideal RSRP is assumed both for model input and the label of model output.

Proposal 10: RAN4 to discuss the measurement impairment considered for verifying the prediction accuracy enabled by AI.

Legacy measurement accuracy requirement is performed based on link level simulation. For AI mobility, we try to discuss the methodology of prediction accuracy from link level simulation perspective.

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

According to Proposal 7 and 8, the measurement prediction accuracy shall be defined at point C. RSRP prediction accuracy is the difference between predicted L3 cell level measurement result and L3 cell level reference/ideal result of the same cell.

Ideal RSRP at point C: input is at point A and output is at C. UE would perform the whole measurement procedure as illustrated in Figure 9.2.4-1 Measurement Model [in TS38.300] (i.e., Input from point A, experiencing channel without noise=> ideal RSRP at point A1=> ideal RSRP at point B=> ideal RSRP at point C). It shall be noted that ideal RSRP does not include any channel estimation noise.

Predicted RSRP at point C:

- Option 1: Input from point A, experiencing channel with noise=> measured RSRP at point A1=>predicted RSRP at point A1=> generated RSRP at point B => generated RSRP at point C

- Option 2: Input from point A, experiencing channel without noise=> ideal RSRP at point A1=>predicted RSRP at point A1=> generated RSRP at point B=> generated RSRP at point C

Where measurement impairment due to channel estimation is considered in option 1 but not in option 2. The prediction function is performed at point A1.

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

Ideal RSRP at point C: input is at point A and output is at C. Ideal RSRP does not include any channel estimation noise.

Estimated RSRP at point C:

- Option 1: Input from point A, experiencing channel with noise=> measured RSRP at point A1 => measured RSRP at point B=> measured RSRP at point C=>predicted RSRP at point C

- Option 2: Input from point A, experiencing channel without noise=> ideal RSRP at point A1 => ideal RSRP at point B=> ideal RSRP at point C=>predicted RSRP at point C

Where measurement impairment due to channel estimation is considered in option 1 but not in option 2. The prediction is performed at point C.

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Ideal RSRP at point C: input is at point A and output is at C. Ideal RSRP does not include any channel estimation noise.

Predicted RSRP at point C:

- Option 1: Input from point A, experiencing channel with noise=> measured RSRP at point A1 =>predicted RSRP at point C

- Option 2: Input from point A, experiencing channel without noise=> ideal RSRP at point A1 => predicted RSRP at point C

Where measurement impairment due to channel estimation is considered in option 1 but not in option 2. The RSRP at point C is predicted based on measurement at Point A.

For AI model training and inference, only the non-ideal RSRP can be obtained. If RAN4 define the prediction accuracy requirements without considering measurement error due to channel estimation, UE would fail to pass the testing. Therefore we prefer option 1 of the estimated RSRP at point C for all 3 sub-use cases.

Proposal 11: The methodology of prediction accuracy from the link level simulation perspective is,

Ideal RSRP at point C:

- Input is at point A and output is at C (i.e., Input from point A, experiencing channel without noise=> ideal RSRP at point A1=> ideal RSRP at point B=> ideal RSRP at point C).

- Ideal RSRP does not include any channel estimation noise.

Estimated RSRP at point C:

- Sub-use case 1

Input from point A, experiencing channel with noise=> measured RSRP at point A1=>predicted RSRP at point A1=> generated RSRP at point B=> generated RSRP at point C

- Sub-use case 2:

Input from point A, experiencing channel with noise=> measured RSRP at point A1 => measured RSRP at point B=> measured RSRP at point C=>predicted RSRP at point C

- Sub-use case 3:

Input from point A, experiencing channel with noise=> measured RSRP at point A1 =>predicted RSRP at point C

LCM requirements

In management function (LCM procedure), the functionality/model activation/deactivation/fallback to non-AI function are to be considered. If the functionality/model activation/deactivation/fallback to non-AI function are indicated by network, e.g., MAC CE/DCI triggered, the delay of activating/deactivating one function would be defined. The side conditions of activation/deactivation/fallback delay may also be discussed. Otherwise if these functionality/model changes are autonomously performed by UE, UE monitors the function performance, then the requirements would be related with monitoring requirements.

Proposal 12: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified.

Proposal 13: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements.

Performance Monitoring requirements

If performance monitoring is mainly executed at UE side, some metrics are to be defined so that UE itself can make decision of activation, deactivation, or fallback to non-AI. Then from RRM requirement perspective, the requirements of monitoring periodicity or reporting delay when certain metric is met may need to be specified.

If performance monitoring is executed at network, the monitoring function is up to network implementation. If new measurement quantity is introduced, new requirements would be specified, otherwise no RRM requirements on monitoring is observed.

As performance monitoring are under discussion in RAN1 R19 AI/ML air interface, RAN4 can wait for more progress.

Proposal 14: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface.

Conclusion

The contribution provides analysis on RRM measurement prediction in AI/ML mobility. The following proposals are proposed:

Proposal 1: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed.

Proposal 2: For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified.

Proposal 3: For intra-frequency temporal domain prediction case A, the RSRP prediction accuracy would be specified.

Proposal 4: For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT with limited PW length may need to be specified.

Proposal 5: For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy may need to be specified.

Proposal 6: The processing time of inference function may need to be considered in measurement reporting delay.

Proposal 7: For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C.

Proposal 8: Predication accuracy is the RSRP difference between predicted L3 cell level measurement and the ideal L3 cell level RSRP measurement. The ideal L3 cell level RSRP measurement does not include any channel estimation noise.

Proposal 9: RAN4 to discuss whether to define sperate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

Observation 1: Measurement impairment may impact the prediction accuracy.

Observation 2: The RSRP with measurement impairment can be obtained in reality, rather than the ideal RSRP.

Observation 3: In RAN2 assumption, the ideal RSRP is assumed both for model input and the label of model output.

Proposal 10: RAN4 to discuss the measurement impairment considered for verifying the prediction accuracy enabled by AI.

Proposal 11: The methodology of prediction accuracy from the link level simulation perspective is,

Ideal RSRP at point C:

- Input is at point A and output is at C (i.e., Input from point A, experiencing channel without noise=> ideal RSRP at point A1=> ideal RSRP at point B=> ideal RSRP at point C).

- Ideal RSRP does not include any channel estimation noise.

Estimated RSRP at point C:

- Sub-use case 1

Input from point A, experiencing channel with noise=> measured RSRP at point A1=>predicted RSRP at point A1=> generated RSRP at point B=> generated RSRP at point C

- Sub-use case 2:

Input from point A, experiencing channel with noise=> measured RSRP at point A1 => measured RSRP at point B=> measured RSRP at point C=>predicted RSRP at point C

- Sub-use case 3:

Input from point A, experiencing channel with noise=> measured RSRP at point A1 =>predicted RSRP at point C

Proposal 12: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified.

Proposal 13: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements.

Proposal 14: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface.

Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- R4-2416855, WF on AI/ML Mobility, Nokia

- R4-2401570, WF on AI/ML, Qualcomm Incorporated

- TR38.843

- TR38.744