# 3GPP TSG-RAN WG4 Meeting #113                               R4-2418685

# Orlando, US, 18th – 22nd November, 2024

# Title: 	Discussion on UE Capability/UE behavior and network signaling for 	Type 4a Type 4b EN-DC/NR-CA

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.2.2

# Document for:	Approval

# 1 Introduction

This contribution shares our views about UE Capability/UE behavior and network signaling type 4a and type 4b.

- Discussion

- < Issue 2-3: BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated) > Way Forward: - Continue further discussion on BS signalling in the next meeting. - < Issue 2-4: BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) > Way Forward: - Continue further discussion on BS signalling in the next meeting.

Some agreements are reached in RAN4 #112bis meeting [2] and some UE behaviour/NW signaling related issues need further discussion, one issue is the BS signaling to switch UE type with different type 1.

Figure 2-1 shows the UE types and the corresponding fallback transmission.

Figure 2-1 The UE type and the corresponding fallback scheme

If a UE only support the capability of UE type 2, it can fallback to type 1 with total 4 layers transmission when NW signaling(nonCollocatedTypeMRDC-r18 or nonCollocatedTypeNR-CA-r18) is configured to the UE. For a UE supporting both UE type 2 and UE type 4, the fallback can be type 1 with 4 layers, type 1 with 6 layers or type 1 with 8 layers.

For Rel-18 NW, only type 2 can be identified and the fallback transmission is type 1 with 4 layers even type 4 is reported by a Rel-19 UE.

For Rel-19 NW, type 2 and type 4 can be identified if both UE types are reported by a UE.

A network signaling can indicate a UE to fallback to type 1 transmission. Rel-19 type 4 UE should also support the Rel-18 type 1 UE with 4 layers per CC considering the maxMIMO layer is up to 8 for Rel-19 type 4 UE.

Following the scheme of Rel-18 type 2, switching from type 4 to type 1 requires NW signaling. When the NW signaling appears type 1(co-located) is configured, when the NW signaling absents type 4(non-collocated) is assumed. Combined with maxMIMO-layers, type 4 can switch to type 1 4 layers/CC, 6 layers/CC and 8 layers/CC.

Proposal 1. The new NW signalling should consider type 4 UE ‘fallback’ to R18/R19 Type 1 UE with 4 layers/CC, 6 layers, 8 layers/CC.

Issue 2-6 needs further discussion for the UE type determination.

- < Issue 2-6: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b > Way Forward: - Continue further discussion on BS signalling in the next meeting.

As UE can not recognize NW release, the initial UE type needs to be clarified for different NW release. Figure 2-2 shows the procedure of UE type determination for different cases.

Figure 2-2 UE type determination for different NW release and maxMIMO-layers

To simplify the discussion, we assume that there will be no conflict between maxMIMO-layers and the explicit NW signaling. The type 2 related UE capability and NW signaling are appended in the annex for reference.

It can be seen in figure 2-2 that due to UE's inability to recognize NW release the UE type need to be clarified when no explicit NW is configured.

Figure 2-2(b) and figure 2-2(e) show that type 2/4 are reported to the NW and NW signaling is not provided and maxMIMO-layers is set to 4. For Rel-18 NW, the combination of NW signaling absence and maxMIMO-layers=4 is not expected as maxMIMO-layers=4 is not supported for UE type 2. Similarly, when Rel-19 NW signaling is not provided, maxMIMO-layers is not expected to be configured larger than 4.

Proposal 2. To consider the following UE type when UE capability of UE type2 and type 4 are reported to NW and NW signaling is not provided:

if maxMIMO-layer = 4, type 4 is assumed;

if maxMIMO-layer = 2, type 2 is assumed;

maxMIMO-layer is not expected to be configured larger than 4.

# 3 Conclusion

In this contribution, we give our viewpoint about requirements for type 4a/4b for NonCol_intraB_ENDC_NR_CA.

The conclusions are:

Proposal 1. The new NW signalling should consider type 4 UE ‘fallback’ to R18/R19 Type 1 UE with 4 layers/CC, 6 layers, 8 layers/CC.

Proposal 2. To consider the following UE type when UE capability of UE type2 and type 4 are reported to NW and NW signaling is not provided:

if maxMIMO-layer = 4, type 4 is assumed;

if maxMIMO-layer = 2, type 2 is assumed;

maxMIMO-layer is not expected to be configured larger than 4.

# 4	Reference

- RP-240838, Rel-19 WID_Intra-band non-collocated EN-DC and NR-CA, Apple

- R4-2417115,WF on rest issues for intra-band non-collocated EN-DC/NR-CA, KDDI

# Annex

nonCollocatedTypeMRDC-r18 (NW signaling) This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16.

nonCollocatedTypeNR-CA-r18 (NW signaling) This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NRCA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

interBandMRDC-WithOverlapDL-Bands-r16(UE capability) -- R4 2-19 FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum Indicates whether the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [5] and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 [5] and inter-band RF requirements. If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [5] and intra-band RF requirements.

requirementTypeIndication-r18(UE capability) -- R4 33-2: Support network control of requirement network applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 Indicates whether the UE supports network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeMRDC-r18 for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands as specified in TS 38.331 [9]. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16.

intraBandNR-CA-non-collocated-r18 (UE capability) -- R4 33-1: Support of intra-band non-collocated NR CA operation Indicates whether the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [2], and TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. If the capability is not reported, the UE only supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. A UE supporting this feature shall also support network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeNR-CA-r18 for intra-band non-collocated NR-CA, as defined in TS 38.331 [9].