3GPP TSG-RAN4 Meeting # 113	R4-2418687

Orlando, USA, Nov.18th – 22th, 2024

# Title: 	Channel raster points for n28 40MHz

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.20.2

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements are captured in the WF[1], in which there are several open issues, such as A-MPR, channel location/channel raster, release independance.

In this contribution, we give some further discussions on the channel raster points of 40MHz for n28.

# 2	Discussion

1.	UE channel bandwidth in band 28

- Issue 1-2-7 channel raster - Agreement: - Alt1: Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz. - Alt2: Cover all the channel raster points of n28 based on the enhanced channel raster to UE RF specification TS 38.101-1 for UE CBW 40MHz.

In the TS38.104 specification, the exception channel raster points are defined for band n28, which is:

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
|  |  | 1446084 | 1556084 |
| NOTE 4: This exceptional raster point is applicable only to n28 and is only applicable for 40MHz BS channel bandwidth to ensure the guardband with 30MHz UE channel bandwidth. |  |  |  |

The exception channel raster points mainly resolve the issues of UE 30MHz reconfiguration fails when UE access 40MHz network of n28 after UE’s self-check of the minimum guard band RF requirements due to the minimum guard band of 30MHz is larger than 40MHz. In the end, some exceptions for n28 40MHz UE CBW are allowed, i.e. allow 40MHz UE CBW not aligned with 100kHz channel raster if future UE support 40MHz CBW.

It should be noted that the exception channel raster points were not introduced in the TS38.101-1 specification as of today. Instead enhanced channel rester for n28 is mandatory supported, regardless of the supported channel bandwidths.

The main purpose for introducting enhanced channel rester were to solve the PRB misalignment when embeding the even/odd RBs in odd/even RB for different channel bandwidth. Although there are no such problems for even/odd RBs are embedded in even/odd RBs for the same channel bandwidth, the enhanced channel rester can be still applied.

For legacy UE, up to 30MHz channe bandwidths are supported in the case of exception channel raster points are 40kHz offset from the closest 100kHz channel raster grid in the BS side since the exception raster point have been introduced in R16.

However, it should be noted that the exception channel raster points for band n28 were agreed before the agreements for enhanced channel raster. Also it should be noted that the enhanced channel raster is early implemention from R16 in TS38.331.

Now, both 30MHz and 40MHz are supported in UE, which means the UE specific bandwiths can be RRC reconfigred to either 30MHz or 40MHz in the case of the exception channel raster points for gNB 40MHz. The exception channel raster points in gNB would cause the misalignment raster grid between 40MHz gNB and 40MHz UE when 100kHz channel raster is used in UE. However, we think this issue can be solved thanks to the 10kHz enhanced channel raster was introduced and mandatory supported for band n28 regardless of channel bandwidth since the existing exception channel raster points are located in 10kHz grid.

Therefore, although we think either exceptional channel raster point or enhanced channel raster can slove the above problem, there is no need to include both exceptional channel raster point and enhanced channel raster in R18 TS38.101-1. We prefer to Alt 2 ‘Cover all the channel raster points of n28 based on the enhanced channel raster to UE RF specification TS 38.101-1 for UE CBW 40MHz.’

Observation 1. For n28 30MHz, there are no exception channel raster points defined in TS38.101-1.

Observation 2. For n28 40MHz, UE can supported the gNB exception channel raster points with the enhanced channel raster.

Observation 3. Enhanced channel raster is early implemented from R16 in TS38.331.

Proposal 1: Prefer to Alt 2 ‘Cover all the channel raster points of n28 based on the enhanced channel raster to UE RF specification TS 38.101-1 for UE CBW 40MHz.’

2.	Release independence

Issue 1-2-8 Release independence Agreement: - Option 1: Rel-16 - Option 2: Rel-15

Due to 40MHz was introduced for BS from Rel-16, and no matter exceptional channel raster or enhanced channel raster, it is applicable from Rel-16. Hence, we prefer Option 1 for UE n28 40MHz.

Proposal 2: Release independence from Rel-16 for UE n28 40MHz.

# 3 Conclusion

In this contribution, we give some further discussion on the RF requirements of 40MHz for n28 PC3. The conclusions are summarized in the following:

Observation 1. For n28 30MHz, there are no exception channel raster points defined in TS38.101-1.

Observation 2. For n28 40MHz, UE can supported the gNB exception channel raster points with the enhanced channel raster.

Observation 3. Enhanced channel raster is early implemented from R16 in TS38.331.

Proposal 1: Prefer to Alt 2 ‘Cover all the channel raster points of n28 based on the enhanced channel raster to UE RF specification TS 38.101-1 for UE CBW 40MHz.’

Proposal 2: Release independence from Rel-16 for UE n28 40MHz.

# 4	Reference

[1] R4-2417082, WF on PC2 and 40MHz CBW for n28, CMCC

[2] R4-2415898, Further discussion on PC3 n28 40MHz, ZTE Corporation, Sanechips.