3GPP TSG-RAN4 Meeting # 113	R4-2418689

Orlando, USA, Nov.18th – 22th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on RF requirement for ATG BS CA

# Agenda item:	7.10.3

# Document for:	Approval

# 1 Introduction

In the past meetings, there are several WFs approved for the RF requirements for ATG BS supporting intra-band contiguous CA and inter-band CA in Rel-19. Although it was agreed to reuse the same requirements (including TAE requirements) as defined for the legacy NR BS RF CA requirements, companies spent several time cycles to discuss how to handle CA TAE requirements for ATG BS CA in the TS38.104 specification due to the DL MIMO is not included in the WID but requested by operator.

In RAN4 #112bis meeting, the agreements in the chairman’s note are:

Chair: The agreed way forward is to postpone the discussion on TAE rewording until the conclusion of the December plenary meeting. No further delay is expected.

The main discussions in the previous meeting were focus on the TAE requirements, however except for TAE requirements, we would like to point out some other aspects which are not discussed in the previous meeting but need to be clarified.

# 2	Discussion

In Rel-18, only single carrier operation is supported for ATG BS, and also the the same requirements as defined for the legacy NR BS RF single carrier requirements are reused. However, there are some RF requirements not supported for ATG BS single carrier operation, which are summarized below:

1. There is no multi-band operation requirement for ATG BS.

2. The requirements for NB-IoT are not applied for ATG BS.

3. There is no TAE requirement for ATG BS.

In Rel-19, both intra-band contiguous CA and inter-band CA are supported for ATG BS. As mentioned above, the same requirements (including TAE requirements) as defined for the legacy NR BS RF CA requirements are reused.

For NB-IoT requirements, they are not applied for ATG BS single carrier operation in R18, and there are no test modes defined for NB-IoT operation for R18 ATG single carrier BS in TS38.141-1, Therefore, it is straightforward that the requirements for NB-IoT are also not applied for ATG BS supporting CA operation.

Proposal 1. To clarify that there are no NB-IoT requirements applied for ATG BS supporting CA operation.

For multi-band operation, the reasons for Rel-18 ATG BS not supporting the requirements are that only single band is considered for Rel-18 ATG BS. For Rel-19 ATG BS CA, it includes:

- - Intra-band contiguous CA

- - Inter-band CA

- For intra-band contiguous CA, multi-band operation requirements are not applied. However, for inter-band CA operation, essentially it belongs to multiple band operation. In TS38.104 specification, except TAE requirements, there are no explicit requirements difference between multi-band operation and inter-band CA operation. Therefore, it should be clarified that the multi-band operation requirement are applied for ATG BS CA.

Proposal 2. To clarify that the multi-band operation requirement are applied for ATG BS supporting CA operation.

# 3 Conclusion

In this contribution, we point out some other aspects which are not discussed in the previous meeting but need to be clarified, including NB-IoT requirements and multi-band operation requirements. The proposals and conclusions are:

Proposal 1. To clarify that there are no NB-IoT requirements applied for ATG BS supporting CA operation.

Proposal 2. To clarify that the multi-band operation requirement are applied for ATG BS supporting CA operation.

# 4	Reference

[1] R4-2406089, Way Forward for [110bis][312] NR_ATG_enh, ZTE Corporation

[2] R4-2409957, Way Forward for [111][312] NR_ATG_enh, ZTE Corporation

[3] R4-2413510, Way Forward for [112][306] NR_ATG_enh, ZTE Corporation