3GPP TSG-RAN WG4 Meeting #113                                      R4-2418718

Orlando, US, November 18th – November 22nd, 2024

Source: 	Huawei, HiSilicon

Title: 	On RF requirements of fragmented carriers

Agenda Item:	7.6.3

Document for:	Discussion

Introduction

The SI of NR FR1 DL Fragmented Carriers was approved at RAN#104. During RAN4#112bis, following agreements related to performance evaluation were captured in the way forward.

- Issue 3-1: Discussion on assumptions on test configurations for evaluation

Agreement:

(a)	RAN4 agree on the general assumptions that are same for legacy NR RF performance evaluation:

o	UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB

o	Reuse the Rx chain image rejection, which is 25dB, as the starting point.

Agreement:

- The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, REFSENS, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos

In this contribution, we provide some initial evaluation results based on the agreements.

Discussion

REFSENS

The REFSENS requirement of NCCA on FDD band is highly dependent on the test configuration, and the ΔRIBNC value due to self-interference is specified in TS 38.101-1 for selected configurations.

Taking band n3 as an example, the ΔRIBNC is specified on SCC for following two configurations.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |

When fully shared RF chain is adopted, the performance will be degraded due to the shared LPF. The attenuation on Tx leakage is less and the AGC/ADC is impacted as a result. Below Table 1 is the analysis of degraded SNR based on simplified link budget analysis. The LPF attenuation on Tx leakage frequency location is derived according to an order 50 FIR filter. The NF is assumed to increase by 10dB when the AGC is adjusted according to the power of Tx leakage. With these assumptions, the SNR is -13.4dB on PCC and -9.4 on SCC, which can’t meet the SNR target of -1dB.

Table 1. Link budget of Self-interference for 1st configuration on band n3

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise (11 bit): -68.0dBFs (6.02N+1.76) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to 0dBFs (PAPR is not considered) | Tx leakage mapping to -12dBFS (PAPR is considered) |
| PA out | 27 | - | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - | - |
| Rx Antenna input | - | -97 | -92.3 | - | - | - |
| FE loss | - | 4 | 4 | - | - | - |
| Duplexer output | -23 | -101 | -96.3 | - | - | - |
| LPF attenuation | 13.4 | 1 | 1 | - | - | - |
| LPF output (dBm) | -36.4 | -102 | -97.3 | - | - | - |
| Noise floor for basic REFSENS | - | - | - | -101 | - | - |
| NF increase due to AGC adjustment | - | - | - | 10 | - | - |
| Noise floor | - | - | - | -91 | - | - |
| In-band noise (Derived based on MSD and SNR = -1dB) | - | - | -96.3 | - | - | - |
| ADC output (dBm) | - | - | - | - | -104.4 | -92.4 |
| SNR after ADC (dB) | - | - | - | - | -11.2(PCC) | -13.4(PCC) |
|  |  |  |  |  | -7.6(SCC) | -9.4(SCC) |

Observation 1: 9~13 dB extra relaxation might be needed with the fully shared RF chain when there is self-interference between Tx/Rx.

ACS/IBB

Based on the discussion in previous meetings, the performance of ACS/IBB needs to be analysed in following configurations:

- Configuration 1: Rx image of ACS/IBB blocker falls into the wanted carrier

- Configuration 2: ACS/IBB blocker is on the LO frequency

- Configuration 3: ACS/IBB blocker is on other frequency locations

Using the similar link budget method, the SNR of ACS/IBB cases are summarized as below Table 2. The results of Configuration 2 need more analysis therefore are not provided.

Table 2	SNR for ACS/IBB with fully shared RF chain

|  | ACS case 1 | ACS case 2 | IBB case 1 | IBB case 2 |
| --- | --- | --- | --- | --- |
| Configuration 1 | -7.7 | -7.6 | -10.9 | -22.1 |
| Configuration 3 | -1.5 | -1 | -3.6 | -6.6 |

Observation 2: For ACS/IBB, 7~22 dB extra relaxation might be needed when the Rx image of blocker falls into the wanted carrier.

Observation 3: For ACS/IBB, 3~6 dB extra relaxation might be needed when the Rx image of blocker falls into the wanted carrier.

Conclusion

In this contribution we proposed to study the fully shared architecture, and provided our view on the potentially impacted RF requirements. According to the analysis, we have the following proposals:

Observation 1: 9~13 dB extra relaxation might be needed with the fully shared RF chain when there is self-interference between Tx/Rx.

Observation 2: For ACS/IBB, 7~22 dB extra relaxation might be needed when the Rx image of blocker falls into the wanted carrier.

Observation 3: For ACS/IBB, 3~6 dB extra relaxation might be needed when the Rx image of blocker falls into the wanted carrier.

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024

- RP-2411691, “On general aspects of fragmented carriers”, Huawei, RAN4#112 Meeting, August 2024

- RP-2415805, “On general aspects of fragmented carriers”, Huawei, RAN4#112bis Meeting, October 2024

- R4-2417201, “WF on NR FR1 DL fragmented carriers study”, MediaTek Inc, RAN#112bis Meeting, October 2024