3GPP TSG-RAN WG4 Meeting #113				                        R4-2418726

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.21.3.1

Source: 	CMCC

Title: 	Discussion on A-IoT BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #102 meeting, a new SID[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- Coexistence study of Ambient IoT and NR/LTE.

- RF requirements study for Ambient IoT:

  - Ambient IoT BS transmission and reception

  - Ambient IoT Device, as per the General Scope, transmission and reception

  - Intermediate node (UE), as per the General Scope, transmission and reception

Following we list our analysis for A-IoT reader and CW nodes for topology 1.

- Discussion

2.1 System parameters

- operation band

For band specific requirements, to reduce workload, it’s suggested to only choose one example band for further study. Band n8 the 900MHz is suggested as example band.

Proposal 1: it’s suggested to only choose band n8 as example band.

- Channel spacing/ channel raster

For guard-band mode, the channel raster and channel spacing is not applicable. For the in-band mode, the channel spacing is not applicable.

Proposal 2: For guard-band mode, the channel raster and channel spacing are not applicable. For in-band mode, the channel spacing is not applicable.

- Standalone: channel raster, channel spacing

- In-band: channel raster

- Guard-band: N/A

2.2 A-IoT BS Tx requirement

Last meeting agreements are listed as below:

Agreement:

- Whether requirements for A-IoT device RX RF potentially needed or not is summarized in the following table:

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | YES, as RAN4#112 agreed |
| Output power dynamic | RE power control dynamic range | TBD |
|  | Total power dynamic range | TBD |
|  | Power boosting | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | TBD |
|  | Transmitter transient period | TBD |
| Transmit signal quality | Frequency error | YES, as RAN4#112 agreed |
|  | Some requirements like EVM (Modulation quality) | YES, as RAN4#112 agreed |
|  | TAE | NA, as RAN4#112 agreed |
| Unwanted emissions | Occupied bandwidth | YES, as RAN4#112 agreed |
|  | ACLR | YES, as RAN4#112 agreed |
|  | IBE | NO |
|  | Operating band unwanted emissions | YES, as RAN4#112 agreed |
|  | Transmitter spurious emissions | YES, as RAN4#112 agreed |
| Transmitter intermodulation |  | TBD |

- Power related requirements

For power dynamic range related requirement,

- RE power control dynamic range: this is related to modulation schemed, according to RAN1 agreements, OOK-1 and OOK-4 are the candidate modulation schemed, so RE power control dynamic range should also be updated accordingly.

- Total power dynamic range: this is related to SCS and max transmission bandwidth configuration. Now there is no agreement of the max channel bandwidth of reader. If we only define 1PRB max transmission bandwidth with 15KHz SCS assumption, then such requirement may not be needed any more. But if higher data rate is supported, 180KHz may not be enough. So we may still need this requirement but updated based on channel bandwidth and SCS definition.

Proposal 3: RE power control dynamic range may needs to be updated since now only OOK modulation is supported at reader side.

Proposal 4: total power dynamic range needs to be updated accordingly based on conclusion of channel bandwidth and SCS.

- Tx signal quality related requirement

BS spec define frequency error and signal quality related requirements.

For frequency error, UE use absolute frequency ARFCN to identify frequency so frequency error requirement is signicifent and RAN4 define strict level, i.e. 0.05/0.1ppm. For RFID, to enhance spectrum efficiency and reduce interference to other systems, there is also frequency accuracy requirement of reader for multiple and dense interrogator deployment scenario, which is almost in the range from 10ppm to 20ppm. For A-IoT, considering devices will use energy/edge detection to detect reader signal, it seems their is no need for very strict frequency error requirement if assume all A-IoT reader using same spectrum.

Proposal 5: current gNB frequency error is enough for reader if assuming no FDM operations between readers.

But if A-IoT reader support FDM operation and the gap between different carriers is much less, it seems current BS frequency error requirements may need to be more strict. For example, if we assume 180kHz reader carrier bandwidth and miller-modulated subcarrier that backscatterred by device will fully occupy 180KHz carrier. It seems current frequency error may cause interference between readers that using adjacent carriers and are located in close proximity.

Observation 1: if reader support FDM operation and carrier bandwidth is much less, more strict frequency error requirement may be needed.

During evaluation, we don’t consider the adjacent-carrier interference between A-IoT systems because usually there is only one operator using one carrier in one individual factory deployment scenario. To reduce workload, it’s suggested that we only focus on single carrier operation mode for reader at first and FDM reader operation as low priority.

Proposal 6: in RAN4, it’s suggested to focus on single carrier operation mode for reader at first.

For signal quality related requirements, gNB spec define EVM requirement which is used to evaluate phase distance. For reader Tx, RAN1 support OOK-1 and OOK-4 modulation. So current gNB quality related requirement is not applicable anymore. RFID[1] use three types ASK modulation types, i.e. DSB-ASK, SSB-ASK or PR-ASK and define RF envelop requirement including modulation depth, RF envelop ripple, RF pluswidth etc which can be referred for OOK modulation. Details illustration of RFID RF envelop requirement is listed as below for information.

Observation 2: RFID spec use RF envelop related parameters to evaluate ASK signal transmission quality requirement, e.g. modulation depth, RF envelop ripple, RF plusewidth etc.

Proposal 7: RAN4 could refer to RFID RF envelop related parameters to define signal transmission quality requirement as starting point, i.e. modulation depth, RF envelop ripple, RF plusewidth.

- Tx time related requirement

RFID also define Tx time related requirement, e.g. rise time, settling time, fall time.

- Rise time: the time periodicity of power-up envelop from 10% point rising monotonically to 90% point.

- Fall time: the time periodicity of power-down envelop from 90% point falling monotonically to 10% point.

- Settling time: the time periodicity of power-up envelop rise from 90% point until at least the ripple limit MI. the 90% point is rising monotonically from 10% point. During the whole settling time, envelop should not fall below the 90% max filed strength.

BS spec defines transient period which is much like rise time and fall time requirements as in RFID spec. Besides, RFID also defines settling time to evaluate signal ripple characteristics. reader support OOK signal which is much like the ASK modulated signal as in RFID spec. It’s suggested to further check whether the settling time as in RFID spec is needed or not.

Proposal 8: it’s suggested to define transient period related requirements for A-IoT reader. Details value can refer to RFID rise/fall time.

Proposal 9: RAN4 further discuss whether settling time as defined in RFID spec is needed or not to evaluate RF envelop ripple characteristics.

2.4 A-IoT CW nodes of outside topology

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| RF Requirement for CW for D1T1 | Requirements potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | TBD |
| Frequency error | YES |
| Phase noise | NO |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | TBD |

For the A-IoT CW nodes, only Tx requirement are needed. Different from legacy UE waveform, CW is single tone or multiple tone cases. So for CW node, we mainly need to consider the unwanted emission related requirements. system parameters and signal quality related requirements are not needed.

If RAN4 define RF requirements for CW outside topology, it seems we can only consider max output power and spurious emission requirements.

Proposal 10: for CW outside of topology, it seems only max output power and spurious emission requirement is applicable. For out of band emission, if we assume almost perfect performance since single tone is assumed, there is no need for such requirements.

# 3. Conclusions

In this contribution, RF requirements for gNB like reader are discussed with following observations and proposals.

Proposal 1: it’s suggested to only choose band n8 as example band.

Proposal 2: For guard-band mode, the channel raster and channel spacing are not applicable. For in-band mode, the channel spacing is not applicable.

- Standalone: channel raster, channel spacing

- In-band: channel raster

- Guard-band: N/A

Proposal 3: RE power control dynamic range may needs to be updated since now only OOK modulation is supported at reader side.

Proposal 4: total power dynamic range needs to be updated accordingly based on conclusion of channel bandwidth and SCS.  Proposal 6: in RAN4, it’s suggested to focus on single carrier operation mode for reader at first.

Proposal 5: current gNB frequency error is enough for reader if assuming no FDM operations between readers.

Proposal 6: in RAN4, it’s suggested to focus on single carrier operation mode for reader at first.

Proposal 7: RAN4 could refer to RFID RF envelop related parameters to define signal transmission quality requirement as starting point, i.e. modulation depth, RF envelop ripple, RF plusewidth.

Proposal 8: it’s suggested to define transient period related requirements for A-IoT reader. Details value can refer to RFID rise/fall time.

Proposal 9: RAN4 further discuss whether settling time as defined in RFID spec is needed or not to evaluate RF envelop ripple characteristics.

Proposal 10: for CW outside of topology, it seems only max output power and spurious emission requirement is applicable. For out of band emission, if we assume almost perfect performance since single tone is assumed, there is no need for such requirements.

# 4. Reference

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz - 960 MHz, Version 2.0.0 Ratified