3GPP TSG-RAN WG4 Meeting #113				                      R4-2418728

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.21.3.3

Source: 	CMCC

Title: 	Discussion on A-IoT intermediate UE RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #102 meeting, a new SID[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- Coexistence study of Ambient IoT and NR/LTE.

- RF requirements study for Ambient IoT:

  - Ambient IoT BS transmission and reception

  - Ambient IoT Device, as per the General Scope, transmission and reception

  - Intermediate node (UE), as per the General Scope, transmission and reception

Following we list our analysis for A-IoT intermediate UE RF requirements.

- Discussion

Different from reader of topology 1 which only act like reader, intermediate UE also need to connects to gNB to receive controlling signal. From this point of view, it has to follow current UE specs.

Proposal 1: intermediate UE should following current UE RF requirements when it connect to controlling gNB.

Another issue is whether we need to separately test RF requirements when intermediate UE acts like UE and acts like reader even if certain RF requirements are the same. Considering signal and functionality are totally different, it’s better to separately testing RF requirements.

Proposal 2: it’s suggested to separately testing intermediate UE when it acts like UE and when it acts like reader even if certain RF requirements are the same.

2.1 A-IoT intermediate UE Tx requirement

Issue 4-2: TX

Agreement:

- Whether all the requirements defined need to be tested for intermediate UE can be further discussed in WI phase.

- Whether requirements potentially needed or not is summarized in the following table:

| RF Requirement for A-IoT intermediate UE- TX part |  |  | Potentially needed or not |
| --- | --- | --- | --- |
| Maximum output power |  |  | YES, as RAN4#112 agreed |
| Output power dynamics | Minimum output power |  | NO |
|  | Transmit OFF power |  | TBD |
|  | Transmit ON/OFF time mask |  | TBD |
|  | Power control |  | NO |
| Transmitted signal quality | Frequency error |  | YES, as RAN4#112 agreed |
|  | Transmit modulation quality | EVM | YES |
|  |  | Carrier leakage | TBD |
|  |  | In-band emissions | TBD |
| Output RF spectrum emissions | Occupied bandwidth |  | YES, as RAN4#112 agreed |
|  | ACLR |  | Depends on the co-existence, as RAN4#112 agreed |
|  | Spurious emissions |  | YES, as RAN4#112 agreed |
|  | Transmit intermodulation |  | TBD |

- Reader power class

According to link budget analysis from RAN1, except for D1T1-A2, for other deployment case, DL link is the coverage bottleneck compared with UL coverage. for D1T1-A2, UL coverage is the bottleneck due to self-interference. From this point of view, it seems higher power class, e.g. PC1.5 is not that urgent in this release.

For PC1.5, in current spec, there is no FDD band supporting PC1.5. For intermediate UE, one issue is whether only limited to current supported max power class per operation band and other higher power class are not supported in this A-IoT WID. To reduce workload, it’s better to focus on current specified power class and when other higher power class are supported in spectrum related WID in future, we can further check the feasibility for A-IoT intermediate UE. Besides, it’s suggested to use PC3 as baseline and further discuss PC2 in work phase.

Proposal 3: max supported power class per band of intermediate UE is only limited to PC3 as baseline.

For OFF power and ON/OFF time mask, as discussed in BS reader tdoc, UE spec defines transient period which is much like rise time and fall time requirements as in RFID spec. Besides, RFID also defines settling time to evaluate signal ripple characteristics. reader support OOK signal which is much like the ASK modulated signal as in RFID spec. It’s suggested to further check whether the settling time as in RFID spec is needed or not.

Proposal 4: it’s suggested to define transient period related requirements for A-IoT reader. Details value can refer to RFID rise/fall time.

Proposal 5: RAN4 further discuss whether settling time as defined in RFID spec is needed or not to evaluate RF envelop ripple characteristics.

- Transmit signal related requirements

Transmit signal related requirements are much similar for both UE reader and BS reader. The same proposal as discussed in [1] are listed as below. Besides RAN4 needs to discuss whether there is any difference between UE reader and BS reader. Since signal quality related requirements are only related to modulation scheme, we can define one unified requirements for both UE reader and BS reader.

Observation 1: RFID spec use RF envelop related parameters to evaluate ASK signal transmission quality requirement, e.g. modulation depth, RF envelop ripple, RF plusewidth etc.

Proposal 6: RAN4 could refer to RFID RF envelop related parameters to define signal transmission quality requirement as starting point, i.e. modulation depth, RF envelop ripple, RF plusewidth. Besides, the RF requirements for BS reader and UE reader can be the same.

- IBE requirements for in-band spectrum deployment mode of topology 2 reader

For co-existence evaluation, current baseline assumption is that R2D for topology 2 will use UL spectrum, the IBE requirements of intermediate UE and the ICS requirements of existing gNB/ will impact final interference level between intermediate UE and legacy existing gNB. The interference is much similar like legacy NR intra-system interference. The main difference is the frequency error since for A-IoT the frequency error is between network control gNB and legacy gNB rather than between two intra-system gNB. But the frequency error is small and final contribution for interference is marginal. So it seems reasonable to assume current IBE requirements of legacy UE still applies for intermediate UE.

Observation 2: It seems current legacy IBE requirements of UE still applies for intermediate UE.

Proposal 7: it’s suggested to assume that legacy IBE requirements of UE still applies for intermediate UE as starting point.

- ACLR requirement

Current co-existence evaluation only focus on in-band spectrum deployment mode because this is the worst interference case compared with adjacent carrier case. Once final co-existence evaluation show that A-IoT system could co-exist with NR system for in-band spectrum mode, then we can conclude legacy UE ACLR requirement applies for A-IoT UE reader. But if certain interference occurs for in-band spectrum deployment mode, further evaluation is needed for corresponding interference case. But we can leave such analysis to work phase to define corresponding requirements.

Observation 3: current co-existence evaluation only focus on in-band spectrum deployment mode but there is no interference analysis for adjacent carrier spectrum deployment mode.

Proposal 8: for ACLR requirement

  - Once final co-existence evaluation show that A-IoT system could co-exist with NR system for in-band spectrum mode, then we can conclude legacy UE ACLR requirement applies for A-IoT UE reader.

2.3 A-IoT intermediate UE Rx requirement

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT intermediate UE- RX RF Requirement | Potentially needed or not |
| --- | --- |
| Reference sensitivity | YES, as RAN4#112 agreed |
| Maximum input power | YES, as RAN4#112 agreed |
| ACS | YES, Depends co-existence study |
| In-band blocking | YES, Depends co-existence study |
| Out-of-band blocking | YES |
| Narrow band blocking | TBD |
| Spurious response | TBD |
| Receiver intermodulation | YES, as RAN4#112 agreed |
| RX spurious emissions | YES |

- ICS requirements for in-band spectrum deployment mode of topology 2 reader

For ICS requirement, currently intermediate UE is allowed to receive at both UL and DL spectrum.

When intermediate UE receive at DL spectrum, we mainly consider the interference from legacy existing gNB to intermediate UE, i.e. the IBE at existing gNB and ICS at intermediate UE will impact final interference. This is much like the intra-system interference scenario. So it seems legacy UE requirement still applies.

When intermediate UE receive at UL spectrum, we mainly consider the interference from legacy existing UE to intermediate UE, i.e. the IBE at existing UE and ICS at intermediate UE will impact final interference. UE-UE interference occurs only with low probability e.g. when such UEs are very near to each other. During analysis of UE feasibility at SBFD study phase, it is approved that ICS performance is much similar as ACS performance, so we can wait for co-existence evaluation to analyze whether ICS could reuse ACS value for intermediate UE.

If final evaluation results show that better IBE/ICS requirement is required, one choice is to reserve guard RB to enhance performance. Then RAN4 needs to discuss whether to explicitly define guard RB in spec or implicitly defined by vendor implementation. If we don’t define such guard RB requirement but leave it for implementation, then network control gNB has no idea of intermediate UE’s different guard RB. Once the guard RB is scheduled by UE, interference occurs. On the other hand, such constant guard RB would reduce spectrum utilization. It’s better to take care of this guard RB requirements and study how to balance interference and spectrum utilization.

Observation 4: ICS requirement needs to wait for co-existence evaluation. Guard RB will help to improve IBE/ICS performance but will reduce spectrum utilization.

- Self-interference issue

Whether the CW uses DL or UL spectrum, self-interference occurs from CW to reader receiver. For the CW inside topology, we can use REFSENSE to reflect residual self-interference. Usually we assume 1dB REFSENSE desense for SBFD, but for A-IoT, the interference is the CW signal with single tone or multi-tone. the interference bandwidth is very narrow and this will lead following issue.

- how to evaluate the REFSENSE desense especially when the interference bandwidth is such narrow, i.e. whether 1dB desense is still applicable or not

Observation 5: due to very narrow bandwidth of CW interference signal, RAN4 needs to further discuss whether 1dB desense self-interference cancellation target is still applicable or not.

When CW is outside the topology, additional spatial isolation will contribute to even less self-interference and the final residual self-interference may be negligible. But as analyzed, final residual self-interference is related to spatial isolation between CW and reader, RAN4 may needs some typical assumption before conclude whether to/how to reflect self-interference for outside topology scenario.

Observation 6: for CW outside topology, RAN4 needs some typical spatial isolation assumption before conclude whether to/ how to reflect self-interference by RF requirements.

- ACS

Same as ACLR requirements, we have following observation and proposal for A-IoT.

Observation: current co-existence evaluation only focus on in-band spectrum deployment mode but there is no interference analysis for adjacent carrier spectrum deployment mode.

Proposal 9: for ACS requirement

  - Once final co-existence evaluation show that A-IoT system could co-exist with NR system for in-band spectrum mode, then we can conclude legacy UE ACS requirement applies for A-IoT UE reader.

- IMD product for multiple-tone CW when define Rx requirements for reader

The same logic as discussed in BS reader RF requirement also applies for UE reader, UE reader should also consider the IMD product produced at device side for multiple single tone case. This seems like equivalent MSD requirement. We may need to define some additional REFSENSE desense requirement to reflect such IMD product. Anyway, it’s better to wait for RAN1 conclusion at first.

Observation 7: IMD product of multi-tone CW is much hard to be suppressed and filter solution may not work considering there is frequency overlapping between IMD products and wanted data.

Proposal 10: RAN4 should wait for RAN1 CW signal design conclusion and then decide whether/how to consider the IMD product of multiple-tone CW when defining Rx requirements for reader.

2.4 A-IoT CW nodes of outside topology

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| RF Requirement for CW node | Potentially needed or not |
| --- | --- |
| Output power | YES, as RAN4#112 agreed |
| Output power dynamic range | TBD |
| Transmit signal quality | TBD |
| RF spectrum emission | YES, as RAN4#112 agreed |
| Spurious emissions | TBD |
| Phase noise | NO |

For the A-IoT CW nodes, only Tx requirement are needed. Different from legacy UE waveform, CW is single tone or multiple tone cases. So for CW node, we mainly need to consider the unwanted emission related requirements. system parameters and signal quality related requirements are not needed.

If RAN4 define RF requirements for CW outside topology, it seems we can only consider max output power and spurious emission requirements.

Proposal 11: for CW outside of topology, it seems only max output power and spurious emission requirement is applicable. For out of band emission, if we assume almost perfect performance since single tone is assumed, there is no need for such requirements.

# 3. Conclusions

In this contribution, RF requirements of intermediate UE are discussed with following observations and proposals.

Proposal 1: intermediate UE should following current UE RF requirements when it connect to controlling gNB.

Proposal 2: it’s suggested to separately testing intermediate UE when it acts like UE and when it acts like reader even if certain RF requirements are the same.

Proposal 3: max supported power class per band of intermediate UE is only limited to PC3 as baseline.

Proposal 4: it’s suggested to define transient period related requirements for A-IoT reader. Details value can refer to RFID rise/fall time.

Proposal 5: RAN4 further discuss whether settling time as defined in RFID spec is needed or not to evaluate RF envelop ripple characteristics.

Observation 1: RFID spec use RF envelop related parameters to evaluate ASK signal transmission quality requirement, e.g. modulation depth, RF envelop ripple, RF plusewidth etc.

Proposal 6: RAN4 could refer to RFID RF envelop related parameters to define signal transmission quality requirement as starting point, i.e. modulation depth, RF envelop ripple, RF plusewidth. Besides, the RF requirements for BS reader and UE reader can be the same.

Observation 2: It seems current legacy IBE requirements of UE still applies for intermediate UE.

Proposal 7: it’s suggested to assume that legacy IBE requirements of UE still applies for intermediate UE as starting point.

Observation 3: current co-existence evaluation only focus on in-band spectrum deployment mode but there is no interference analysis for adjacent carrier spectrum deployment mode.

Proposal 8: for ACLR requirement

  - Once final co-existence evaluation show that A-IoT system could co-exist with NR system for in-band spectrum mode, then we can conclude legacy UE ACLR requirement applies for A-IoT UE reader.

  - But if certain interference occurs for in-band spectrum deployment mode, further evaluation is needed for corresponding interference case. But we can leave such analysis to work phase to define corresponding requirements.

Observation 4: ICS requirement needs to wait for co-existence evaluation. Guard RB will help to improve IBE/ICS performance but will reduce spectrum utilization.

Observation 5: due to very narrow bandwidth of CW interference signal, RAN4 needs to further discuss whether 1dB desense self-interference cancellation target is still applicable or not.

Observation 6: for CW outside topology, RAN4 needs some typical spatial isolation assumption before conclude whether to/ how to reflect self-interference by RF requirements.

Proposal 9: for ACS requirement

  - Once final co-existence evaluation show that A-IoT system could co-exist with NR system for in-band spectrum mode, then we can conclude legacy UE ACS requirement applies for A-IoT UE reader.

  - But if certain interference occurs for in-band spectrum deployment mode, further evaluation is needed for corresponding interference case. But we can leave such analysis to work phase to define corresponding requirements.

Observation 7: IMD product of multi-tone CW is much hard to be suppressed and filter solution may not work considering there is frequency overlapping between IMD products and wanted data.

Proposal 10: RAN4 should wait for RAN1 CW signal design conclusion and then decide whether/how to consider the IMD product of multiple-tone CW when defining Rx requirements for reader.

Proposal 11: for CW outside of topology, it seems only max output power and spurious emission requirement is applicable. For out of band emission, if we assume almost perfect performance since single tone is assumed, there is no need for such requirements.

# 4. Reference

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei