3GPP TSG-RAN WG4 Meeting # 113                                                                R4-2418734

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.23.2.2

Source: 	CMCC

Title: 	Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In this contribution, we focus on discuss of UE LP-WUS RF requirements including REFSENSE, ACS and ASCS.

- Discussion

2.1 REFSENSE requirements

During the last meeting, RAN4 reached the following agreement below.

Issue 2-1-2: Target FR1 SNR value for LP-WUS/WUR Agreement: - RAN4 can further discuss target SNR for OOK-based and OFDM-based receivers. Issue 2-2-3: REFSENS, NF, IM, SNR aspects for FR1 OOK-based and OFDM-based LR Agreement: - Collect input on REFSENS with values for each element（NF, IM, SNR） in the table next meeting. The value for each element can be discussed. - Encourage companies to also input the justification for each element.

Considering SNR is relevant to RF and demodulation and RAN1 has two options of OFDM signal and OOK signal, the required SNR for demodulation on OOK and OFDM sequence could be different, so two sets of SNR values could be specified. Based on the simulation results, OFDM receiver perform better than OOK receiver. Since the case OOK-4, M=4 has the worst performance, target SNR should be chosen based on this case.

Table 1 SNR@1%BLER with 4bit ADC under AWGN

| Modulation | Channel structure | Receiver type | SNR@1% BLER |
| --- | --- | --- | --- |
| OOK-1 | 8bit | Envelope | -6.5dB |
|  |  | OFDM | -10.6dB |
|  | 16bit | Envelope | -6.1dB |
|  |  | OFDM | -10.3dB |
| OOK-4 M=2 | 8bit | Envelope | -4.7dB |
|  |  | OFDM | -7.6dB |
|  | 16bit | Envelope | -4.2dB |
|  |  | OFDM | -7.2dB |
| OOK-4 M=4 | 8bit | Envelope | -2.5dB |
|  |  | OFDM | -4.7dB |
|  | 16bit | Envelope | -2.2dB |
|  |  | OFDM | -4.3dB |

Proposal 1: 2.5dB SNR for OOK-based signal, and 4.7dB for OFDM-based signal.

Considering the difference between OFDM and OOK signals, two sets of noise figure requirements should be specified. For the OFDM signal, the NF value of the OFDM receiver and legacy UE receiver are similar, so we could assume the legacy UE receiver noise figure value 9dB as the baseline for the OFDM signal.

Proposal 2: Set 9dB as the OFDM noise figure for the OFDM signal as the baseline.

For the OOK signal receiver, the NF value is much larger than the NF of legacy UE. During the study item phase, we listed some example NF value ranges:

- RF envelope detection: The noise figure is in the range of 12~22 dB.

Consider zero-IF receiver as a baseline architecture for envelop based LP_WUR. We could choose the same NF range for the LP-WUS signal, which is 12-15dB. Since RAN1 chooses 7dB as the NF of MR, and the gap between MR and LR is 2/5/8dB, it is reasonable to choose 12dB as the noise figure for the OOK signal.

Proposal 3: Choose 12dB noise figure as the baseline for the OOK signal.

And the Table is shown as below:

Table 2. REFSENS with values for each element

| Type | NF | SNR | IM |
| --- | --- | --- | --- |
| OOK-based | 12dB | 2.5dB | 2.5dB |
| OFDM-based | 9dB | 4.7dB | 2.5dB |

2.1 ASCS and ACS

When defining ACS and ASCS for LP-WUR, two factors should be considered: the ASCS value on a dB scale and the required guard RB. If we only define the guard RB requirement without specifying an ASCS value, there would be no restriction on the in-band selectivity of LP-WUR. In extreme cases, even LP-WUR with the worst performance might pass the test, as there is only a guard RB limitation. Conversely, if we only define the ASCS value without any guard RB limitation, LP-WUR with the worst performance could also pass the test in extreme cases, as long as the guard RB is large enough.

Proposal 4: For ASCS and ACS requirement, RAN4 should specify the both guard RB and ASCS/ACS requirements.

Since ASCS requirement cannot be specified directly, we should define the test case for the ASCS requirement in the specification. The test parameters should include the power level, bandwidth, MDR, and other relevant parameters. The MDR will be verified in the test with the guard RBs as defined in the specification. And we could use the following table as the starting point to define the ASCS test parameters.

Table 3: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | [1] at each side |
| MDR |  | 1% |

Proposal 5: Use the table 3 as the starting point of the ASCS test parameters.

For ACS requirement, we can use the legacy requirement as the starting point, and further consider to relax the value with guard RB.

Proposal 6: Use the legacy ACS requirement as the starting point.

# 3. Conclusions

In this contribution, ASCS, ACS and REFSENSE are discussed with following observations and proposals.

Proposal 1: 2.5dB SNR for OOK-based signal, and 4.7dB for OFDM-based signal.

Proposal 2: Set 9dB as the OFDM noise figure for the OFDM signal as the baseline.

Proposal 3: Choose 12dB noise figure as the baseline for the OOK signal.

Table 2. REFSENS with values for each element

| Type | NF | SNR | IM |
| --- | --- | --- | --- |
| OOK-based | 12dB | 2.5dB | 2.5dB |
| OFDM-based | 9dB | 4.7dB | 2.5dB |

Proposal 4: For ASCS and ACS requirement, RAN4 should specify the both guard RB and ASCS/ACS requirements.

Proposal 5: Use the table 3 as the starting point of the ASCS test parameters.

Table 3: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | [1] at each side |
| MDR |  | 1% |

Proposal 6: Use the legacy ACS requirement as the starting point.

# 4. Reference

- R4-2415235, Topic summary for [112bis][131] NR_LPWUS_UERF