3GPP TSG-RAN WG4 Meeting # 113                                    R4-2418737

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.23.2.1

Source: 	CMCC

Title: 	Discussion on LP-WUS UE system parameters requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #103 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In RAN4#110-bis, the LP-WUS RF requirements had been discussed and the WF was approved in [1]. This paper will focus on some issue about system parameters.

- Discussion

### 2.2 Channel raster for LP-WUR

Issue 1-2-1: Channel raster for LP-WUR Agreements: - Companies directly bring the specification impact if any and those will be discussed instead of whether a channel raster is required or not. Issue 1-2-2: considerations on LP-WUS allocation Agreements: - RAN4 to assume PRB grid alignment between LP-WUS and NR. - RAN4 to assume the LP-WUS RBs can be flexibly allocated within the wider NR carrier.

After last meeting discussion, companies still have different understanding on the issue for the channel raster. In our opinion, for in-band case, since LP-WUS RBs will be transmitted together with NR RBs, the existing requirements of NR channel raster could be reused for LP-WUS.

Proposal 1: Channel raster is needed for LP-WUS.

Issue 1-2-2 has reached the agreement that assume PRB grid alignment between LP-WUS and NR, and LP-WUS RBs can be flexibly allocated within the wider NR carrier. If 100kHz channel raster is used for some low bands, it means that the LP-WUS RB with 30kHz/15kHz SCS couldn’t be allocated within the NR carrier.

Observation 1: LP-WUS RB with 30kHz/15kHz SCS couldn’t be flexibly allocated within the NR carrier with 100kHz channel raster, and this will conflict with the conclusions we have already reached.

Some companies suggest to introduce new channel raster like 10kHz for LP-WUS. In our opinion, there is no need to introduce new channel, and the primary reason for the above issue is that the 100kHz channel raster will restrict the placement of RB. So we suggest the legacy channel raster could be reused for LP-WUS without 100kHz channel raster.

Proposal 2: The legacy channel raster could be reused for LP-WUS without 100kHz channel raster.

# 3. Conclusions

Proposal 1: Channel raster is needed for LP-WUS.

Observation 1: LP-WUS RB with 30kHz/15kHz SCS couldn’t be flexibly allocated within the NR carrier with 100kHz channel raster, and this will conflict with the conclusions we have already reached.

Proposal 2: The legacy channel raster could be reused for LP-WUS without 100kHz channel raster.

# 4. Reference

- R4-2417112, WF on UE RF requirements for LP-WUS