3GPP TSG RAN WG4 Meeting #113	R4-2418753

Orlando, Florida, USA, 18th – 22nd November, 2024

Agenda item:			7.17.5.1

Source:	vivo

Title:	Discussion on	RRM core requirements for positioning accuracy enhancement

Document for:	Discussion

- Introduction

RAN4 has studied many aspects about AI/ML use case aspects in Rel-18 SI stage, including CSI feedback enhancement, beam management enhancement and positioning accuracy enhancement. The outcomes of the study for positioning are captured in TR 38.843 [1]. In the RAN4#112bis meeting, the WF [2] for AI/ML based positioning is captured as

Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.

In this contribution, we further provide our views on RRM core requirements for positioning accuracy enhancement.

- Discussion

For AI/ML positioning cases, 5 sub-use cases are identified and summarized in Table 1.

- A detailed list of various positioning cases

| Cases | Priority | Model deployment | Positioning methods | Measured by which entity | Model output | Position calculation |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1st priority | UE-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | UE-side |
| Case 2a | 2nd priority | UE-side | AI/ML assisted positioning | UE (Downlink positioning) | Intermediate feature | LMF |
| Case 2b | 2nd priority | LMF-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | LMF |
| Case 3a | 1st priority | gNB-side | AI/ML assisted positioning | gNB (uplink positioning) | Intermediate feature | LMF |
| Case 3b | 1st priority | LMF-side | Direct AI/ML positioning | gNB (uplink positioning) | Position | LMF |

- 2.1	Potential RAN4 requirement for case 1

In RAN2#126 meeting, agreements about UE side model are reached as

Agreements - The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. - wait for RAN1 for associate ID discussion - At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements

For case 1, RAN2 agreed to use the existing LPP procedures related to Location Information Transfer to provide the UE sided model inference results, which is the UE position, to the NW side. It means that there is a potential interactive process between NW and UE. RAN4 could consider define a delay requirement from the start of UE measuring PRS, through the AI processing, to the end of UE report location to LMF. Since further signaling enhancements may needed in RAN2, more progresses are also needed for defining delay requirement for case 1.

Proposal 1: RAN4 to define delay requirement for case 1 at least considering the following procedures: PRS measuring, AI processing, UE reporting location to LMF.

Besides, it also means that accuracy requirements for UE position needs to be defined for case 1, e.g., the straight-line distance (unit: m) between the real position and the estimated position could be considered as the metric for accuracy requirement. In TS 38.355, the location coordinate is described as an 3D location “ellipsoid point” in x, y, z axis in CommonIEsProvideLocationInformation. Here we give an example for accuracy of UE position. The reported contents may be a coordinate (x1, y1, z1) for P1, x1, y1 and z1 are the related location compared with the local coordinate system, and the real coordinate of the target UE is (x0, y0, z0) for P0, then the accuracy metric can be the Euclidean distance of these two coordinates:

In these methods, the accuracy requirement can be simply defined and the unit is meter. If the reported content is other types, it can be simplified as . Dmin represents the straight-line distance of two position points.

Proposal 2: RAN4 to consider the straight-line distance (unit: m) between the real position and the estimated position as the metric for accuracy requirement for case 1.

- 2.2	Potential RAN4 requirement for case 2

For case 2, RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024 according to the RANP, which is case 2. For case 2b, the channel measurements (power information, timing information, phase information) are used as the model input and need to be reported to LMF and RAN1 is discussing whether the measurements are sample-based or path-based since the last meeting. Now the discussing has to be postponed and RAN4 does not need to discuss this case anymore in the meeting.

Proposal 3: RAN4 to postpone the discuss of requirement for positioning case 2b.

However, UE needs to report intermediate features to LMF in case 2a and the procedure is similar as case 1, including PRS measuring, AI processing, and UE reporting. RAN4 could define delay requirements for case 2a by considering the mentioned aspects.

Proposal 4: RAN4 to define delay requirement for case 2a at least considering the following procedures: PRS measuring, AI processing, UE reporting intermediate features to LMF.

RAN1 has agreed at least the following intermediate features are supported for reporting.

Agreement For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

For the timing-related intermediate features: RSTD, UE Rx-Tx time difference, RAN4 could reuse the already defined accuracy requirement.

Proposal 5: RAN4 to reuse the existing accuracy requirements for the timing-related intermediate features: RSTD, UE Rx-Tx time difference for positioning case 2a.

For LoS/NLoS indicator, RAN1 has discussed in Rel-17 and reached the agreement that no RAN4 requirements are expected for the LoS/NLoS indicator in RAN1’s understanding. However, in Rel-19 AI/ML, the LoS/NLoS indicator is a very important intermediate feature, which is already agreed by RAN1 that the indicator is support to be reported to NW. In legacy NR positioning, the applied scenarios are always assumed as LoS environment, or NLoS environment while the signal attenuation in the LoS direction is not very large, so that the position of the target UE can be calculated e.g., by some additional paths report in LMF. In AI/ML based positioning, the assumed scenarios are indoor factory and the environment is complicated, includes LoS, NLoS or even heavy NLoS, which makes the LoS/NLoS indicator more important, and the accuracy requirement may need to be defined by RAN4.

The LoS/NLoS indicator can be categorized to two types, as described in TS 38.214

LoS/NLoS indicator captured in TS 38.214 The UE may be requested, subject to UE capability, to report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-IndicatorRequest. The UE can report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator associated with each DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, and UE Rx-Tx time difference measurements. The UE can report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator associated with each dl-PRS-ID in a measurement report. For the LoS/NLoS indicator(s) associated with DL RSTD, the UE may report one indicator associated with the dl-PRS-ID indicated by higher layer parameter dl-PRS-ReferenceInfo and one indicator associated with the dl-PRS-ID of the DL RSTD measurement. A UE may be provided with LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator, and it may be associated with each DL PRS resource of each configured dl-PRS-ID or may be associated with each configured dl-PRS-ID. The values of the higher layer parameter LOS-NLOS-Indicator may be soft values (0, 0.1, …, 0.9, 1) or hard values (0, 1) with the values corresponding to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS.

For the LoS/NLoS indictor soft value, it is a probability as (0, 0.1, …, 0.9, 1) and it is not possible to define requirement for the soft value. But for the hard value, it is (0, 1) with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS, and it is possible to define a metric to reflect the estimated accuracy of LoS/NLoS environment.

Observation 1: It is maybe not possible to define requirement for the LoS/NLoS indicator soft value.

Here we give an example for accuracy and test of LoS/NLoS indicator hard value as illustrated in Fig. 1.

Fig. 1. The illustration of an example of accuracy and test for LoS/NLoS indicator hard value.

Where M is the number of samples/paths, N is the number of AI/ML inference times, M1 is the number of times of misestimation. P0 is the accuracy requirement under a certain condition, which is a probability, e.g., 90%. P1avg is the averaged estimated probability for LOS/NLOS indicator.

Proposal 6: RAN4 to define performance requirement for LoS/NLoS indicator hard value, the accuracy requirement can be the probability of maximum LoS/NLoS misestimation.

- 2.3	Potential RAN4 requirement for case 3

For the UL positioning method case 3a and 3b, the model is deployed in gNB side and LMF side, respectively. RAN4 does not need to define any core requirement for such 2 cases.

Proposal 7: RAN4 not to define requirements for positioning case 3.

- 2.4 potential RAN4 requirement for performance monitoring

In RAN1#116bis and 118 meeting, following agreements about performance monitoring for case 1 and case 3a are reached as

RAN1#116bis Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. RAN1#118 For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

For case 3a, the monitoring procedures are performed at NW side and no need to define delay requirement for RAN4. For case 1, there are many options under discussion that may have RAN4 impacts for delay requirements.

For option A-1, the label-based monitoring metrics calculation is performed at UE side and the ground truth label is generated by LMF and provided to the target UE. In that case, LMF may indicate UE to perform monitoring procedure through signaling and then the target UE may inform LMF the monitoring decision or monitoring metrics calculation results (NW decision) through report. These procedures imply a delay requirement and the details are depending on RAN1/2 progress.

For option A-2 and 3, the label-based monitoring metrics calculation is performed at UE side and some assistance data can be transferred from LMF to UE side. These procedures may also have delay requirement and more progress of RAN1/2 are needed for discussing.

For option A-4, the PRU measurement are sent from PRU to the target UE, it is more like implementations and vendors could use different ways for sending information, e.g., OTT server, sidelink. Since RAN1 has agreed that it can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. RAN4 would not discuss this option for defining any core requirements.

For option B-1, the label-based monitoring metrics calculation is performed at LMF side and the model inference needs to be sent from target UE to LMF. In that case, it is very clear for RAN4 to identify the delay requirement. For delay requirement, the LMF may indicate UE to perform AI operations, and then UE reports the inference results (e.g. AI estimated UE position) to LMF.

For option B-2, the target UE uses the PRU channel measurements as the model input and outputs the estimated PRU position as the model inference result to report to LMF for performing monitoring procedure. It is also very clear that these procedures may also have the delay requirement from RAN4 perspective.

It can be observed that the delay requirements for many options about the monitoring procedure in case 1 needs to be defined, but more progress and down-selection from RAN1/2 are needed for delated further discussion.

Proposal 8: RAN4 to wait for more progress from other WGs to consider define delay requirements for monitoring for positioning case 1.

- Summary

In this contribution, we provided our views on AI/ML based positioning. Based on above analysis, following observations and proposals are present.

Proposal 1: RAN4 to define delay requirement for case 1 at least considering the following procedures: PRS measuring, AI processing, UE reporting location to LMF.

Proposal 2: RAN4 to consider the straight-line distance (unit: m) between the real position and the estimated position as the metric for accuracy requirement for case 1.

Proposal 3: RAN4 to postpone the discuss of requirement for positioning case 2b.

Proposal 4: RAN4 to define delay requirement for case 2a at least considering the following procedures: PRS measuring, AI processing, UE reporting intermediate features to LMF.

Proposal 5: RAN4 to reuse the existing accuracy requirements for the timing-related intermediate features: RSTD, UE Rx-Tx time difference for positioning case 2a.

Observation 1: It is maybe not possible to define requirement for the LoS/NLoS indicator soft value.

Proposal 6: RAN4 to define performance requirement for LoS/NLoS indicator hard value, the accuracy requirement can be the probability of maximum LoS/NLoS misestimation.

Proposal 7: RAN4 not to define requirements for positioning case 3.

Proposal 8: RAN4 to wait for more progress from other WGs to consider define delay requirements for monitoring for positioning case 1.

- References

- TR 38.842 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

- R4-2417212 WF on AI/ML Qualcomm Incorporated