3GPP TSG RAN WG4 Meeting #113	R4-2418755

Orlando, Florida, USA, 18th – 22nd November, 2024

Agenda item:			7.18.2

Source:	vivo

Title:	Discussion on impacts on RAN4 requirements for AI mobility

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we provide our initial views on impacts on RAN4 requirements for AI/ML mobility.

- Discussion

- 2.1 Potential requirements in AI/ML for mobility

In the RAN4#112bis meeting, the following agreements are captured in the WF [2] as

Issue 1-1-1: Priorities of different use cases Agreement: - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting. Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025). Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Agreement: - Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed. Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Scenario clarifcation

For the scenarios 2,3,4, we would like to provide further clarifications on the sub-scenarios from RAN4 perspective:

Scenario 2:

- Serving cell (input) to predict the serving cell (output)

- Neigbour cell i (input) to predict the neigbour cell i (output).

Scenario 3:

- Serving cell (input) to predict inter-frequency neigbour cell i (output)

- Neigbour cell i (input) to predict co-located neigbour cell j (output), where i≠j

  - Neigbour cell i and neigbour cell j are inter-frequency

Scenario 4:

- Serving cell (input) to predict the serving cell (output)

- Neigbour cell i (input) to predict the Neigbour cell i (output).

Note that in RAN2 understanding, the inter/intra-frequency description is only for the cells corresponding to AI input and output, respectively. This description is different with the RAN4 understanding that the inter/intra-frequency description is for the relationship of the center frequency between the serving cell and the other cells.

Observation 1: RAN2 understanding for inter/intra-frequency is for descriping the relationship of the center frequency between the cell corresponding to AI input and the cell corresponding to AI output, which is different with RAN4 understanding.

Besides, it will be hard to performing AI prediction with only RSRP input for sencario 3. More information types, such BS Tx power, center frequency, co-located information, cell ID, etc, maybe also needed as model input or other conditions to improve the accuracy and robustness of model outputs.

Observation 2: More information types except RSRP, e.g., BS Tx power, center frequency, co-located information, cell ID, etc., maybe also helpful as model input or other conditions to performing AI prediction in scenario 3.

Sub-use cases clarification:

In Fig. 1, beam-level results in A and A1 are all possible to be the input and output in the current stage. Based on the agreed sub-use cases and Fig. 1, we have further clarifications as:

- Sub-use case 1: Use A to predict A, or A1 to predict A1, or A to predict A1, and calculate C;

- Sub-use case 2: Use C to predict C;

- Sub-use case 3: Use A to predict C, or use A1 to predict C;

Where point A refers to the measurements (beam-specific samples) internal to the physical layer. And point A1 refers to the measurements (i.e. beam specific measurements) reported by layer 1 to layer 3 after layer 1 filtering. The internal layer 1 filters the inputs obtained at point A. The implementation (inputs A and Layer 1 filtering) of how the measurements are executed in the physical layer is not constrained in the standard.

Fig. 1 Measurement Model

For further simplifiying the RAN4 discussion, we have the following proposals for scenario and sub-use case clarification based on the above analysis:

Proposal 1: RAN4 to take the following cases as the start point for scenarios 2,3,4

Scenario 2:

- Serving cell (input) to predict the serving cell (output)

- Neigbour cell i (input) to predict the neigbour cell i (output).

Scenario 3:

- Serving cell (input) to predict inter-frequency neigbour cell i (output)

- Neigbour cell i (input) to predict co-located neigbour cell j (output), where i≠j

  - Neigbour cell i and neigbour cell j are inter-frequency

Scenario 4:

- Serving cell (input) to predict the serving cell (output)

- Neigbour cell i (input) to predict the neigbour cell i (output).

Proposal 2: RAN4 to take the following cases as the start point for sub-use cases

Sub-use case 1:

- Use A to predict A, or A1 to predict A1, or A to predict A1, and calculate C;

Sub-use case 2:

- Use C to predict C;

Sub-use case 3:

- Use A to predict C, or use A1 to predict C;

Performance Metrics/KPIs for Measurement Prediction use case

For the performance merics issue, the KPI needs to be discussed case by case.

For sub-use case 1, A/A1 point values are as input to predict the A/A1, than generate C point values by applying L3 filtering algorithm which is configured by RRC signalling. Though point C value is the metric to be eventually reported, there is no necessary to define performance requirement. From our perspective, define prediction accuracy requirement for point A1 is enough since the L3 filtering is a nearly linear algorithm, the quality of A1 value will lineary impact the quality of point C. Besides, there will be hundreds of combinations for L3 filtering when considering the RRC parameters filterCoefficient so that the ground truth in point C will be hard to choose. Note that a fixed L3 filtering value is used for RAN2 evaluation as

Table 1 Fixed L3 filtering parameter in RAN2 evaluation

| L3 filtering parameter | value |
| --- | --- |
| FR1 FilterCoefficient | 4 |
| FR2 FilterCoefficient | 4 |

For sub-use case 2 and 3, since the prediction metrics are all from point C, RAN4 needs to define prediction accuracy in point C for such two cases. However, RAN4 still needs to choose an applicable the ground truth value in point C when consider the hundreds combinations for L3 filtering when define accuracy requirement.

Besides, for sub-use case 1,2,3, there will be no L3 beam-level value (Point E) and no need to consider in RAN4.

Proposal 3: RAN4 only to consider the AI predict metric as the KPI and study for each cases:

Sub-use case 1:

- Evaluate prediction accuracy for point A/A1

Sub-use case 2:

- Evaluate prediction accuracy for point C

Sub-use case 3:

- Study performance metrics for point C with considering:

  - RAN4 to find out how to obtain the ground truth value by considering the RRC configuration, e.g., L3 filtering.

Proposal 4: RAN4 not to study the performance metric for L3 beam-level value (Point E).

- 2.2 Simulation aspects

RAN2 has agreed the simulation assumptions as Table 5.1-1 in TR 38.744. Both beam level and cell level results for training and inference all derived from the system level simulation in UMi and UMa channel. From RAN4 perspective, it may need to consider additional factors that may not be considered in RAN2 evaluation on prediction accuracy, e.g., measurement error modelling. At the initial study stage in RAN4 discussion, it can be considered as one of the study aspects. For beam level results, the measurement errors can be modelled as some types of probability distributions, or derived from link level simulations. System level simulation based beam level RSRPs are used as ideal dataset and add the beam level errors into the ideal dataset to derive the nonideal dataset to evaluate the measurement error impact is one feasible option which is also discussed in AI BM simulation assumptions in RAN4#112bis [3]. Besides, if the model input is the cell-level result, there are two options for error modelling. First is to use the L3 filter to the nonideal beam-level results mentioned above to derive the nonideal cell-level results, second is to directly modelling the cell-level errors to add into the ideal cell-level dataset and the modelling method needs to be further studied and both options could be considered in RAN4. Since it is still in SI phase, RAN4 could evaluate the performance when considering measurement errors for defining requirement in the future WI phase.

Proposal 5: RAN4 to evaluate the prediction accuracy performance with consideration of the impact of measurement error in WI .

- Summary

In this contribution, we provide our initial views on impacts on RAN4 requirements for AI/ML mobility. Based on above analysis, following observations and proposals are present.

Observation 1: RAN2 understanding for inter/intra-frequency is for descriping the relationship of the center frequency between the cell corresponding to AI input and the cell corresponding to AI output, which is different with RAN4 understanding.

Observation 2: More information types except RSRP, e.g., BS Tx power, center frequency, co-located information, cell ID, etc., maybe also helpful as model input or other conditions to performing AI prediction in scenario 3.

Proposal 1: RAN4 to take the following cases as the start point for scenarios 2,3,4

Scenario 2:

- Serving cell (input) to predict the serving cell (output)

- Neigbour cell i (input) to predict the neigbour cell i (output).

Scenario 3:

- Serving cell (input) to predict inter-frequency neigbour cell i (output)

- Neigbour cell i (input) to predict co-located neigbour cell j (output), where i≠j

  - Neigbour cell i and neigbour cell j are inter-frequency

Scenario 4:

- Serving cell (input) to predict the serving cell (output)

- Neigbour cell i (input) to predict the neigbour cell i (output).

Proposal 2: RAN4 to take the following cases as the start point for sub-use cases

Sub-use case 1:

- Use A to predict A, or A1 to predict A1, or A to predict A1, and calculate C;

Sub-use case 2:

- Use C to predict C;

Sub-use case 3:

- Use A to predict C, or use A1 to predict C;

Proposal 3: RAN4 only to consider the AI predict metric as the KPI and study for each cases:

Sub-use case 1:

- Evaluate prediction accuracy for point A/A1

Sub-use case 2:

- Evaluate prediction accuracy for point C

Sub-use case 3:

- Study performance metrics for point C with considering:

  - RAN4 to find out how to obtain the ground truth value by considering the RRC configuration, e.g., L3 filtering.

Proposal 4: RAN4 not to study the performance metric for L3 beam-level value (Point E).

Proposal 5: RAN4 to evaluate the prediction accuracy performance with consideration of the impact of measurement error in WI .

- References

- RP-242393 Revised SID on AIML for mobility in NR, OPPO

- R4-2416855 WF on AI/ML Mobility, Nokia

- R4-2417211 Simulation assumptions for beam prediction, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT