3GPP TSG RAN WG4 Meeting #113	R4-2418758

Orlando, Florida, USA, 18th – 22nd November, 2024

Agenda item:			7.15.2.2

Source:	vivo

Title:	Discussion on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

- Discussion

- 2.1 L3 measurement delay reduction by optimizing CSSF for UE not in multiple-Rx simultaneous reception mode

RAN4#112 Issue 2-1: Clarification on the bullets in WID for this CSSF optimization [Way forward]: FFS: - Option a (ZTE, CMCC, Intel, CATT, MTK): - Rel-19 CSSF optimization applies for the both cases: (1)UE is not capable of Rel-18 multi-Rx simultaneous reception, (2)UE is capable of Rel-18 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently. - Note: if option a is agreeable, then work on wording polishing based on option 1a/1b. - Option b (HW, MTK, Samsung, E///): - Rel-19 CSSF optimization applies for case that UE is not capable of Rel-18 multi-Rx simultaneous reception. - Option c (Apple, Nokia, QC): - Rel-19 CSSF optimization applies for CA/DC scenarios with at least two FR2 serving cells, independently of the UE support of multi-Rx capabilities. - Option d (newly added): - Same Rel-19 CSSF optimization applies for the both cases: (1) UE is not capable of Rel-18 and Rel-19 multi-Rx simultaneous reception, (2) UE is capable of Rel-18 or Rel-19 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently.

For the issue of clarification on the bullets in WID for this CSSF optimization, it was discussed in RAN#105 and “for UE not in multiple-Rx simultaneous reception mode” has been revised to “assuming a UE not operating with multiple-Rx based simultaneous L3 measurement”.

From our perspective, there may be different understanding and some clarifications are needed for this revised bullet. The intention for the original wording is assumed that there is no multi-Rx, but the revised wording assumes that there would be no simultaneous L3 measurement operation. In Rel-18, simultaneous L1 measurements feature was introduced and there may be some different impacts to CSSF reduction in CA/DC between simultaneous L3 and simultaneous L1 measurements, which both are considered as multi-Rx. If simultaneous L3 measurements are based on the assumption that one searcher for measurements on a single carrier with multi-Rx reception, there would be no impact to CSSF reduction, which is similar as the measurements on the carrier with single panel. Therefore, there is no need to have such limitations for CSSF optimization.

Besides, multi-Rx is for single-carrier as the baseline and the CSSF optimization is for CA/DC. Although the other CA/DC modes for BSF reduction is FFS, there may be nothing to specify for this scenario in the end since there in difference scenarios. The progress we made is still sufficient and the current clarification in main bullet still needs to be revised from our perspective.

In summary, it is premature to have this clarification “assuming a UE not operating with multiple-Rx based simultaneous L3 measurement” in the main bullet and it depends on the RAN4 progress. RAN4 could find solutions for CSSF optimizations without considering the impact of multi-Rx and come back to this issue after enough progress are made.

Proposal 1: RAN4 to develop CSSF optimization solutions without considering multi-Rx and further study if it is applicable to multi-Rx scenario after more progress are made.

RAN4#112 [Way forward]: FFS: - Option 1 (CATT): Not to preclude the solutions based on 3 searchers assumption in current stage. - Option 1a (CMCC, HW): it is proposed to consider both the CSSF optimization based on 2 searchers and the CSSF optimization based on 3 searchers. - Option 1b (QC): RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS). - Option 2 (Apple, CTC, Nokia, Ericsson): RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF. - Option 2a (Apple): If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. - Option 2b (Nokia) RAN4 to confirm if one of the searchers is assumed for PCC/PSCC measurement and the other is assumed for the measurements on all SCCs. RAN4#112bis Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF Agreement in online session: On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers: - Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later.

For UE not in multiple-Rx simultaneous reception mode in CA/DC scenario, the current delay requirements for measurement outside gap are defined based on 2 searchers assumption for total CCs. For FR2, more resources are needed and another independent searcher may be needed. In the current specification, a UE support 2-searcher is assumed as a PCC use one searcher, and SCC share the other searcher. However, consider the scenario that a UE is measuring a FR1 PCC and a FR2 SCC outside gap (CSSF=1), while a FR2 SCC is within the gap. In this scenario, a UE support 3-searchers measurement is perhaps implied to be considered to meet the requirements. However, changing the number of searchers is a very challenged job for vendors and whether the 3 searcher is a missing piece of current spec needs to be discussed. According to the WID, RAN4 could work with the assumption as 2 searchers as the baseline for CSSF reduction. For 3 searchers, RAN4 could also discuss feasible way and find the applicable conditions, but not to delay the WI completion.

Observation 1: It is a very challenged job to change the number of searchers and 3 searcher-implementation may impacts a lot of aspects of UE architecture.

Observation 2:3-searchers based requirements only applicable in limited scenario(s) and RAN4 workload needs to be considered in WI.

RAN4#112bis Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC Candidate options for further discussion: - Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B - Option 1A: Reuse the legacy principle for neighboring cell measurement. - Option 1B: Based on network indication - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band. - Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions For 3 searchers: In addition to option 1 and 2 - Option 3: consider to use 3 searchers for CSSF reduction. - Depending on issue 2-4.

For the solution to CSSF optimization in CA/DC, option 1 and option 2 are all applicable. For option 1 and option 2, potential solution may be:

Opt 1: UE only needs to measure one serving CC per band if multiple serving CCs are in the same band

- Measure PCC if PCC in band or

- Measure PSCC if PSCC in band or

- Measure SCC with neighbor cell MO or

- UE implementation

Opt 2: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

- Dynamic ratio changed as, e.g.,

  - PCC/PSCC (1st searcher 100%) to PCC/PSCC (1st searcher X%);

  - SCC (1st searcher 0% and 2nd searcher 100%) to SCC (1st searcher 1-X% and 2nd searcher 100%);

  - FFS details and X value, e.g., 50.

- FFS whether specific conditions for this ratio change are needed

Proposals 2: RAN4 to consider option 1 and option 2 as baseline:

Option 1: UE only needs to measure one serving CC per band if multiple serving CCs are in the same band

- Measure PCC if PCC in band or

- Measure PSCC if PSCC in band or

- Measure SCC with neighbor cell MO or

- UE implementation

Option 2: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

- Dynamic ratio changed as, e.g.,

  - PCC/PSCC (1st searcher 100%) to PCC/PSCC (1st searcher X%);

  - SCC (1st searcher 0% and 2nd searcher 100%) to SCC (1st searcher 1-X% and 2nd searcher 100%);

  - FFS details and X value, e.g., 50.

- FFS whether specific conditions for this ratio change are needed

- Summary

In this contribution, we provided our views on FR2-1 SSB based L3 measurement delay reduction for connected mode. Based on above analysis, following observations and proposals are present.

Proposal 1: RAN4 to develop CSSF optimization solutions without considering multi-Rx and further study if it is applicable to multi-Rx scenario after more progress are made.

Observation 1: It is a very challenged job to change the number of searchers and 3 searcher-implementation may impacts a lot of aspects of UE architecture.

Observation 2:3-searchers based requirements only applicable in limited scenario(s) and RAN4 workload needs to be considered in WI.

Proposals 2: RAN4 to consider option 1 and option 2 as baseline:

Option 1: UE only needs to measure one serving CC per band if multiple serving CCs are in the same band

- Measure PCC if PCC in band or

- Measure PSCC if PSCC in band or

- Measure SCC with neighbor cell MO or

- UE implementation

Option 2: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

- Dynamic ratio changed as, e.g.,

  - PCC/PSCC (1st searcher 100%) to PCC/PSCC (1st searcher X%);

  - SCC (1st searcher 0% and 2nd searcher 100%) to SCC (1st searcher 1-X% and 2nd searcher 100%);

  - FFS details and X value, e.g., 50.

- FFS whether specific conditions for this ratio change are needed

- References

- R4-242380 Revised WID: NR Radio Resource Management (RRM) Phase 5

- R4-2416853 WF on RRM requirements for NR_RRM_Ph5_Part1