# 3GPP TSG-RAN WG4 Meeting #113	R4-2418768

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Source: 	OPPO

Agenda item:	7.15.2.1

Document for:	Discussion

Introduction

In last meeting, a WF[1] was approved on NR_RRM_Ph5_Part1. For Topic#1 FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, the following agreements have been achieved.

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF: Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment. Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” : Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656. Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF Agreement: - RAN4 is to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19. - The detailed definition can be postponed to the end of the core part discussion.

In this contribution, we will provide our views on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor.

Discussion

In our view, two issues should be discussed for UE power consumption due to FBS with multi-Rx operation: how to define entering and exiting rules of FBS, and how UE can meet the conditions and corresponding requirements.

For entering rules of FBS, RAN4 agreed to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions.

Alt 2: RAN4 to consider conditions for triggering FBS for L3 measurement f, including active condition and de-active conditions. - Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions: - Alt 2-1 - UE location (cell centre or cell edge) - UE mobility (stationary or moving) - Both above - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline - Alt 2-2 - faster beam sweeping always applies when a L3 mobility event is met. FFS whether additional signalling is needed.

We prefer to consider the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused. UE location (cell centre or cell edge) and UE mobility (stationary or moving) can be set as conditions to activate and deactivate FBS for L3 measurement.

Proposal 1: For entering rule, support to consider UE location and UE location status as the conditions to activate and deactivate FBS for L3 measurement. The existing criteria can be reused.

For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”: - For Scenario 1/2/3/4, - Option 1 (CATT, LGE, OPPO, MTK): use UE indication of Multi-Rx preference for L3 measurement and further discuss whether to reuse the existing signaling. - FFS on option 1a/1b/1c. - Option 3 (Samsung, Ericsson): enable FBS in a time period after FBS is triggered by the condition in issue 1-1-1 - FFS on option 3a/3b. - Option 5 (Samsung): UE autonomously fallback based on measurement - For Scenario 5/7, - FFS: disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed.

For exiting rules, UE is allowed to turn the panels off even during conducting FBS procedure. Once the delay requirements of FBS can be met, UE still has the chance to fall back to single panel or deactivate multi-Rx for some while. We are not sure it is mandatory to ensure network can know this immediately, but UE assistance information can be beneficial for network scheduling optimization. In our view, either the legacy signalling like the overheating indication, or new UAI indication of UE preference is feasible. We can leave these options to RAN2 decision.

Besides the conditions for activation and de-activation in Proposal 1, also support to consider either the legacy overheating indication or new UAI indication and it should be up to RAN2 decision.

Proposal 2: For exiting rule, besides the conditions in Proposal 1，also support to consider either the legacy overheating indication or new UAI indication. It should be up to RAN2 decision.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC [Moderator]: FFS the following summarized options. (CA/DC is low priority, and therefore CA/DC based options are not reflected below): - For scenario 1/2/3/4 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms - Option 2: no need to add additional processing time in the delay requirement. - For scenario 5 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 2: In the handover delay equation, SSB processing time (2 ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - For scenario 7 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 2: no need to add additional processing time in the delay requirement.

As agreed in last meeting, scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios.

For SSB processing delay for multiple beams received in a SMTC, we prefer option 1 for scenario 1/2/3/4/7, considering additional processing delay after the last SMTC periodicity. For Scenario 5, we also prefer to consider the extended SSB processing delay. We think the solution proposed in option 2 for scenario 5 can apply for all scenarios, by scaling the SSB processing time by the number of RX beams that UE employs to simultaneously receive signals from multiple directions, which is defined as M.

For scenario 1/2/3/4/7, the additional SSB processing delay is M*TSSB_processing, and for scenario 5, the additional SSB post-processing delay is M* Tmargin, where TSSB_processing, and Tmargin can be up to 2ms.

Proposal 3: For scenario 1/2/3/4/7, the additional SSB processing delay is M*TSSB_processing, and for scenario 5, the additional SSB post-processing delay is M* Tmargin, where TSSB_processing, and Tmargin can be up to 2ms.

Issue 1-4: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF - Option 1 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Option 2 (CATT, vivo, CMCC, Apple, ZTE, CTC, NTT DCM, Nokia, OPPO, MTK): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability - Option 2a (CATT, CMCC, ZTE, CTC): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Option 2b (vivo): Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4. - Option 2c (Apple, CTC): For UE supporting R19 multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Option 2d (Nokia): At least support reduction of N from 8 to 4 in Rel-19. Support of other values such as N = 2, 6 to be discussed, and UE capability signalling to be introduced, if more than one value for reduced N is supported. - Option 2e (OPPO): Discuss the reduced scaling factors due to Rx BSF, considering both UE capabilities and applicable conditions. - Option 3 (vivo, Samsung, LGE): Introduce the UE capability to indicate the reduced M value (M=sample*BSF), - Option 3a (vivo): e.g., M=12, 14, 16, etc. - Option 3b (Samsung): suggest to take N=4, M=20 as the baseline. Whether to/How to define the other values can be based on SLS results (if necessary). - Option 3c (LGE): 3 or 4 Rx BSF value or additional scaling factor < 1 to reduce M value - Option 4 (vivo, LGE): - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc. - Option 5 (Ericsson): - In order to cover all possible directions of neighbor cells, RX beam sweep doesn’t dynamically change, the factor can be defined as a single number, e.g., [4]. - Option 6 (Intel): - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Beam sweeping factor/total sample number of measurements can be cut half as a baseline while a margin can be added to the cut factor according to different SSB/SMTC configurations. - Recommended WF - [Moderator]: discuss option 2 and option 3 together with the scenarios 1/2/3/4/5/7.

Besides the different values of RX beam sweep factors, the corresponding conditions are also considered. Taking FR2 intra-frequency L3 measurements as example, the legacy M considers the beam sweep factor as 8 for different UE power classes. The new factor M can be decided by both UE capability and side conditions. Different values can apply for different cases. We prefer option 2 that for UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability

Proposal 4: Define the reduced scaling factors due to Rx BSF based on different UE capabilities.

Conclusions

In this paper we provided our views on FR2-1 SSB based L3 measurement delay reduction for optimizing BSF. The followings are provided.

Proposal 1: For entering rule, support to consider UE location and UE location status as the conditions to activate and deactivate FBS for L3 measurement. The existing criteria can be reused.

Proposal 2: For exiting rule, besides the conditions in Proposal 1，also support to consider either the legacy overheating indication or new UAI indication. It should be up to RAN2 decision.

Proposal 3: For scenario 1/2/3/4/7, the additional SSB processing delay is M*TSSB_processing, and for scenario 5, the additional SSB post-processing delay is M* Tmargin, where TSSB_processing, and Tmargin can be up to 2ms.

Proposal 4: Define the reduced scaling factors due to Rx BSF based on different UE capabilities.

Reference

- R4-2415656, WF for NR_RRM_Ph5_Part1, Apple