# 3GPP TSG-RAN WG4 Meeting #113	R4-2418777

Orlando, US, 18th – 22nd November, 2024

Title: 	On (e)RedCap RRM requirements of R19 NR NTN

Source: 	OPPO

Agenda item:	7.26.4.1

Document for:	Discussion

Introduction

In last RAN4 meeting, a WF on RRM requirements for Rel-19 NR NTN phase3 was approved [1], including downlink coverage enhancements, support of regenerative payload, and support of (e)RedCap UEs with NR FR1-NTN.

In this paper, we will provide our views on support of (e)RedCap UEs with NR FR1-NTN.

Discussion on support of (e)RedCap UE with NTN

<Agreement>: - Define NR Rel-17 and Rel-18 Conditional Handover requirements. - Include location-based CHO and time-based CHO. - Support earth moving cell scenario - Define RACH-less Handover requirements. - Define SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state. - FFS TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - FFS: MDT requirements in RRC_IDLE and RRC_INACTIVE state.

For SDT requirements, regarding TA validation based on the RSRP change criterion, we think it can be removed for Redcap NTN due to the un-reliability of RSRP evaluation, and the requirements may refer to the PUR requirements for IoT for NTN.

For MDT requirements, since no RAN4 MDT requirement for RedCap TN UE, we prefer not to define MDT requirements for Redcap NTN UE considering the similar UE battery, storage and computing capability.

Proposal 1: TA validation based on the RSRP change criterion is not supported for Redcap NTN UE.

Proposal 2: Not to define MDT requirements for Redcap NTN UE.

<Agreement>: - FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline. - If defined, the requirements apply provided that target cell is GSO

In last meeting, RAN4 agreed on a list of requirements defined for both NTN and (e)RedCap UE requirements, only defined for NTN, and defined for (e)Redcap and not been defined for FR1-NTN, which are the most fundamental ones for the basic RRM related procedures. But for other RedCap features, e.g., eDRX, NCD-SSB and R17 relaxed measurement, we think RAN4 needs to discuss case by case.

For GSO, Rel-17 relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands can be applied, and the corresponding requirements defined for RedCap UE can be the baseline.

Agreement: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO

Similarly, for eDRX, in legacy NTN requirement, 2.56s cycle is not assumed to be well utilized in the scenario for earth-moving LEO as serving cell or target satellite. If eDRX in PTW window is greater than 2.56s, UE may drop the measurement/detection/evaluation and cannot meet the measurement requirements. Thus, we suggest UE is not required to meet the RRM requirement with eDRX for earth moving LEO deployment.

Proposal 3: For earth moving LEO deployment, Redcap NTN UE is not required to meet the RRM requirement with eDRX and RedCap relaxed measurement requirements defined in R17.

RAN4 has agreed to define separate sets of RRM requirements for 1Rx and 2 Rx (e)Redcap UE with FR1-NTN bands. For 2Rx (e)RedCap UEs with FR1-NTN, RAN4 reuses the existing requirements for NTN as a baseline. For 1RX (e)RedCap UE, we can also relax measurement requirements including extending the measurement delays with more measurement samples and relaxing the measurement accuracy.

Proposal 4: For 1RX (e)RedCap UE with FR1-NTN bands, support to relax measurement requirements including extending the measurement delays with more measurement samples and relaxing the measurement accuracy.

The most of  NTN features should be supported in RedCap + NTN requirements. But as companies pointed out, one exception is the support of multiple SMTCs. So far, NTN is mandatorily required to support 2 SMTCs per MO, and supporting more is an optional capability. When the two SMTCs do not overlap there is no scaling in measurement period meaning UE has to support parallel measurement for the two SMTCs. This may be a bit challenging for (e)RedCap UE with reduced complexity. We also share the similar concern on support of multiple SMTCs parallelly.

Proposal 5: RAN4 to not consider the support of measurements with multiple SMTCs in parallel for (e)RedCap UE with FR1-NTN bands.

Conclusions

In this paper we provided our views on RRM requirements for support of (e)RedCap UEs with NR FR1-NTN.

Proposal 1: TA validation based on the RSRP change criterion is not supported for Redcap NTN UE.

Proposal 2: Not to define MDT requirements for Redcap NTN UE.

Proposal 3: For earth moving LEO deployment, Redcap NTN UE is not required to meet the RRM requirement with eDRX and RedCap relaxed measurement requirements defined in R17.

Proposal 4: For 1RX (e)RedCap UE with FR1-NTN bands, support to relax measurement requirements including extending the measurement delays with more measurement samples and relaxing the measurement accuracy.

Proposal 5: RAN4 to not consider the support of measurements with multiple SMTCs in parallel for (e)RedCap UE with FR1-NTN bands.

Reference

- R4-2416865, WF on RRM requirements for Rel-19 NR NTN phase3, CATT