3GPP TSG-RAN WG4 Meeting #113	R4-2418791

Orlando, US, November 18th – 22nd 2024

Agenda Item:	7.20.2.1

Source:	Ericsson

Title:	Potentially new SBFD BS RF requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of Potentially new BS RF requirements for SBFD slots. Discussion on impact on existing BS RF TX and RX requirements for SBFD slots and SBFD general aspects are presented in separate submissions [1-3].

# 2	Discussion

Transient period

##### Issue 2-1-1: Transient period requirement applicability on frequency/antenna Way Forward: - FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498. ##### Issue 2-1-2: Location of transient period between SBFD and non-SBFD Way Forward: - FFS location of transient period between SBFD and non-SBFD: - Option 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not. - Option 2: Place the transient period outside of both SBFD and non-SBFD slots. - Option 3: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol).

In the WF [4] approved during RAN4#112bis, one issue remains regarding the transient period, i.e., the location of the transient period between SBFD and non-SBFD in RAN4 RF requirement. In legacy TDD, the transient period is considered part of the guard period, as illustrated in Figure 1 below, where TGUARD represents the total allocated guard period (including both DL to UL and UL to DL guard periods); TAoffset is the guard period allocated for the UL to DL switch; TDL_UL is the guard period allocated for the DL to UL switch. Our understanding is the transient period is located outside of both the DL slot and the UL slot in legacy TDD. Therefore, we believe that the location of the transient period between SBFD and non-SBFD should be consistent with its placement in legacy TDD.

Figure 1: Illustration of guard period for legacy TDD

Place the transient period outside of both SBFD and non-SBFD slots.

Another remaining issue is to further study the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498, as shown below.

Proposal 1: The following text proposal is given for the clause of transmitter transient period for SBFD: =================== Start of Text Proposal =================== ### 6.4.2B Transmitter transient period for SBFD #### 6.4.2.1B General Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS. The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1. The transmitter transient period for SBFD shall be measured in the SBFD TX and RX antenna penal and transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD antenna configuration: Table 6.4.2.1B-1: Transmitter transient period for SBFD Frequency regions for transition; SBFD antenna configuration Option-1 (Same panel area as TDD); SBFD antenna configuration Option-2&3 (Twice panel area as TDD); BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); SBFD RX antenna panel; N/A; BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; SBFD TX antenna panel; SBFD TX antenna panel; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); SBFD RX antenna panel; N/A; BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; SBFD TX antenna panel; SBFD TX antenna panel; BS transmitter OFF-to-ON. For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band. =================== End of Text Proposal ===================

It is proposed that transmitter transient period for SBFD is not applicable to Case-A (SBFD symbol to DL symbol) and Case-C (DL symbol to SBFD symbol) when SBFD antenna configuration is Option-2&3 (Twice panel area as TDD). In our view, for Case-C (from DL symbol to SBFD symbol), switching off part of the TX antenna panel, i.e., the DL subband resource blocks that are corresponding to the UL subband resource blocks, is the same as switching off the whole antenna panel from TX to RX. In addition, RAN4 should not define an architect dependent requirement. Therefore, we don’t think the details in Table 6.4.2.1B-1: Transmitter transient period for SBFD should be captured in the spec.

RAN4 do not define an architect dependent requirement. Transient period between SBFD and non-SBFD is applicable to all four cases.

In-channel adjacent sub-band leakage ratio

[Background] In RAN4#112, the following WF is approved: - Way forward: - FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.. Way Forward: - FFS based on the way forward approved in RAN4#112.

In the approved WF [4] during RAN4#112bis, the necessity of In-channel adjacent sub-band leakage ratio requires further study.

Even though conformance testing does not include other sites, a new requirement needs to be set to enable site dimensioning design in real networks. While self-interference is captured in the OTA sensitivity, interference from other sectors and other sites need to be considered, i.e., requirements on ACLR and ACS/blocking between the sub-bands are needed, which is the basis for site dimensioning design. Moreover, in the feasibility analysis, ACLR and ACS/blocking suppression between sub-bands were assumed by all companies.

Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

In-channel adjacent sub-band Blocking and adjacent sub-band selectivity

[Background] In RAN4#112, the following WF is approved: - Way forward: - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline. - Way forward: - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.. Way Forward: - FFS based on the way forward approved in RAN4#112. - Consider the possibility of specifying RX requirement to cover both in-channel adjacent subband selectivity/blocking requirements

In the approved WF [4] during RAN4#112bis, the necessity of In-channel adjacent sub-band Blocking and adjacent sub-band selectivity requires further study and consider the possibility of specifying RX requirement to cover both in-channel adjacent subband selectivity/blocking requirements.

The definition of Adjacent channel selectivity (ACS) in TS 38.104 [5] is,

a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

The definition of in-band blocking in TS 38.104 [5] is,

a measure of the receiver's ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

From the definition, we know that ACS is the requirement to consider an unwanted interferer (e.g., -52 dBm for WA) from the adjacent channel, while in-band blocking is the requirement to consider an unwanted interferer at a higher level comparing to ACS (e.g., -43 dBm for WA) within the same frequency band. In SBFD, there is only one adjacent DL sub-band, which makes adjacent sub-band Blocking and adjacent sub-band selectivity somewhat the same requirements with two different interferer levels. Therefore, we propose to define a new requirement on RX sub-band blocking similar to the in-band blocking requirement only and use existing in-band blocking requirement as baseline.

Define a requirement on RX sub-band blocking similar to the in-band blocking requirement and use existing in-band blocking requirement as baseline.

Inter-sector and inter-site interference

During the feasibility studies of SBFD, the importance of inter-sector and inter-site interference was thoroughly discussed. The inter-sector interference (depicted in Figure 1) was a parameter with specific feasibility tables. In [6, 7], the need to capture inter-sector and inter-site interference handling via a new requirement was discussed in detail. In this section, we re-iterate the discussion and emphasize the need to capture at least inter-sector interference handling as most of the sites are sectorized. In addition, from testing perspective, inter-sector interference handling is fully testable as RAN4 specifications currently contain OTA co-location testing. The inter-site interference is difficult to capture in the lab environment and thus can be handled as operator considerations while inter-sector interference handling can be fully tested. Given the importance of inter-sector interference handling of SBFD BS and the fact there are currently OTA test methods for testing the inter-sector interference based on OTA co-location, we propose to add inter-sector interference handling via a new requirement which can be expressed as allowed sensitivity degradation [0.5-1 dB] of UL sub-band.

New requirement to capture Inter-sector interference shall be adopted.

Figure 1: Illustration of inter-sector interference

# Conclusion

In this contribution, overview of impact on SBFD BS RF requirements is presented with the following observations and proposals.

No table of figures entries found.

Proposal 1	Place the transient period outside of both SBFD and non-SBFD slots.

Proposal 2	RAN4 do not define an architect dependent requirement. Transient period between SBFD and non-SBFD is applicable to all four cases.

Proposal 3	Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

Proposal 4	Define a requirement on RX sub-band blocking similar to the in-band blocking requirement and use existing in-band blocking requirement as baseline.

Proposal 5	New requirement to capture Inter-sector interference shall be adopted.

# References

R4-2418790, Discussion on SBFD general aspects, Ericsson

R4-2418792, Impact on SBFD BS RF TX requirements, Ericsson

R4-2418793, Impact on SBFD BS RF RX requirements, Ericsson

R4-2416584, Way forward for [112bis][306] NR_duplex_evo_General

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

R4-2415677, On potentially new BS RF requirements for SBFD operation, Nokia

R4-2416135, Potentially new SBFD BS RF requirements, Ericsson