3GPP TSG-RAN WG4 Meeting #113	R4-2418794

Orlando, USA, 18th - 22nd November 2024

Title: 	SRS imbalance solution considerations

Agenda Item:	7.1.1.3.4

Source: 	Nokia

Document for:	Approval

# Introduction

In previous RAN4 meeting WF of 6Rx UE requirements was approved. This contribution discusses about of SRS imbalance addressing solutions. Following issues were listed as WF [1].

## Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19

Way Forward: RAN4 to further discuss the following options.

-	Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

-	Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 4-2-1.

## Issue 4-1-2: Initial Considerations for SRS IL imbalance issue

Way forward: RAN4 to focus on the compromised solution proposals in Issue 4-2-1.

## Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue

Way forward: Companies are encouraged to provide comments and indication of support for the solution proposals listed below by RAN4#113. Proponent companies (in parentheses) for the general solution and solution proposals below are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed. Other proposals including expected performance gains and draftCRs are not precluded.

- General candidate solutions

  - Option 1 (Intel):

    - UE performs self-compensation of SRS IL up to its maximum Tx power capabilities

    - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g.

      - Per SRS resource power headroom reporting

      - Configured maximum output power per SRS resource reporting

      - SRS insertion loss value reporting (per SRS resource or per UE)

      - Other

    - Details of SRS IL self-compensation and assistance framework are FFS

  - Other options are not precluded

- Proposals

  - Proposal 1: Only introduce UE compensation and new accurate SRS power headroom reporting for non-CA/DC scenarios can be a compromised solution. Further study the granularity of new SRS power headroom report (China Telecom)

  - Proposal 2: IL imbalance reporting mechanism for SRS AS should include the configured maximum output power per SRS resource, the power headroom per SRS resource and ΔPPowerClass . The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission (Ericsson).

  - Proposal 3: Another way to resolve the SRS IL imbalance reporting issue would be to introduce two types of reporting for a UE: a “baseline” and an “advanced” reporting. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. The advanced reporting could in addition include the information on e.g. the configured maximum output power (Ericsson).

  - Proposal 4: Solution for IL imbalance issue as identified below. Reporting power threshold needs to be considered for static and dynamic reporting. The granularity of reporting could be per band (Spreadtrum).

    - If UE reports statically, report the actual SRS insertion loss with no UE self-compensation

    - If UE reports dynamically, UE report the difference value of each diversity branch output power to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time

  - Proposal 5: Introduce semi-static UE assistance or capability signalling on the information on the UE SRS IL (∆TRxSRS,UE) (Intel).

Adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

  - Proposal 6: Given that specification defines the Tx power should be equally distributed across SRS ports for each SRS transmission, per SRS resource IL reporting would be sufficient (Huawei, HiSilicon).

  - Proposal 7: If dynamic reporting can be considered for SRS IL reporting, network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency (Huawei, HiSilicon).

  - Proposal 8: To keep the current UE implementation untouched and avoid specification impact to most extent, following solution can be considered in Rel-19 (Huawei, Hi Silicon):

    - UE is allowed to indicate whether it enables self-compensation on the SRS IL once the network requests such information, which would benefit the network by adjusting expectation on the antenna switching SRS based PMI estimation.

  - Proposal 9: Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered. (ZTE, Sanechips)

  - Proposal 10: For Rel-19 UE, at least, optionally indicates self-compensation of SRS IL up to its maximum Tx power capabilities.

# Discussion

There have been a lot of discussions about SRS imbalance related to UE HW implementation, antenna interface and its impact on network performance started already Rel-18 both in RAN4 and RAN1 as stated in [2]. SRS imbalance issues enlarge as number of antennas increase. To address the issue there are solutions proposed since Rel-18 that are focusing on UE self-compensation of SRS imbalance or by signalling the imbalance to NW or both. The former can work only when PA has enough maximum linear power headroom on main antenna while providing SRS resources to other antennas. If the value of SRS IL is high, compensation may be partial but this is a feasible, UE based, solution to address imbalance issue. Even though the compensation is partial any amount compensation would improve SNR in gNodeB.

UE is MOP limited in cell edge scenarios but this not likely relevant scenario for MIMO DL reception. On the other hand on cell edge scenarios there are other means than SRS based sounding to estimate CSI which has been discussed other papers.

Figure 1 represents a simplified solution of a SRS AS solution for a one band. Only 1t6r considered here as the methodology described here can be applied to any SRS AS solutions. After the PA there is a switch arrangement for allocating SRS resource to a required antenna. Same arrangement can be used for transmit antenna switching (TAS) for optimizing OTA performance which might be compromised by user interaction with transmit antenna. Dashed line between the switches and antenna ports represents a reference plane in which TX and RX are calibrated in factory. At the same time, it provides a reference plane for channel sounding.

Figure 1 Switching arrangement for SRS resource routing

The idea of channel sounding is to provide gNodeB information about radio channel condition by utilizing UL/DL reciprocity and specific reference signal (SRS) transmitted to each of the DL antennas for gNodeB to acquire DL CSI. UE side antennas are to be included into this estimation.

SRS imbalance occurs due to different losses from PA to each antenna. Different power level of SRS symbols due to UE implementation arriving to gNodeB provide erroneous indication of channel conditions which is considered to have non-negligible impact on the DL throughput.

Up to now the ways to address the SRS imbalance can be grouped into three groups as stated in [1]

- Per SRS resource power headroom reporting

- Configured maximum output power per SRS resource reporting

- SRS insertion loss value reporting (per SRS resource or per UE)

Other options are not precluded.

There are 10 individual proposals listed in WF [1] that are falling into these categories. Following there are some considerations for these.

## SRS resource power headroom reporting

Proposals 1,2,3 in WF [1] are suggesting the utilizing PH reporting to indicate SRS imbalance to gNodeB. [3] proposed deriving SRS imbalance dynamically from PCMAX report for primary antenna and PH report for SRS using type 3 kind of signal with modification to add new MAC-CE and to cover PUCSH transmission and.. In [4] there was a proposal to define accurate power headroom reporting. Both proposals mean changes to signalling.

Following are some other considerations for PH reporting. Currently type 3 PH report is defined in TS 38.213 as;

and PSRS,b,f,c(i,qs,l) in TS 38.213,

where,

-	PCMAX,f,c(i) is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS 38.101-2] and [TS 38.101-3] for carrier f of serving cell c in SRS transmission occasion i

-	PO_SRS,b,f,c(qs) is provided by p0 for active UL BWP b of carrier f of serving cell c and SRS resource set qs provided by SRS-ResourceSet and SRS-ResourceSetId

-	MSRS,b,f,c(i) is a SRS bandwidth expressed in number of resource blocks for SRS transmission occasion i on active UL BWP b of carrier f of serving cell c and μ is a SCS configuration defined in [4, TS 38.211]

-	αSRS,b,f,c(qs) is provided by alpha for active UL BWP b of carrier f of serving cell c and SRS resource set qs

-	PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using RS resource index qd as described in clause 7.1.1 for the active DL BWP of serving cell c and SRS resource set qs [6, TS 38.214]. The RS resource index qd is provided by pathlossReferenceRS associated with the SRS resource set qs and is either an ssb-Index providing a SS/PBCH block index or a csi-RS-Index providing a CSI-RS resource index. If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, a MAC CE [11, TS 38.321] can provide by SRS-PathlossReferenceRS-Id a corresponding RS resource index qd for aperiodic or semi-persistent SRS resource set qs

PCMAX,f,c  is defined in TS 38.101-1 as,

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c

PCMAX_L,f,c=MIN{PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass)– MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c+∆TC,c + ∆TRxSRS,

P-MPRc)}

PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass }

Following observations can be drawn;

Observation 1: PCMAX,f,c is defined for a slot whereas PO_SRS,b,f,c(qs) is defined for a resource set. Extracting SRS IL estimate for x antennas requires x times more time than current channel sounding.

Regarding the first equation even though UE can fairly accurately know the output power at it’s antenna terminal for every transmit occasion PH reporting toward network is carried out implicitly using several parameters.

Observation 2: PH reporting is dependent on two loosely defined parameters PCMAX,f,c and PLb,f,c(qd) The former is very much UE specific with worst case implementation margins included in various parameters. Also tolerance in TS 38.101-1 Table 6.2.4-1 adds more ambiguity to PCMAX,f,c. PL term derived from DL RS is not necessarily associated with an antenna to be sounded.

Observation 3: ∆TRxSRS used in PCMAX,f,c equation is a worst case estimate of possible SRS IL that may happen in an UE and not associated to a specific antenna.

Observation 4: Previous observations indicate great uncertainty on the SRS IL estimation by PH reporting.

## Configured maximum output power per SRS resource reporting

Proposal 5 in WF[1] and in [5] suggest to modify existing equation in TS 38.101-1 for PCMAX,f,c as below,

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) –

MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

In other words replacing ∆TRxSRS by UE specific value ∆TRxSRS,UE. which would be relatively easy to define by a UE vendor.

Observation 5: Reporting ∆TRxSRS,UE would improve the SNR for SRS in gNodeB side but as it is a value used for multiple antennas and yet it would not solve problem on SRS imbalance in high end of the output power range between antennas.

## SRS insertion loss value reporting (per SRS resource or per UE)

Other proposals in WF are related to SRS IL reporting to gNodeB. SRS IL reporting can be divided into two different means: static and dynamic reporting.

SRS IL reporting mean reporting of insertion loss associated to each DL antenna transmitting the SRS resource. This association itself is static and does not change according to UL antenna allocation or RX chain allocation as depicted in table1. UE must report this association for each DL antenna and also the antenna allocation it is using for DL. If static SRS IL reporting is used UE compensation for SRS IL shall not be used. However, UE may compensate frequency response for the SRS transmission so that one value per antenna represents SRS insertion loss accurately.

Observation 6: SRS IL association per antenna is static and is not dependent on transmitter antenna swapping or RX chain allocation.

Observation 7: SRS IL reporting shall be based per antenna.

Observation 8: SRS IL reporting shall include report for DL antenna allocation.

Observation 9: UE may compensate SRS IL frequency response of a band for static SRS IL reporting.

| ## Scenario | ## SRS IL |  |  |  |  |  | ## Assigned UL Antenna |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ## Ant1 | ## Ant2 | ## Ant3 | ## Ant4 | ## Ant5 | ## Ant6 |  |
| ## 1 (6+6 DL) | ## aa | ## bb | ## cc | ## dd | ## ee | ## ff | ## Ant1 |
| ## 2 (6+6 DL) | ## aa | ## bb | ## cc | ## dd | ## ee | ## ff | ## Ant4 |
| ## 3 (4+4 DL) | ## aa |  | ## cc |  | ## ee | ## ff | ## Ant6 |

Table 1 SRS IL and UL antenna association

Dynamic SRS IL compensation relates to the fact that SRS imbalance per antenna may not be constant as a function of output power. RAN1 specification is written in a way that it assumes SRS IL compensation when UL power level ≤  PCMAX_L,f,c Compensation beyond this level may not be possible due power headroom available in the PA. Not utilizing UE capabilities for compensation for SRS IL fully may lead to compromised SNR at gNodeB. If dynamic power reporting utilizing the UE compensation for SRS IL can be considered UE has to report residual IL imbalance above a certain power threshold for each antenna dynamically to gNodeB.

Observation 10: dynamic reporting assumes that UE is compensating the SRS IL based on its capabilities.

Observation 11: For dynamic reporting UE shall report gNodeB output power dependent SRS imbalance per antenna.

# Conclusion

In this contribution, we have made number of observations. Summarizing all the observations we can make following conclusions.

- Depending on the addressing solution changes are required to signalling to cover: PH reporting, static/dynamic SRS IL imbalance per antenna and antenna allocation.

- PH reporting and Pcmax reporting both provide inaccurate SRS IL estimation.

- Static/dynamic SRS IL reporting provide more accurate SRS IL estimation but may require extensive signalling overhead.

Therefore, we articulate our proposal from previous meetings again.

Proposal 1: No changes to RAN4, RAN2 or RAN1 specifications. SRS IL compensation shall be up to UE implementation.

# 4	References

[1] R4-2417095, WF on 6Rx UE requirements, RAN4#112bis

[2] R4-2404935, Views on 6Rx for handheld and FWA UE, RAN4#110bis, Samsung

[3] R4-2415313, On the SRS insertion loss imbalance reporting, RAN4#112bis, Ericsson

[4] R4- 2415266, Further study on SRS IL imbalance, RAN4#112bis, China Telecom

[5] R4-2415686, Views on SRS insertion loss compensation and reporting enhancements, RAN4#112bis, intel Corporation