3GPP TSG RAN WG4 #113													            R4-2418826

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:		7.12.2.2

Source:				Inmarsat, Viasat

Title:	OTA Conformance Testing with Circular Polarisation

Document for:	Discussion

# Background

The intent of this paper is to provide insight into the real-world satellite communication OTA characteristics, and to highlight the need for Circular Polarisation (CP) testing within the OTA specifications.

# Existing Systems

Currently all FR1 satellite systems operate with a satellite antenna using right-hand-circular-polarisation (RHCP) or left-hand-circular-polarisation (LHCP).

This is desirable, since the orientation of the satellite with respect to the UE is not entirely predictable, so using a purely linear satellite antenna would result in some loss. In addition, when radio signals propagate through a satellite communications link, the signal may experience some rotation e.g. Faraday rotation in the ionosphere.

Observation 1: Most current FR1 satellite communications systems use circular polarisation in order to remove the unpredictability of signal phase rotation in the satellite communications link.

Antenna systems operating on one circular polarisation intrinsically reject systems operating on the opposite polarisation. This helps with frequency reuse and co-channel interference, since you can in theory operate two systems on the same frequency, provided that the circular polarisation of the Tx and Rx antennas are of sufficient quality as to provide sufficient rejection of the opposite polarisation. This rejection is characterised by the axial ratio, which is discussed in a later section.

In addition, every time a circularly polarised signal is reflected it reverses the polarisation. E.g. in a multipath scenario if an RHCP signal is reflected, it becomes an LHCP signal. The rejection of opposite polarisations helps make systems which are multipath resilient intrinsically.

Observation 2: Cross-circular-polarisation isolation can be used to provide isolation between satellite systems operating on the same / adjacent frequency bands, and can provide multipath rejection.

Fundamentally, if a perfectly linear UE antenna is used with a perfectly circularly polarised satellite antenna, there will be a 3dB loss experienced at both ends of the link i.e. when at the UE antenna terminals there will be 3dB less power, and at the satellite antenna terminals there will be 3dB less power. This addresses a fundamental issue with the coupling of linear and circular polarisations antenna systems.

Observation 3: A linear antenna can experience up to 3dB degradation in performance when receiving a circularly polarised signal.

## Axial Ratio

Current satellite systems use axial ratio to characterise the circular polarisation quality of the antenna.

Axial ratio is calculated from the following equation:

ARdB= Gco-pol-Gx-pol

Gco-pol being the gain of the antenna when operating at the desired circular polarisation

Gx-pol being the gain of the antenna when operating at the non-desired circular polarisation

When expressed in dB, the lower the Axial Ratio value (preferably negative) the greater the rejection of opposite polarisation signals.

This is called cross polarisation discrimination (XPD) and is calculated using the following equation:

XPD=20log10r+1r-1

r is the axial ratio expressed in linear terms and XPD is returned in dB. This provides the exact rejection an antenna of one circular polarisation scheme would have to the reception of a signal on the opposite scheme.

As can be seen, for an antenna with an linear AR of 0, the XPD would be in theory infinite.

Observation 4: Axial ratio metric is used to assess the rejection of the opposite circular polarisation signals, and XPD can characterise the exact isolation between polarisations.

## Linear Antennas

Linear antennas also reject circularly polarised signals, circularly polarised antennas have two orthogonal components, one at 0 degrees and another at +/-90 degrees depending on whether RHCP / LHCP is used.

As a result, the maximum misalignment a linear polarised antenna can be from these orthogonal components is 45 degrees. This results in a maximum polarisation loss of 3dB:

Pol LossdB=20log10cos⁡(θ)=20log10cos⁡(45)=3dB

Much like a circular polarisation antenna will not be perfectly circular, a linear antenna will not be perfectly linear. This means a loss of up to 3dB can be associated with any linear antenna, depending on the quality of the linear polarisation characteristics of the antenna.

# Test Metrics & Rationale

Mobile handsets traditionally use linear polarised antennas. For TN operation they do not operate in as much of a link constrained environment when compared with NTN. In NTN applications, 0.1dB is usually fought over, let alone 3dB which is the anticipated maximum polarisation loss a linear handset may experience when operating over a circularly polarised satellite.

Observation 5: NTN devices are much more link constrained than TN devices, so taking an ‘up-to 3dB’ loss or gain in performance is significant and should be characterised in order to assess real world performance.

We propose to use circular polarisation test antennas in order to assess the conformance of UEs to the performance metrics specified in the Rel-19 OTA work item for NTN. The rationale for this is two-fold. Consider the following scenarios in which a linear test antenna is used to assess the AUT (antenna/device under test):

- The AUT is linear – the results of the OTA testing of the AUT would yield improvements of up to 3dB when compared to the real world performance of the device when operating over a circularly polarised satellite system

  - In the link constrained satcom environment, we cannot accept a real-world degradation of up to 3dB

  - We must be able to exactly characterise the loss to ensure the device meets performance requirements, so circular polarisation measurements must be taken

- The AUT is circular – the results of the OTA testing of the AUT would yield degradations of up to 3dB when compared to the real world performance of the device when operating over a circularly polarised satellite system

  - The device would effectively have to exceed the OTA requirements by up to 3dB in order to meet the performance metrics derived from the OTA work item

  - In the form factors of mobile handsets and other small devices, this is an impossible ask

  - We must be able to exactly characterise the polarisation alignment to an RHCP/LHCP signal to understand the actual device performance

Proposal 1: Use circular polarisation test antennas to assess the performance of the UE. This enables us to characterise the exact loss seen by the UE when operating in a system that utilises circular polarisation antennas, as all satcom systems do. 3dB is a large margin of error in the mobile handset form factors.

In addition to the above, a linear antenna will not be perfectly linear and a circular polarised antenna will not be perfectly circular, so optimisations can be made to the antenna in the handset to design / orient the antenna in a way which complements the chosen polarisation scheme of the satcom system to reduce the loss as much as possible.

Proposal 2: The circular polarisation testing would enable the antenna designer to assess the real circular polarisation characteristics of linear antennas, which will not be perfectly linear. This may enable the antenna designer to tweak the design slightly to get slight increments in performance for specific polarisations.

It is clear from the above that circular polarisation measurements at both RHCP and LHCP must be characterised in order to assess the ‘up to 3dB’ degradation or exceedance of the requirements since for the form factors of mobile handsets either a 3dB relaxation or tightening of the requirement is a huge margin to give.

It is the view of the operator that the performance metrics should be measured at both RHCP and LHCP circular polarisations to best assess the real-world performance of the antenna system, preventing either the over-specification or relaxation of the devices.

From these performance metric measurements the axial ratio can be inferred, establishing the quality of the antenna when considering the rejection of the undesired polarisations be it from other systems or multipath components. This is a good precedent to set for future FR1 VSAT-like terminals which are to be studied in this work item at a later date. This also futureproofs the use of circular polarisation isolation to provide co-channel rejection, which in theory can double the capacity of a system.

Proposal 3: The specification of FR1 Circular polarisation test procedures sets a good precedent for VSAT like UE which will use circular polarised antennas. This would enable the assessment of cross polarisation discrimination (XPD) which can increase the capacity of a system if leveraged correctly. If we ignore circular polarisation testing at this stage, it may preclude the ability of an operator to leverage XPD to increase the capacity of their system.

## Test Metrics

In addition to testing the overarching metrics such as TRP/TRS for mobile handsets, several other metrics can be introduced that assess the quality of the circular polarisation, and the rejection between polarisations without an increased testing burden, just more post-processing of data.

Observation 6: XPD and AR of the antennas can be assesses without any other additional tests after the main performance metrics (e.g. TRP/TRS) have been established. Only post-processing of data is required.

The introduction of Axial ratio as a performance metric would allow us to assess the quality of circular polarisation of the antenna and would then allow us to calculate the XPD to assess the rejection of the opposite polarisation.

Proposal 4: Introduce AR and XPD as performance metrics for NTN systems, to assess the quality of the antenna’s circular polarisation and the rejection of the opposite polarisation.

# Conclusions & Proposals

Proposal 1: Use circular polarisation test antennas to assess the performance of the UE. This enables us to characterise the exact loss seen by the UE when operating in a system that utilises circular polarisation antennas, as all satcom systems do. 3dB is a large margin of error in the mobile handset form factors.

Proposal 2: The circular polarisation testing would enable the antenna designer to assess the real circular polarisation characteristics of linear antennas, which will not be perfectly linear. This may enable the antenna designer to tweak the design slightly to get slight increments in performance for specific polarisations.

Proposal 3: The specification of FR1 Circular polarisation test procedures sets a good precedent for VSAT like UE which will use circular polarised antennas. This would enable the assessment of cross polarisation discrimination (XPD) which can increase the capacity of a system if leveraged correctly. If we ignore circular polarisation testing at this stage, it may preclude the ability of an operator to leverage XPD to increase the capacity of their system.

Proposal 4: Introduce AR and XPD as performance metrics for NTN systems, to assess the quality of the antenna’s circular polarisation and the rejection of the opposite polarisation.

Observation 1: Most current FR1 satellite communications systems use circular polarisation in order to remove the unpredictability of signal phase rotation in the satellite communications link.

Observation 2: Cross-circular-polarisation isolation can be used to provide isolation between satellite systems operating on the same / adjacent frequency bands, and can provide multipath rejection.

Observation 3: A linear antenna can experience up to 3dB degradation in performance when receiving a circularly polarised signal.

Observation 4: Axial ratio metric is used to assess the rejection of the opposite circular polarisation signals, and XPD can characterise the exact isolation between polarisations.

Observation 5: NTN devices are much more link constrained than TN devices, so taking an ‘up-to 3dB’ loss or gain in performance is significant and should be characterised in order to assess real world performance.

Observation 6: XPD and AR of the antennas can be assesses without any other additional tests after the main performance metrics (e.g. TRP/TRS) have been established. Only post-processing of data is required.