# 3GPP TSG-RAN WG4 Meeting #113                               R4-2418828

# Orlando, USA, 18th-22th Nov, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on REFSENS, ASCS, ACS of LP-WUR

# Agenda Item:   7.23.2.2

# Document for:  Approval

- Introduction

In this contribution, we provided our views and simulation results on REFSENS, ASCS and ACS for LP-WUR.

- Discussion

## 2.1 REFSENS

In last meeting, RAN4 did not achieve agreements on defining single target SNR or separate target SNR for OOK-based receiver and OFDM-based receiver because the performance of these two receivers are different. The value of target SNR is also under discussion. Besides, companies have different opinions on NF and IM. However, all these metrics aim to derive REFSENS for LP-WUR. Therefore, companies are encouraged to provide the values for all of these elements [1].

Issue 2-2-3: REFSENS, NF, IM, SNR aspects for FR1 OOK-based and OFDM-based LR Agreements: - Collect input on REFSENS with values for each element (NF, IM, SNR) in the table next meeting. The value for each element can be discussed. - Encourage companies to also input the justification for each element.

REFSENS for LP-WUR can be expressed as

REFSENS(dBm) = kT0 + 10log10(NRB_WUS*12*SCS) + NF + SNR + IM

where some other related parameters are not included for LP-WUR, such as diversity gain, because compared to the main receiver, 1Rx is used in LP-WUR architecture without the corresponding diversity Rx chain so that there is no need to consider diversity gain.

For SNR, we show the SNR evaluation results for both OOK-based receiver and OFDM-based receiver under different waveform types in Table 1. We can find that OFDM-based receiver has better performance than OOK-based receiver thus it is necessary to differentiate these two receivers. Besides, OOK-4 waveform with M=4 has higher SNR requirement than other waveform, so we also provide the simulation results of OOK-4, M=4 with 3rd and 5th order filters in Fig. 1.

Table 1  SNR evaluation results for LP-WUR

| Receiver Type | Waveform Type | SNR to achieve 1% MDR/BLER |
| --- | --- | --- |
| OOK-based Receiver | OOK-4, M=1 | -6 |
|  | OOK-4, M=2 | -4 |
|  | OOK-4, M=4 | -2 |
| OFDM-based Receiver | OOK-4, M=1 | -10.5 |
|  | OOK-4, M=2 | -8 |
|  | OOK-4, M=4 | -5 |

Fig. 1  SNR evaluation for OOK-based receiver and OFDM-based receiver

We find that filter order had minor impact on LP-WUR performance in REFSENS environment.

Observation 1: OFDM-based receiver has better performance than OOK-based receiver. OOK-4 waveform with M=4 has higher SNR requirement than other waveform.

Observation 2: Filter order has minor impact on LP-WUR performance in REFSENS environment.

Based on the simulation results above, we propose that -2dB SNR is enough for OOK-based receiver and -5dB SNR is enough for OFDM-based receiver.

For NF values, OOK-based receiver and OFDM-based receiver have quite different architectures and we should differentiate NF for OOK-based receiver and OFDM-based receiver. It is the common understanding that the NF for LP is higher than that for MR. We propose +5dB and +2dB on top of MR NF (9dB) for OOK-based receiver and OFDM-based receiver, respectively, based on RAN1’s assumption.

For IM, we agree that we can use legacy value as a start point.

We summarize our results on SNR, IM and NF for REFSENS of OOK-based receiver and OFDM-based receiver based on our simulation results and analysis in Table 2.

Proposal 1: Consider the REFSENS results including SNR, IM and NF for OOK-based LP-WUR and OFDM-based LP-WUR in Table 2.

Table 2  SNR, IM and NF to derive REFSENS of LP-WUR

| Receiver Type | SCS (kHz) | WUS RB | SNR (dB) | IM (dB) | NF (dB) | REFSENS (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| OOK-based Receiver | 15 | 11 | -2 | 2.5 | 14 | -93.5 |
|  | 30 | 11 | -2 | 2.5 | 14 | -96.5 |
| OFDM-based Receiver | 15 | 11 | -5 | 2.5 | 11 | -99.5 |
|  | 30 | 11 | -5 | 2.5 | 11 | -102.5 |

## 2.2 ASCS

The agreements on ASCS in last meeting are listed as follows [1].

Issue 2-3-1: ASCS requirements value Agreements: - Confirm ASCS RF requirements is needed for LP-WUS/WUR. Issue 2-3-2: Required number of guard RB for ASCS Agreements: - RAN4 further discuss required guard RBs for ASCS requirements. Issue 2-3-3: Whether Test case for ASCS should be defined in RAN4 Agreements: - Detailed test case for ASCS should be defined.

According to the WF, we show the simulation results on the guard RB evaluation for ASCS in Fig. 2 and Fig. 3 at 2.6GHz center frequency. In the simulation, adjacent sub-carrier interference (NR signal) has the same PSD as LP-WUS and only OOK-4 M=4 waveform is shown.

Fig. 2  LP-WUR performance in ASCS case with 0 guard RB

Fig. 3  LP-WUR performance in ASCS case with 1 guard RB

Comparing Fig. 2 and Fig. 3, we can find that with 1 guard RB, there is little improvement on the required SNR to achieve 1% BLER for given waveform compared to 0 guard RB. Besides, there is no obvious improvement compared to REFSENS environment in Fig. 1, which means that LP-WUR can address adjacent sub-carrier interference well.

Besides, we also check the LP-WUR performance in ASCS case under different residual frequency error. It is observed that residual frequency error does not has much impact on OOK-based receiver and OFDM-based receiver. Besides, the guard RB can not help deal with residual frequency error in ASCS case.

Observation 3: With 1 guard RB, there is minor improvement on the required SNR to achieve 1% BLER compared to 0 guard RB.

Observation 4: Residual frequency error does not impact on OOK-based receiver and has minor impact on OFDM-based receiver.

Observation 5: Guard RB can not help to deal with residual frequency error in ASCS case.

Proposal 2: It is proposed that no guard RB is needed for ASCS requirements.

To perform ASCS test, no guard RB is needed and NR PDSCH should occupy all the RBs not used for LP-WUS with the same PSD as WUS signal.

Proposal 3: Define the test parameters for ASCS of LP-WUR in Table 3. For these test parameters, the missed detection rate shall be ≤1%.

Table 3  Test parameters for ASCS of LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- |
|  | 11RB |
| LP-WUS Power (dBm) | LR REFSENS |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is set at center of the NR carrier, and 5MHz CBW with 30kHz SCS should be precluded. |  |

## 2.3 ACS

The agreements on ACS for LP-WUR are listed as follows [1].

Issue 2-4-1: Phase noise for simulation assumption Agreements: - Phase noise can be considered in LLS simulation analysis. - Phase noise used in R4-2415201, R4-2415780, and R4-2309204 could be considered as a reference. Other profiles are not precluded. - Number of guard RBs will be further discussed until Feb meeting. Encourage companies to provide simulation results with phase noise modelling. Issue 2-4-4: ACS requirements value Agreements: - FFS. Issue 2-4-5: Required guard RB for ACS requirements Agreements: - RAN4 further discuss the required number of guard RBs for ACS requirements Issue 2-4-7: Test parameters for LP-WUR ACS case Agreements: - Further discuss test parameters for LP-WUS ACS Issue 2-4-8: whether limit ADC bit for ACS requirements Agreements: - No limiting to a specific ADC bit for ACS requirements.

According to WF, we updated our simulation results with phase noise as shown in R4-2415780 [2]. The results are summarized in Table 4 with OOK-4, M=4, OOK-based receiver and 3rd order Butterworth filter. Other simulation assumptions follow the agreements in [3] and [4]. We show the target SNR to achieve 1% MDR/BLER under different interferer levels and with guard RBs if needed.

Table 4  ACS results for LP-WUR under different interferer levels

| EPRE ratio between Interferer and LP-WUS (dB) | Target SNR to achieve 1% MDR/BLER (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Guard RB = 0 | Guard RB = 1 | Guard RB = 2 | Guard RB = 3 | Guard RB = 4 |
| 15 | -1 |  |  |  |  |
| 16 | -1 |  |  |  |  |
| 17 | -1 | This part means no more guard RBs are needed. |  |  |  |
| 18 | -1 |  |  |  |  |
| 19 | 0 |  |  |  |  |
| 20 | 1 |  |  |  |  |
| 21 | 3 |  |  |  |  |
| 22 | N/A | 1 |  |  |  |
| 23 |  | 4 |  |  |  |
| 24 |  | N/A | 2 |  |  |
| 25 |  |  | 7 |  |  |
| 26 |  |  | N/A | 6 |  |
| 27 |  |  |  | N/A | 6 |
| 28 | This part means MDR/BLER will not achieve 1% even under high SNR condition. |  |  |  | N/A |
| 29 |  |  |  |  |  |
| 30 |  |  |  |  |  |
| 31 |  |  |  |  |  |
| 32 |  |  |  |  |  |
| 33 |  |  |  |  |  |

In legacy spec, to perform ACS test, the wanted signal power is set under a good condition with MR REFSENS + 14dB. Thus, we also provide the simulation results under LR REFSENS + 14dB condition for LP-WUR as shown in Fig. 4, which matches the results in Table 4.

Fig. 4  LP-WUR performance in ACS case under LR REFSENS + 14dB

Based on the simulation results in Table 4 and Fig. 4, it can be found that, the maximum interferer level is approximately 27dB higher than LP-WUS power with 4 guard RBs needed. Therefore, we propose to define ACS requirement as 27dB.

Proposal 4: ACS requirement for LP-WUR could be 27dB.

However, for the number of guard RBs, to some degree it depends on implementation, such as filter suppression capability. Besides, it should be noticed that for 5MHz CBW with 30kHz SCS, all the RBs are occupied by LP-WUS and no additional RBs are left for guard RBs. In this case, we suggest not to limit the number of guard RBs as a core requirement and it can be left to implementation.

Observation 6: For 5MHz CBW with 30kHz SCS, all the RBs are occupied by LP-WUS and no additional RBs are left for guard RBs.

Proposal 5: Not to define the number of guard RBs for ACS as a core requirement.

Then, we propose the test parameters in Table 5 and Table 6 as follows, where 1.5dB implementation margin is considered.

Proposal 6: Define the test parameters for ACS of LP-WUR in Table 5 and Table 6. For these test parameters, the missed detection rate shall be ≤1%.

Table 5  Test parameters for ACS of LP-WUR, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 14 dB |
| Pinterferer | dBm | LR REFSENS + [39.5] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + guard RB + 2.5 / -(minimum guard band3 + guard RB + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 6  Test parameters for ACS of LP-WUR, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-31] |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + guard RB + 2.5 / -(minimum guard band3 + guard RB + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

- Conclusion

Observations:

Observation 1: OFDM-based receiver has better performance than OOK-based receiver. OOK-4 waveform with M=4 has higher SNR requirement than other waveform.

Observation 2: Filter order has minor impact on LP-WUR performance in REFSENS environment.

Observation 3: With 1 guard RB, there is minor improvement on the required SNR to achieve 1% BLER compared to 0 guard RB.

Observation 4: Residual frequency error does not impact on OOK-based receiver and has minor impact on OFDM-based receiver.

Observation 5: Guard RB can not help to deal with residual frequency error in ASCS case.

Observation 6: For 5MHz CBW with 30kHz SCS, all the RBs are occupied by LP-WUS and no additional RBs are left for guard RBs.

Proposals:

Proposal 1: Consider the REFSENS results including SNR, IM and NF for OOK-based LP-WUR and OFDM-based LP-WUR in Table 2.

Table 2  SNR, IM and NF to derive REFSENS of LP-WUR

| Receiver Type | SCS (kHz) | WUS RB | SNR (dB) | IM (dB) | NF (dB) | REFSENS (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| OOK-based Receiver | 15 | 11 | -2 | 2.5 | 14 | -93.5 |
|  | 30 | 11 | -2 | 2.5 | 14 | -96.5 |
| OFDM-based Receiver | 15 | 11 | -5 | 2.5 | 11 | -99.5 |
|  | 30 | 11 | -5 | 2.5 | 11 | -102.5 |

Proposal 2: It is proposed that no guard RB is needed for ASCS requirements.

Proposal 3: Define the test parameters for ASCS of LP-WUR in Table 3. For these test parameters, the missed detection rate shall be ≤1%.

Table 3  Test parameters for ASCS of LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- |
|  | 11RB |
| LP-WUS Power (dBm) | LR REFSENS |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is set at center of the NR carrier, and 5MHz CBW with 30kHz SCS should be precluded. |  |

Proposal 4: ACS requirement for LP-WUR could be 27dB.

Proposal 5: Not to define the number of guard RBs for ACS as a core requirement.

Proposal 6: Define the test parameters for ACS of LP-WUR in Table 5 and Table 6. For these test parameters, the missed detection rate shall be ≤1%.

Table 5  Test parameters for ACS of LP-WUR, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 14 dB |
| Pinterferer | dBm | LR REFSENS + [39.5] dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + guard RB + 2.5 / -(minimum guard band3 + guard RB + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 6  Test parameters for ACS of LP-WUR, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | [-31] |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + guard RB + 2.5 / -(minimum guard band3 + guard RB + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

- Reference

- R4-2417112 WF on UE RF requirements for LP-WUS, RAN4#112bis, Hefei, China, Oct 2024.

- R4-2415780 Further discussions on LP-WUS REFSENS, ASCS and ACS, RAN4#112bis, Hefei, China, Oct 2024.

- R4-2414309 WF for LP-WUS UE RF, RAN4#112, Maastricht, Netherlands, Aug. 2024.

- R4-2410569 WF for [111][136] NR_LPWUS_UERF, RAN4#111, Fukuoka, Japan, May 2024.