3GPP TSG-RAN WG4 Meeting # 113	R4-2418838

# Orlando, USA, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on simulation assumptions and results for LP-WUS/WUR

Agenda item:	7.23.4.1

Document for:	Discussion

Introduction

In this contribution, we provide our initial simulation results for NR LP-WUS/WUR based on listed simulation assumptions in approved WF [1].

Discussion

- Topic #2: Simulation assumptions and results

Sub-topic 2-2 Simulation assumptions and results summary

Simulation assumptions

In the last meeting, RAN4 agreed as below:

Note: Parameters highlighted by green color are agreed during the meeting. Other parameters are only for recommendation and information purpose. Table 1: General parameters Simulation parameters; Comments/values. Carrier frequency for Cell 1 and Cell 2; Agreement: 2.6 GHz initially. Prior knowledge of Cell 1 / Cell 2 by the UE; Interfering cell (Cell 2) is not known to UE. DRX; No applicable for LP-WUR. BS transmit antennas for LP-SS blocks; 1 Tx. UE receive antennas; 1 Rx. Data and control channel subcarrier spacing; [Data, SSB and LP-SS have the same SCS]. Subcarrier spacing; 30KHz initially TBD for 15 KHz. Measurement period (in number of measurement samples); [4, 5, other number could be studied upon a need]. LP-SS/SSB measurement interval; LP-SS: 320 ms SSB: 320 ms. LP-SS BW; 132 subcarriers for SCS=30kHz for LP-SS initially TBD for 15KHz SCS. SSS; 30KHz for SSS, TBD for 15KHz. Actual LP-SS transmissions; always transmitted. Guard band; 1 RB on each side of LP-SS/LP-WUS signal. Table 2: Cell-specific parameters Parameter; Unit; Cell 1; Cell 2. RF Channel number; -; Channel 1; Channel 1. NR-PSS, NR-SSS (OFDM based LP-WUR); -; To be indicated by companies; To be indicated by companies. LP-SS; OOK-1; or OOK-4 with M = [2,4] Note: M value [2 4] are up to company selection; when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2. LP-SS pattern; [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them; [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them. PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS; dB; 0; 0. Data and control PSD relative to NR-PSS,NR-SSS and LP-SS; dB; 0; 0. RB Utilization; %; 100; 100. Data Modulation; -; QPSK; QPSK. Slot length; -; 14 symbols; 14 symbols. CP Length; -; Normal; Normal. Frequency offset relative to UE frequency reference; Hz; OFDM based receiver [5] ppm OOK based receiver [ 5 10] ppm; N/A. Timing error; Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: - T= 5us for OOK-1 - T= 2us for OOK-4 with M=2 - T= 1us for OOK-4 with M=4. 1)Relative Delay of 1st Path (synchronous); µs; 0; CP/2. 2) Relative Delay of 1st Path (asynchronous): Fixed delay; ms; 0; 3 ms. SNR; dB; SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1. When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4]; When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4]. When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1; When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1. When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1; When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1. Ês/Iot; dB; -3; -0.5dB; 2dB; N/A. Propagation conditions; -; FR1: AWGN TDL-C 300ns. UE speed; 3 km/h. Table 3: UE-specific parameters [Receiver Filter]; [3th/5th Order Butterworth with 3.96MHz bandwidth]. [Receiver ADC bit width]; [4 or 8-bitADC]. [Receiver Sampling Rate for LP-SS only]; [3.84 or 7.68MHz].

Based on the additional simulation assumption agreements, we tried to get additioanl simulation results. In this document, we share our simulation model and simulation results based on additional simulation assumptions like time error and frequency offset.

Timing offset

| Parameters | Assumption |
| --- | --- |
| SCS | 30 kHz |
| LP-SS BW | 5 MHz (132 subcarriers) |
| FFT size | 256 |
| Symbol duration | 36 us |
| Sample duration | 0.14 us |
| CP size | 22 samples |
| Timing offset | 1 us: 7 samples |
|  | 2 us: 14 samples |
|  | 5 us: 35 samples |

Based on the parameter, for TO greater than 3us, the symbol start point is in the neighbering symbol.

Simulation model

Basic elements of the system model

- Waveform generation (OOK-1, OOK-4(M-2, 4), SL(P/S-SSS))

- Digital-to-analog (D/A) converter

- TX/RX LPF: Butterworth filter

- RF modulation/demodulation signal

- RSRP measurement: measured by the maximum response of the Swiveled correlator

  - is the mean value of the received power in the linear scale under the assumption of no noise and perfect channel knowledge

  - is the measured mean value

Simulation assumption

Baseline assumption

| Parameters | Assumption |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| LP-SS BW | 5 MHz (132 subcarriers) |
| Waveforms | OOK-1, OOK-4 (M=2,4),SL |
| SCS | 30 kHz |
| Measurement period (in number of measurement samples) | 1-5, 10, 20 |
| LP-SS measurement interval | 320 ms |
| Filter | Butterworth (5 Order) |
| ADC | 4 bits |
| Channel propagation | AWGN, TDLC300 |
| CFO | 0, 5, 10, 20 ppm |
| Time offset | 0-5 us |
| Mobility | 3 km/h |
| Antenna configuration | 1x1 |

Cell parameters

- We considered NR types of interference (same bandwidth with the useful signal)

SINR and SNR configurations

- Three SINR values are considered, for each three SNR values for different interference power levels

Simulation results

| LR receiver | Time offset | Channel propagation | SINR | Required number of samples |
| --- | --- | --- | --- | --- |
| OOK-1 | 5 us | AWGN | -3 dB | Target accuracy 2.5 dB : 1 |
|  |  | TDLC300 | -3 dB | Target accuracy 2.5 dB : 3 Target accuracy 3.5 dB : 2 |
| OOK-4 (M=2) | 2 us | AWGN | -3 dB | Target accuracy 2.5 dB : 1 |
|  |  | TDLC300 | -3 dB | Target accuracy 2.5 dB : 3 Target accuracy 3.5 dB : 2 |
| OOK-4 (M=4) | 1 us | AWGN | -3 dB | Target accuracy 2.5 dB : 1 |
|  |  | TDLC300 | -3 dB | Target accuracy 2.5 dB : 3 Target accuracy 3.5 dB : 2 |
| OFDM-based | 1 us | AWGN | -3 dB | Target accuracy 2.5 dB : 1 |
|  |  | TDLC300 | -3 dB | Target accuracy 2.5 dB : 3 Target accuracy 3.5 dB : 2 |

Observation 1: There is no big difference according to the time error from 1 us to 5 us.

Observation 2: In AWGN channel propagation, only 1 sample can meet the accuracy requirements at every scenario.

Observation 3: In TDLC300 channel propagation, 3 samples are necessary to meet the 2.5dB accuracy requirements and 2 samples are necessary to meet the 3.5dB accuracy requirements.

Conclusion

In this contribution, we provide views on issues listed in WF [1], and we propose

Observation 1: There is no big difference according to the time error from 1 us to 5 us.

Observation 2: In AWGN channel propagation, only 1 sample can meet the accuracy requirements at every scenario.

Observation 3: In TDLC300 channel propagation, 3 samples are necessary to meet the 2.5dB accuracy requirements and 2 samples are necessary to meet the 3.5dB accuracy requirements.

Reference

[1] R4-2413899, “WF for RRM requirements for LP-WUSWUR,” vivo