3GPP TSG RAN WG4 #113													            R4-2418840

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:		7.8.2

Source:				Inmarsat, Viasat

Title:	HPUE – Methods of reducing SAR without P-MPR

Document for:	Discussion

# Background

The aim of this paper is to share an operator’s view on the viability of some of the SAR management techniques, highlighting why that for NTN handsets P-MPR is unsuitable, and that other measures are already in place passively, or new measures can be taken.

# P-MPR Problem

P-MPR is the process by which a UE reduces its output power, when it is in danger of exceeding the regulatory SAR limits imposed by the relevant regulator. It has been proposed at previous meetings that the SAR can be reduced by two distinct methodologies:

- P-MPR – UE reduces its power in order to reduce the absolute magnitude of the radio emissions causing a heating effect in the users body

- Scheduling based SAR reduction – either the network or the UE begins allocating data for uplink more sporadically, meaning that the magnitude of radio emissions integrated over time is reduced when the SAR limit is close to being reached.

Observation 1: There are two distinct methods for SAR management – P-MPR power backoff and scheduling based SAR reduction which can be either Network or UE instigated.

The satellite communications environment is inherently link-budget constrained, and UEs, particularly mobile handsets, will be operating at the bottom end of the SNR limits for either NR or NB-IoT physical layer. A reduction in transmit power through P-MPR could kill the link altogether, taking the SNR below the limit of the physical layer, or having to use a number of repetitions which makes the user experience significantly degraded.

In addition, the use of P-MPR would mean that the benefits of HPUE are not capitalised upon. If it turns out that the SAR of the UE is exceeded quite quickly, when for example a video call is made, the UE will have to operate at a degraded transmit power for a significant amount of operation time, effectively bringing it down a power class.

Observation 2: The satcom environment is link constrained and using power backoff methods would be unsuitable for NTN as a result, and the benefits of HPUE may be lost.

# Scheduling Based SAR Management

At the previous meetings, the use of scheduling based SAR management has been proposed. This comes in two forms:

- Network instigated SAR management – When the network detects a HPUE capable device has been transmitting at a specific duty cycle for sufficient time, it will begin to reduce the effective duty cycle of the uplink transmissions.

- UE instigated SAR management – the UE has an awareness of its effective UL duty cycle, and a knowledge of the specific devices SAR profiles. The UE can skip uplink slots if it needs to reduce the SAR levels for a period of time.

The problem with the network instigated SAR management technique is that the network does not have an awareness of every UEs exact SAR profile, or UE orientation, and hence may revoke uplink scheduling slots from a UE which has not practically exceeded its own SAR profile.

Observation 3: The network based SAR management method may lead to pre-emptively reducing UL allocation slots since the Network has no awareness of specific UE SAR profiles

The UE instigated SAR management is much more preferable. Each UE has to have an awareness of its own SAR emissions, and what the orientation of the UE is. E.g. if it is being held to the users’ head the SAR limit will be reached sooner. With this information, and knowledge of the UE’s own UL schedule, the UE can skip allocated UL slots if the SAR limit is neared. This provides a more efficient SAR management strategy which considers each UE individually, and accepts different emissions profiles.

Proposal 1: UE based SAR management method is preferable since the UE can manage its own uplink allocations based on knowledge of its own SAR emissions profiles and UL schedule. This makes the process much more dynamic and on a UE-by-UE basis.

That being said, it is not anticipated that the UE will constantly be transmitting in an NTN environment and will have a low effective duty cycle in the uplink. For NR the UE is expected to not use more than 6RBs for uplink transmission, and for the foreseen NR use cases this is unlikely to require 100% uplink allocation.

E.g. even for a single RB at the lowest MCS, the data rate is around 27kbps.

Observation 4: NTN UL data rates are not likely to require a high duty cycle.

# Regulatory Issue

The issue with the above proposal is that we cannot simply tell the regulatory that we don’t think that the UE will ever be scheduled to transmit at a duty cycle which exceeds the SAR. Since the UE technically could exceed the SAR limit if the uplink schedule was sufficiently busy. As a result, a method of SAR management does have to be implemented to sate the regulators.

Proposal 2: To say that the UE UL scheduling is ‘unlikely to cause SAR issues’ is insufficient and a method of SAR management needs to be demonstrated.

# Justification

The case that the UE will be using the highest uplink duty cycle will be in the edge of cell cases. The SNR of the downlink is the lowest here, and the UE will be utilising a low MCS (high code rate with lots of redundancy bits) in order to compensate for this. In order to meet the data rates of the desired application, the UE will have to have more consecutive uplink slots allocated to it, meaning the UE duty cycle would be higher.

In this case, the UE is compensating for the poor SNR by decreasing the MCS and thus improving the link budget, at the cost of an increased UL duty cycle.

If we used P-MPR as a SAR management technique, since the SNR is already low in this EoC case, an additional power back-off may kill the link budget since the UE is already operating at the lowest MCS. As a result, P-MPR for NTN applications is unfeasible.

If we used a network-based SAR scheduling method, the UE would be given fewer uplink slots to reduce the SAR if the network detects it is a HPUE and has a duty cycle sufficient to warrant a back-off. However, the network may pre-emptively do this since it doesn’t know the exact SAR characteristics of the UE, so may reduce the throughput of the network without needing to.

If we used a UE based SAR scheduling method, the UE can skip UL slots as it pleases, which reduces the UL capacity of the system. This however, is the preferred method since the UE has an individual awareness of its own SAR emissions, the orientation of the UE and its duty cycle, so can dynamically change the UL duty cycle to compensate this only when absolutely necessary on a UE-by-UE basis.

Proposal 3: P-MPR is unsuitable for NTN applications since it may kill the link budget. Network based SAR scheduling management is less suitable because it does not consider the emissions of individual UEs and may pre-emptively reduce the capacity of the system. UE based SAR techniques is the best approach, it considers the individual SAR emissions of each UE and only backs off the duty cycle when absolutely necessary.

# Observations & Proposals

Observation 1: There are two distinct methods for SAR management – P-MPR power backoff and scheduling based SAR reduction which can be either Network or UE instigated.

Observation 2: The satcom environment is link constrained and using power backoff methods would be unsuitable for NTN as a result, and the benefits of HPUE may be lost.

Observation 3: The network based SAR management method may lead to pre-emptively reducing UL allocation slots since the Network has no awareness of specific UE SAR profiles

Observation 4: NTN UL data rates are not likely to require a high duty cycle.

Proposal 1: UE based SAR management method is preferable since the UE can manage its own uplink allocations based on knowledge of its own SAR emissions profiles and UL schedule. This makes the process much more dynamic and on a UE-by-UE basis.

Proposal 2: To say that the UE UL scheduling is ‘unlikely to cause SAR issues’ is insufficient and a method of SAR management needs to be demonstrated.

Proposal 3: P-MPR is unsuitable for NTN applications since it may kill the link budget. Network based SAR scheduling management is less suitable because it does not consider the emissions of individual UEs and may pre-emptively reduce the capacity of the system. UE based SAR techniques is the best approach, it considers the individual SAR emissions of each UE and only backs off the duty cycle when absolutely necessary.