# 3GPP TSG-RAN WG4 Meeting #113		  R4-2418852

# Orlando, United States, 18th - 22nd November, 2024

Agenda item:			7.21.2.2

Source:	Qualcomm Incorporated

Title:	Discussion on Ambient IoT Coexistence Evaluations

Document for:	Approval

# 1  Introduction

In RAN4 #112-bis meeting, the agreements on ambient IoT coexistence study were marked in [1], and the evaluation conclusion was summarized as below.

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor -> Victim) | Tentative Co-existence conclusion |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | b | NR DL->reader | TBA |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | b | NR DL->reader | TBA |
|  |  |  | c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | f | NR UL->reader | TBA |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |
|  |  |  | f | NR UL->reader | TBA |
|  |  |  | c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range level FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | TBA |
|  |  |  | b | NR DL->reader | TBA |
|  |  |  | g | Reader->NR UL | TBA |
|  |  |  | h | NR UL->device | TBA |
| 4 (O) | D1T1-B-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | TBA |
|  |  |  | f | NR UL->reader | TBA |
|  |  |  | g | Reader->NR UL | TBA |
|  |  |  | h | NR UL->device | TBA |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |
|  |  |  | g | Reader->NR UL | TBA |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | f | NR UL->reader | TBA |
|  |  |  | g | Reader->NR UL | FFS: NR UL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range Interference coordination/mitigation solution can be considered. |
|  |  |  | h | NR UL->device | TBA |
| 6-1(O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | TBA |
|  |  |  | b | NR DL->reader | TBA |
|  |  |  | g | Reader->NR UL | TBA |
|  |  |  | h | NR UL->device | TBA |
| 6-2(O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | TBA |
|  |  |  | b | NR DL->reader | TBA |
|  |  |  | g | Reader->NR UL | TBA |
|  |  |  | h | NR UL->device | TBA |

This paper provides our considerations on the issues that identified and our proposals on Ambient IoT coexistence study.

# 2  Discussion for co-existence evaluation

## 2.1 Legacy NR UE indoor scenario for D1T1

For D1T1 with legacy UE indoor scenario, R2D on DL spectrum, NR DL throughput degradation for both average throughput and cell edge throughput is observed. To avoid the severe performance degradation for NR DL, the case 3 for deployment scenario and topology was proposed and evaluated. Based on our simulation results, the following observations are made.

Table 1: Coexistence evaluation results

| deployment scenario and topology | spectrum |  | Evaluation case No. | (Aggressor -> Victim) | Tentative Co-existence conclusion |
| --- | --- | --- | --- | --- | --- |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |
|  |  |  | b | NR DL->reader | SINR degradation @5-ile is 2dB, 50-ile is 0.8dB with 140dB CW cancellation capability |
|  |  |  | g | Reader->NR UL | 8.73% mean T-put loss, cell edge T-put is N/A due to outage |
|  |  |  | h | NR UL->device | 1.78dB for 5-ile SINR degradation, 1.22dB for 50-ile SINR degradation |

Observation 1: For D1T1-A2-leagcy UE indoor scenario, transmitting R2D link in UL spectrum can reduce the interference between TN and AIoT systems.

Proposal 1: Take co-existence conclusion listed in the above table for case 3 (O).

## 2.2 Reader interfering to NR UL for D2T2 co-located scenario

For the case 5-1-g, D2T2-A2-legacy UE only outdoor, there was no conclusion for the scenario reader interfering to the NR UL. The current Tx power assumptions for reader in D2T2 are configuring maximum transmit power, i.e., 23dBm. The interference from reader to NR UE UL in D2T2 can not be ignored even NR UE is outdoor. This is due to the high transmission power of the reader, the lower propagation loss in the low frequency band and high leakage from reader to NR UL.

Observation 2: The interference from reader to NR UE UL in D2T2 is not ignored even NR UE is outdoor. This is due to the high transmission power of the reader, the lower propagation loss in the low frequency band and high leakage from reader to NR UL.

Figure 1: IBE model for NR UE and intermediate UE

We evaluated the following 3 sets of ACLR assumptions and have the following results:

- Set 1: If ACLR = [18, 18, 30], mean throughput loss 47.9%, edge throughput loss 72.2%;

- Set 2: If ACLR = [23, 23, 30], mean throughput loss 30.1%, edge throughput loss 37.6%;

- Set 3: If ACLR = [30, 30, 30], mean throughput loss 12.4%, edge throughput loss 6.6%;

We can observe that with 0RB guard band (Set 1) and 2RBs guard band (Set 2), the performance degradation for NR UL is very high. Even with 4RBs or more as the guard band, the performance degradation is still larger than the coexistence criteria (less than 5%).

Observation 3: With legacy IBE model, guard band is not very helpful in reducing interference from reader to NR UL.

Another potential solution is to reduce the Tx power of reader. Figure 2 and Figure 3 show the SINR CDF of R2D and D2R with different Tx power configurations for reader/intermediate UE, respectively.

Figure 2: SINR CDF for R2D in D2T2 with 23dBm, 18dBm and 15dBm Tx power for reader

Figure 3: SINR CDF for D2R in D2T2 with 23dBm, 18dBm and 15dBm Tx power for reader

Compared to 23dBm Tx power, the performance for D2R and R2D has marginal degradation with 18dBm and 15dBm Tx power for reader at SINR range of less than 20dB.

Observation 4: D2T2 is an interference-limited scenario. Decreasing the Tx power for reader/intermediate UE has marginal impact on the AIoT system.

With lower Tx power for reader, the coexistence performance is enhanced and there is no issue for reader interfering NR UL.

- Reader Tx power 18dBm, mean throughput loss 5.91%, edge throughput loss 2.62%

- Reader Tx power 15dBm, mean throughput loss 3.47%, edge throughput loss 0.66%

Proposal 2: RAN4 consider decreasing the Tx power by power control for reader in D2T2 to solve the coexistence issue for reader interfering NR UL.

## 2.2 NR DL to D2R for D2T2 CW outside scenario

In [1], the phase noise model was agreed to be considered for CW outside.

Agreement: - For -B and -A1, consider following baseline SINR: - Option 1: baseline SINR for CW outside=received wanted signal powernoise + intrasysteminterferencewithin total receiver bandwidth+residual CW interference after cancellationin linear scale - Option 2: baseline SINR for CW outside=received wanted signal powernoise + intrasysteminterference+phase noisewithin total receiver bandwidth+residual CW interference after cancellationin linear scale Note: If option2 is used, companies needs to report phase noise modelling.

We ran the simulations to analyze the impact to SINR due to multiple phase noise values. Figure 4 shows the results with the assumptions: No phase noise, phase noise models are -132dBc/Hz, -122dBc/Hz and -112dBc/Hz.

Figure 4: Impact of phase noise to SINR

Note that we chose 15 kHz D2R bandwidth and assumed it is offset from the CW sufficiently to avoid increase in integrated noise because of high flicker noise close to the CW. In that way, the simulated results show most optimised case. If the bandwidth is larger, the integrated noise is also larger.

Observation 5: Phase noise has non-negligible impact to the D2R SINR.

Proposal 3: RAN4 should specify phase noise requirements for the component which provides the phase noise to the system.

# 3  Conclusion

In summary, we have following observations and proposals.

Observation 1: For D1T1-A2-leagcy UE indoor scenario, transmitting R2D link in UL spectrum can reduce the interference between TN and AIoT systems.

Observation 2: The interference from reader to NR UE UL in D2T2 is not ignored even NR UE is outdoor. This is due to the high transmission power of the reader, the lower propagation loss in the low frequency band and high leakage from reader to NR UL.

Observation 3: With legacy IBE model, guard band is not very helpful in reducing interference from reader to NR UL.

Observation 4: D2T2 is an interference-limited scenario. Decreasing the Tx power for reader/intermediate UE has marginal impact on the AIoT system.

Observation 5: Phase noise has non-negligible impact to the D2R SINR.

Proposal 1: Take co-existence conclusion listed in the above table for case 3 (O).

Proposal 2: RAN4 consider decreasing the Tx power by power control for reader in D2T2 to solve the coexistence issue for reader interfering NR UL.

Proposal 3: RAN4 should specify phase noise requirements for the component which provides the phase noise to the system.

# 4  References

[1] R4-2417087, WF on co-existence evaluation for ambient IoT