3GPP TSG-RAN WG5 Meeting #105							        	R4-2418861

Orlando, US, November 18-22, 2024

Title:	RRM Core requirements on conditional intra-CU LTM

Source:	China Telecom

Agenda item:			6.24.2.2

Document for:	Discussion

- Introduction

In RAN2#127bis meeting, some conclusions on conditional intra-CU LTM have been duplicated as follows [1]. From this meeting, we will start to discuss the RRM Core requirements on conditional intra-CU LTM.

1. Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. 2. Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. 3. Source cell and each candidate cell provides its own execution condition for conditional LTM. 4. It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. 5. RACH-less Conditional intra-CU LTM is supported. 6. RACH based conditional intra-CU LTM is supported. 7. UE based TA measurement mechanism is supported for conditional intra-CU LTM. 8. PDCCH ordered early TA acquisition is supported for conditional LTM. 9. Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. 10. For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. 11. The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

- Discussion

Issue 1-1-1: RAN4 scope of Conditional LTM

< Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.

In RAN#105 meeting, the supporting of conditional intra-CU LTM RAN has been introduced. In RAN4#112bis meeting, we have achieved the agreement that wait for other WGs input to further discuss detailed RRM requirements. And in RAN2#127bis meeting, some conclusions have been achieved as follows.

7. UE based TA measurement mechanism is supported for conditional intra-CU LTM. 8. PDCCH ordered early TA acquisition is supported for conditional LTM. 9. Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

So, in addition the cell switch delay requirements need to be defined in RAN4, requirements on early UL sync, i.e., UE based TA measurement and PDCCH ordered early TA acquisition for conditional intra-CU LTM also need to be defined. What is more, requirements on early candidate TCI State activation/deactivation for conditional intra-CU LTM is necessary.

Proposal 1: RAN4 needs to define requirements on cell switch delay, UE based TA measurement and PDCCH ordered early TA acquisition and early candidate TCI State activation/deactivation for conditional intra-CU LTM.

Issue 1-1-3: Measurement requirements on conditional LTM

< Agreement > - On whether L3 measurement based conditional LTM in FR1 can be supported - RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached. - As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified.

For this issue, RAN2 does not exclude the case where L3 measurement is used to generate the execution condition, but the detail needs further discussion. So, we think we could discuss L1 measurement requirements on conditional LTM firstly and wait the conclusion on the case where L3 measurement is used.

Proposal 2: Discuss L1 measurement requirements on conditional LTM first and wait for the conclusion on the case where L3 measurement is used.

Issue 1-1-4: Subsequent conditional LTM

< Way Forward > FFS the following proposals: - Proposal 1: FFS whether there is any difference in the delay for initial and subsequent CLTM cell switch. - Proposal 2: RAN4 should further clarify the scenario for conditional LTM including subsequent LTM. - Proposal 3: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell. - Proposal 4: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

In the last meeting, we have agreed to defined requirements on subsequent conditional LTM. But with the different procedure between the initial conditional LTM and subsequent conditional LTM, the requirements are different too. Since no new RRCReconfiguration message on conditional LTM before subsequent conditional LTM, the starting point of initial conditional LTM is not suitable. But the skeleton is similar, we can discuss requirements on initial conditional LTM firstly and take it as baseline.

Proposal 3: The starting point of initial conditional LTM and subsequent conditional LTM is different. And the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Issue 1-1-5: The specification structure of the requirements for conditional LTM

< Way Forward > FFS the following options: - Option 1: For specification structure, the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM. - Other option is not precluded.

Conditional LTM is the extension of LTM, according to that CHO is defined in the clauses of Handover, the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM.

Proposal 4: The requirements for conditional LTM can be defined in the clauses of Rel-18 LTM.

Conditional Intra-CU LTM delay

Firstly, we should define the starting point and ending point of conditional intra-CU LTM delay. As a reference, the definitions the starting point and ending point of LTM delay and CHO delay are copied as follows

CHO: When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. LTM: LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

So, we think the starting point of conditional intra-CU LTM delay is When the UE receives a RRC message implying conditional intra-CU LTM. If we want to align with the definition of LTM delay, we can treat it as the starting point of conditional intra-CU LTM delay is the end of the last TTI containing the RRC command for conditional intra-CU LTM.

Since RAN2 has agreed that the LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM, the ending point of conditional intra-CU LTM delay is the time the UE transmits the first UL message on the target cell.

Proposal 5: Conditional intra-CU LTM delay is the delay from the end of the last TTI containing the RRC command for conditional intra-CU LTM until the time the UE transmits the first UL message on the target cell.

Base on the definition of the conditional intra-CU LTM delay as shown above. The components of Conditional intra-CU LTM delay can contain these as follows

DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +Tearly_process.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- Tmeasure is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

- TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM, and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmits the first UL message on the target cell.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

- Tearly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further.

Proposal 6:

DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts.

- TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

- TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Due to this is the first meeting on conditional intra-CU LTM in all RAN WGs. We should revisit this issue when other WGs achieve further conclusions.

Proposal 7：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions.

DL-sync, UL-sync and Early TCI state activate

RAN2 has achieved some conclusions on Early-sync and Early TCI state activate, we duplicate them as follows:

7. UE based TA measurement mechanism is supported for conditional intra-CU LTM. 8. PDCCH ordered early TA acquisition is supported for conditional LTM. 9. Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

For early TCI state activate and UL-sync, we think R18 LTM early TCI state activate and UL-sync can be as baseline.

Proposal 8:

- Discuss the requirements on early candidate TCI State activation/deactivation and early UL-sync.

- R18 LTM early TCI state activate and PDCCH ordered RACH could be baseline.

Whether to cover both RACH-based and RACH-less conditional intra-CU LTM

As the previous issue saying, we think UE can acquire TA without RAR-based TA acquisition. So, RACH-less conditional intra-CU LTM is important. We should cover both RACH-based and RACH-less conditional intra-CU LTM. What is more, RAN2 has agreed to introduce both RACH-based and RACH-less.

Proposal 9: Cover both RACH-based and RACH-less conditional intra-CU LTM.

- Summary

In this paper, we provide our views on conditional intra-CU LTM. From this discussion we have derived the following proposals:

Proposal 1: RAN4 needs to define requirements on cell switch delay, UE based TA measurement and PDCCH ordered early TA acquisition and early candidate TCI State activation/deactivation for conditional intra-CU LTM.

Proposal 2: Discuss L1 measurement requirements on conditional LTM first and wait for the conclusion on the case where L3 measurement is used.

Proposal 3: The starting point of initial conditional LTM and subsequent conditional LTM is different. And the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Proposal 4: The requirements for conditional LTM can be defined in the clauses of Rel-18 LTM.

Proposal 5: Conditional intra-CU LTM delay is the delay from the end of the last TTI containing the RRC command for conditional intra-CU LTM until the time the UE transmits the first UL message on the target cell.

Proposal 6:

DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts.

- TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

- TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Proposal 7：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions.

Proposal 8:

- Discuss the requirements on early candidate TCI State activation/deactivation and early UL-sync.

- R18 LTM early TCI state activate and PDCCH ordered RACH could be baseline.

Proposal 9: Cover both RACH-based and RACH-less conditional intra-CU LTM.

- References

[1] R2-2409211, Report from session on V2X/SL, R18/19 MOB, and R19 NES, Samsung.