3GPP TSG-RAN WG4 Meeting #113	R4-2418890

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	XR simulation results

Agenda item:	7.25.2

Document for:	Discussion

Introduction

In this paper we present simulation results of the policies discussed in RAN1, including comparison of dynamic and semi-static approaches, as well as comparison of different time offsets.

Results highlights

- Semi static (Alt 3) approach achieved only 25% capacity gain in comparison with baseline using measurement gaps.

- DCI-based approach (Alt 1-1) achieved from 50 to 75% capacity gain (from 4 user per cell to 6 or 7 XR users per cell) depending on the time offset between DCI and measurement gap.

- The smaller the time offset between the DCI and start of the measurement gap is, the better the skipping performance is, with less skipping being necessary to achieve a good capacity gain.

System level simulation results

## Evaluation assumptions

In this subsection, we present the results of system-level simulations (SLS) of the skipping policies discussed in RAN1. Simulations have been performed in the Dense Urban FR1 scenario with parameters configured according to TR38.835. On top of the parameters defined in TR38.835, we configured measurement gaps considering two configurations: (40,6), where the interval is 40ms and the gap length is 6ms. In our evaluation, we consider the following KPIs:

XR capacity: system load CP of policy P in terms of number of UEs/cell (N) when the ratio of satisfied UEs/cell RP is above or equal to 90% (CP=maxN:RP≥90%. A UE is marked as satisfied if 99% of XR frames are delivered within the packet delay budget (PDB) of 10ms (for VR/AR) or 15ms (for CG). The KPI is reported by computing the average across all UEs, all cells, and all SLS drops.

Skipping ratio: fraction of skipped measurement opportunities with respect to the total number of measurement opportunities defined by the measurement gaps configuration. The KPI is reported by computing the average across all UEs, all cells, and all SLS drops.

XR capacity loss: loss of XR capacity measured considering the system load when MG is disabled.

XR capacity gain: gain of XR capacity measured considering the system load when MG and skipping policies are enabled.

We evaluate the following skipping policies (alternatives):

Explicit DCI indication (Alt. 1-1): the logic of the skipping decision is implemented by the network and the skipping decision is dynamically delivered with dedicated DCI signaling to the UE.

Pattern-based (Alt 3): the network communicates the pattern of the XR traffic to the UE e.g. via RRC signalling. Both the UE and the network determine whether to skip using both the pattern of XR traffic and the pattern of measurement occasions communicated by the network.

Additionally, we compare the skipping policies against the two following baseline configurations:

No MGs: measurement gaps are disabled. This corresponds to the ideal case where scheduling restrictions are not delaying scheduling decisions. This configuration provides an upper bound in terms of XR capacity.

MG enabled: measurement gaps and corresponding scheduling restrictions are enabled. This configuration provides a lower bound in terms of XR capacity.

Genie: scheduling restrictions are skipped every time skipping is needed to deliver a XR frame. This policy assumes perfect knowledge of traffic and channel status and skipping is triggered even when a transmission or retransmission happens during a measurement occasion. The skipping decision is delivered to the UE using Alt-1-1. While being ideal (traffic might arrives when measurement occasion has already started and prediction missed this case), this policy provides an upper bound in terms of skipping ratio.

## Skipping policy based on explicit DCI indication (Alt-1-1)

Figure 1 and Figure 2 illustrate the timeline of the “Explicit DCI indication” (Alt 1-1) policy to trigger the skipping of the next measurement occasion. In the figure, tMG represents the beginning of the measurement occasion, when the UE becomes unavailable for scheduling, TMG is the gap length of the measurement occasion, TSRT is the UE timeline (i.e., the retuning time of the RF interface). Our Alt-1-1 skipping policy combines two skipping strategies: buffer-based and prediction-based, respectively. As illustrated in the figures, these two policies make use of two windows to decide whether to skip the next measurement occasion: TB for buffer-based and TP for prediction-based. More specifically, the buffer-based policy uses the time window TB as follows: if the scheduler buffer for a UE is not empty during the time window TB, the network indicates to skip the next measurement occasion to the UE. Skipping is communicated using a dedicated indication (e.g., a bit in the DCI format). This flexibility proves essential in the presence of the UE timeline constraint (i.e., the latency due to the re-tuning of RF interface) especially when the XR frames get fragmented, and the fragments enter the buffer at different instants. For example, as illustrated in Figure 1, the buffer-based policy can use the first DCI command to indicate the skipping of the next measurement occasion and transmit the first TB as soon as resources are available.

If the XR frame (or PDU Set) enters the buffer outside the time window TB, the network uses the prediction-based policy to decide whether to skip the next measurement occasion as follows: if the next XR frame for a UE is predicted/expected to enter the scheduler buffer during the time window TP, the network indicates to skip the next measurement occasion to the UE. As illustrated in Figure 2, when the XR frame k-1 enters the buffer of the gNB scheduler, the next measurement occasion is still far ahead in the future. However, gNB can estimate the next XR frame k is expected to collide with the time window TP, which includes the next measurement occasion, and trigger the skipping indication. In the figure, we illustrate the skipping command as a separate message for the sake of explanation to show that the skipping can be delivered to the UE anytime between the arrival of frame k-1 and the UE re-tuning deadline. However, the skipping indication can be delivered with the first or second scheduling DCI in the example of Figure 2.

We observe that setting the time window TP for triggering skipping equal to the jitter of the traffic may be unnecessary. As illustrated in Figure 2, the XR frame k is expected close to the end of the MG and avoiding skipping in this case would slightly delay the transmission of the XR frame. Therefore, the window of the prediction-based skipping policy can be optimized to minimize the skipping ratio.

Figure 1 Explicit DCI indication (Alt 1-1) based on buffer status. Skipping is triggered by the status of the buffer during the time window TB and communicated to the UE by a dedicated skipping indication.

Figure 2 Explicit DCI indication (Alt 1-1) based on traffic prediction. Skipping is triggered by a prediction of the next frame colliding with the measurement occasion and communicated by an explicit “skip indication” in the DCI format.

## Skipping policy based on pattern (Alt-3)

Pattern-based policy is based on three parameters. In particular, the pattern is defined using the following parameters, as depicted in Figure 3:

PS: Period of the skipping pattern/cycle.

LS: Skipping duration/length within a cycle where scheduling is always prioritized.

OS: Offset of the skipping pattern indicating when skipping duration starts (i.e., when scheduling is always prioritized within a cycle).

The skipping pattern defines a cycle that repeats every PS time units where OS and LS indicate when and for how long scheduling is always prioritized over RRM measurements within a cycle. This means that MG is skipped every time the gap length (TMG) overlaps with the skipping duration (LS).

As it was observed for Alt 1-1, setting a skipping duration/length equal to the jitter of the traffic may be unnecessary when the traffic is expected after the beginning of the MG. As illustrated in Figure 3, both depicted MGs (on the bottom of the figure) are skipped since they overlap with the skip pattern (on the top of the figure). However, the first XR frame (first skip length) is expected after the beginning of first MG, whereas the third XR frame (third skip length) is expected before the second MG. Only this second case is critical from scheduling perspective, whereas in the first case skipping can be avoided resulting in a small extra delay. However, the skip pattern is static and must be configured to cover both cases. Therefore, the optimization prevents the optimization of the skipping duration/length LS to minimize the skipping ratio.

With configured pattern the skipping duration/length cannot be dynamically adjusted. This prevents the optimization of the skipping duration/length LS to minimize the skipping ratio.

The three parameters of the skipping pattern are indicated as follows {PS, LS, OS} in plots and tables.

Figure 3 Pattern (Alt 3) alternative for skipping MG. Skipping is triggered if skipping length LS and MG length TMG overlaps.

## Main parameters and results

In our SLS, we evaluate several configurations including the retuning time TSRT and parameters specific to the alternative solution. Table 2 in Section 3 summarizes the values of the parameters used in our analysis. Figure 4 shows the XR capacity versus the skipping ratio when TSRT=1 or 5 ms, while Figure 5 shows the ratio of satisfied UEs, which is measured when the cell load is equal to 7 UEs/cell (i.e., the capacity when MGs are not configured), versus the skipping ratio when TSRT=1 or 5 ms. It can be observed that the larger the skipping ratio is, the larger the capacity is. For example, with TSRT=1ms the Alt 1-1 skipping policy can be configured to achieve the same number of satisfied XR UEs as without scheduling restrictions (black point w/o MG), which corresponds to skipping all gaps. It can be further observed that the parameters of the Alt 1-1 skipping policy can be optimized to maximize the number of satisfied XR UEs and minimize the skipping ratio. For example, Alt 1-1 with TSRT=1 (magenta cross) can be configured to have the same skipping ratio of Alt 1-1 when TSRT=5ms (green square), namely 44%. However, a smaller retuning time TSRT  allows higher flexibility to Alt 1-1 since skip decision can be communicated close to the beginning of the MG. This additional flexibility can be used to minimize the time window TP used by the prediction-based skipping policy and, hence reducing the skipping ratio. In contrast, it can be observed that Alt 3 experiences a larger capacity loss than Alt 1-1 even with comparable skipping ratio due to the impossibility to react to unexpected events such as cell load variation, interference, transmission errors and retransmissions.

Figure 4  XR capacity(measured in #UEs/cell) vs skipping ratio for Alt 1-1 when TSRT=1 or 5 ms and Alt 3.

Figure 5  Ratio of satisfied UEs vs skipping ratio for Alt 1-1 when TSRT=1 or 5 ms and Alt 3. Ratio of satisfied UEs has been measured when cell load is equal to 7 UEs/cell (i.e., the XR capacity of ‘w/o MG’ when PDB=10ms).

Table 1 below illustrates the capacity for different PDBs (10ms and 15ms as per TR38.835) and the skipping ratio of the alternatives we have considered in our SLSs. We also evaluated the same KPIs with legacy MGs (label “MGs enabled”) and without any MG configured (label “No MGs”).

In the presence of scheduling restrictions, results show that the system should skip approximately 70% of measurement occasions to fully recover the same XR capacity as when scheduling restrictions are disabled. When using Explicit DCI indication (Alt 1-1) the system experiences the largest capacity gain (in some configurations up to 75%). Instead, pattern-based alternatives (Alt. 3) result in lower capacity gain (around 25%) than Explicit DCI (Alt 1-1). Skipping ratio for Alt. 1-1 can be reduced down to 26% (for Alt. 3 it is around 43%) while still maintaining higher capacity gain than Alt. 3 (50% for Alt. 1-1 vs 25% for Alt. 3).

More specifically, in our SLS evaluation summarized in Table 1 for measurement gap configuration (40, 6) ms, we show that Explicit DCI indication (Alt 1-1) achieves the best performance among all alternatives. For example, Explicit DCI indication with TB=3ms and TP=4ms achieves XR capacity of 7 UEs/cell and capacity gain of 43% with PDB=10ms. Even with large UE skipping retuning time (TSRT=5 or 6 ms) the capacity gain can be kept around 50% and the skipping ratio can be minimized down to 26%. This is obtained by combining the prediction-based and the buffer-based skipping strategies as well as the flexibility provided by the dedicated DCI skipping indication.

Instead, pattern-based policy achieves smaller capacity and lower capacity gain than Explicit DCI even with comparable skipping ratio. For example, Pattern-based policies with {PS,LS,OS}={17,8,12}ms {PS,LS,OS}={17,8,RRC}ms archive XR capacity of 5 UEs/cell with PDB=10ms even if skipping ratio is 43% and comparable to Explicit DCI. Additionally, reconfiguring the offset using RRC reconfiguration helps in reducing variability of capacity and skipping ratio: as it can be observed in the table both {16,8,RRC} and {17,8,RRC} achieve very similar performance. However, Alt 3 (pattern) still underperforms in capacity compared to Alt 1-1 (Explicit DCI) in the worst case and with comparable skipping ratio.

Based on the above observations and results, we propose in our companion contributions R4-2419326 and R4-2419327 that RAN4 should agree that Alt. 1-1 (explicit indication via DCI) is a feasible solution for gaps/restrictions skipping.

Table 1 – Results of SLS in DU deployment with (MG interval, MG gap length) equal to (40,6) ms. N/A means Not Applicable.

| Scheme | Alternative | Retune Time TSRT [ms] | Capacity (PDB: 10ms) [#UEs/cell] | Capacity (PDB: 15ms) [#UEs/cell] | Capacity Loss (PDB: 10ms) | Capacity Loss (PDB: 15ms) | Capacity Gain (PDB: 10ms) | Capacity Gain (PDB: 15ms) | Skipping ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No MGs | N/A | N/A | 7 | 9 | N/A | N/A | N/A | N/A | N/A |
| MGs enabled (40,6) | N/A | N/A | 4 | 7 | 43% | 22% | N/A | N/A | N/A |
| Genie | N/A | N/A | 7 | 9 | N/A | N/A | 75% | 29% | 72% |
| Explicit DCI TB=3ms, TP=6ms | Alt 1-1 | 3 | 7 | 8 | 0% | 11% | 75% | 14% | 55% |
| Explicit DCI TB=2ms, TP=6ms | Alt 1-1 | 4 | 6 | 8 | 14% | 11% | 50% | 14% | 50% |
| Explicit DCI TB=1ms, TP=6ms | Alt 1-1 | 5 | 6 | 8 | 14% | 11% | 50% | 14% | 44% |
| Explicit DCI TB=0ms, TP=6ms | Alt 1-1 | {1;6} | 6 | 8 | 14% | 11% | 50% | 14% | 38% |
| Explicit DCI TB=3ms, TP=4ms | Alt 1-1 | 1 | 7 | 8 | 0% | 11% | 50% | 14% | 44% |
| Explicit DCI TB=2ms, TP=4ms | Alt 1-1 | 2 | 6 | 8 | 14% | 11% | 50% | 14% | 38% |
| Explicit DCI TB=1ms, TP=4ms | Alt 1-1 | 3 | 6 | 8 | 14% | 11% | 50% | 14% | 32% |
| Explicit DCI TB=0ms, TP=4ms | Alt 1-1 | {1-6} | 6 | 8 | 14% | 11% | 50% | 14% | 26% |
| Pattern {16,8,12}ms | Alt 3 | N/A | 5 | 7 | 29% | 22% | 25% | 0% | 50% |
| Pattern {17,8,12}ms | Alt 3 | N/A | 5 | 7 | 29% | 22% | 25% | 0% | 43% |
| Pattern {16,8, 1st PDU} | Alt 3 | N/A | 5 | 7 | 29% | 22% | 25% | 0% | 45% |
| Pattern {17,8,1st PDU} | Alt 3 | N/A | 5 | 7 | 29% | 22% | 25% | 0% | 43% |

- 4 System-level simulation parameters

This section illustrates the results obtained considering all configurations discussed in Section 3. Results are clustered by alternative as follows:

Baseline: it includes the results of legacy with MG disabled, legacy with MG enabled, and Genie. Results are illustrated in Table 2.

Alt 1-1: it includes the results of Explicit DCI indication solution where the network uses both buffer-based and prediction-based policies to decide whether to skip next measurement opportunity. Curves with the same value of prediction-based window TP and different values of UE timeline constraint (skipping retuning time) TSRT are depicted in the same plot. The buffer-based window is equal to the difference between the prediction-based window and the UE timeline constraint: TB=TP-TSRT. Results are illustrated in Table 3.

Alt 3: it includes the results of Pattern-based solution indication where the patter is configured semi-statically by the network. Two set of plots are illustrated: with fixed offset and with offset set as first PDU transmission. Results are illustrated in Table 4.

Table 1 below summarizes the values of the parameters used in our analysis.

Table 1 – Parameters used in SLS. Note that {X}+ means max{0, X}.

| Parameter | Description | Value(s) |
| --- | --- | --- |
| TMGI | Measurement gap interval | 40 ms |
| TMG | Measurement gap length | 6 ms |
| TSRT | Skipping retuning time | {1, 2, 3, 4, 5, 6} ms |
| TB | Buffer-based window for Explicit DCI policy (Alt 1-1) | {6 – TSRT, 4– TSRT}+ ms: {5 – TSRT}+ ={4,3,2,1} ms {4 – TSRT}+ ={2,1,0} ms |
| TP | Prediction-based window for Explicit DCI policy (Alt 1-1) | {6, 5, 4} ms |
| PS | Period of the skipping cycle for pattern-based policy (Alt 3) | {16, 17} ms |
| LS | Duration of skipping within the cycle for pattern-based policy (Alt 3) | 8 ms |
| OS | Start of skipping within the cycle for pattern-based policy (Alt 3) | 12 ms RRC (learned from PDU transmission) |

We observe that the traffic pattern of video stream for XR applications is defined in TR38.835 as follows:

tk=k∙1fps+J

Where k≥1 is the index of the XR frame, fps is the frame rate (frames per second), and J is the jitter distributed as a Truncated Gaussian distribution in [-4; 4]ms. With this model and assuming no offset in frame generation, the first frame (k=1) is expected to enter the gNB buffer in the time interval [12.67; 20.67]ms. Therefore, for pattern-based policy (Alt 3) we evaluate as offsets the following options:

OS=12: it matches the first frame arrival (labels {16,8,12} and {17,8,12});

OS=RRC reconfiguration: offset is reconfigured once after first packet transmission (labels {16,8,1st PDU} and {17,8,1st PDU}).

Table 2 – Results of baseline schemes (legacy without MG, legacy with MG, and Genie). Note that for these schemes the UE timeline constraint does not affect performance.

Table 3 –Results of Alt 1-1 considering different configurations of the skipping policy (i.e., TB and TP) and UE timeline constraints. The parameter TB is equal to TB=TP-TSRT. From top to bottom: TP from 4 to 6 ms.

Table 4 – Results of Alt 3 (Pattern) considering different configurations of the skipping pattern (i.e., PS , LS, and OS). The duration/length of the window LS where transmission/reception is prioritized over RRM measurement is LS = 8 ms in all simulations (i.e., it is equal to the jitter). From top to bottom: PS from 16 to 17 ms.