3GPP TSG-RAN WG4 Meeting #113	R4-2418900

Orlando, US, November 18th-22nd, 2024

Source: 	Ericsson

Title:  	Discussion on testability and interoperability issues for beam management

Agenda Item:	7.17.4.2

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #112bis, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to testability and interoperability.

# Discussion

Issue 2-8: Channel model for beam prediction testing

- Proposals - Option 1: simplified CDL - Option 2: TDL - Option 3: others - Recommended WF - Option 1 FFS how to simplify the CDL channel models, there are some proposals in this meeting FFS to investigate a quantitative metric to estimate the “goodness” of a simplified CDL channel model (see Proposal 2 in R4-2415497)

First of all, even though some proposals from companies raised the ‘simplified’ CDL channel model in last meetings, it isn’t commonly understood that which parameters may be simplified, especially when there’re more than one parameter which can be simplified independently or dependently. To our understanding, AOA and AOD are two parameters which may be simplified with respect to the beam numbers of Set A and Set B.

Proposal 1: RAN4 to clarify the following parameters which simplifies the CDL channel:

- Number of AOA

- Number of AOD

To evaluate the quality of a simplified CDL channel, a general approach is verifying the prediction performance difference between the simplified CDL channel and the reference CDL channel (not simplified or before being simplified).

The simplified CDL channel essentially can be applied for training and inference. However, what we shall specify with common test configurations is inference only, since training isn’t in the scope of the conformance test, such that we don’t expect to compare the simplified CDL channel and the reference CDL channel in training, in other words, the training data is done by applying the reference CDL channel.

Proposal 2: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training.

Proposal 3: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

Step 1: Test (inference) with the reference CDL channel.

Step 2: Test (inference) with the simplified CDL channel.

Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

Issue 2-9: Test setup requirements

Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

We’d like to draw attention to the ratio between the numbers of Set A and B, instead of directly discussing the achievable maximal beam numbers from implementation or test perspective.

First of all, we’d like to raise the below questions:

- What AI/ML model performance or characteristics shall be verified by the AI/ML model test?

- What impact to the prediction performance may be caused by the beam number varieties (the ratio between the numbers of Set A and B) in the model?

Since we have not exploited the unclearness, we don’t think it’s ready to define the exact numbers of Set A and B for the conformance test.

In our opinion, the test shall at least verify the following concerns on the numbers of Set A and B,

- Prediction scaling, i.e., the capability of using fewer inputs to predict same outputs, or the capability of using same inputs to predict more outputs.

- Prediction model computational power/complexity, i.e., the capability of handling more inputs or outputs.

Observation 2: Different ratios between the number of Set A and Set B may demonstrate various AI/ML model characteristics.

Since the two concerns take care different AI/ML model characteristics, to get the two diverse evaluation results, eventually more than one set of number of Set A/Set B shall be defined in the conformance test in perspective.

Proposal 4: For BM-case1, at least two sets of numbers of Set A and Set B are provided as test configurations. Below are two examples:

- For verifying the prediction model computational power/complexity of AI/ML model, the number of set A = X and the number of set B = X/4.

- For verifying the prediction scaling capability of AI/ML model, the number of set A = X and the number of set B = X/16

Note: Different accuracy requirements may be applied for different sets of number of set A and set B.

Where X may be the maximum number of Set A that a UE supports mandatorily.

Issue 2-11: Beam prediction testing process

- Proposals - Option 1: Test needs to have at least 2 parts: - 1. inference part of the test (UE has to make the actual prediction) - 2. predicted beam measurement (UE has to measure and report “the ground truth”) - Option 2: others - Recommended WF - Take Option 1 as baseline, discuss further details of tests such as under what condition should the ground truth be measured, how to ensure the UE does not cheat, etc

Below are some preliminary analyses on the condition of ground truth, e.g., the SNR condition.

The purpose of defining the SNR condition is to get an ideal RSRP measurement accuracy by minimizing (most of) measurement errors caused by low SNR. Hence, determining the SNR condition may rely on the configurations of reference siganl, e.g., periodicity, density, bandwidth, subcarrier spacing and channel model in order to obtain a SNR level which is applicable for most typical field scenarios.

Recap the procedure deriving the accuracy requirement on the measured L1-RSRP in Rel-15, the achievable accuracy at certain SNR level is derived by simulations. In a same way, it is rational to extend those simulations to a higher SNR level where the L1-RSRP estimation error can be less than a threshold with X percentile.

Generating such SNR level in the test environment is another issue to be addressed, a concern is that the realization of SNR level is not only about generating a proper signal level but also the noise floor level generated as a side effect needs to be considered. To realize high SNR, the probes in test environment shall be able to generate signals at high output power level over the air. An underlying problem is that the wanted signal level can sometimes become too high to be created in the OTA environment. In that case, the noise floor becomes dominant which may deteriorate the test performance. Test equipment vendors are encouraged to comment on the SNR level from a test environment perspective.

Proposal 5: RAN4 to evaluate the SNR condition for the ground truth by simulation.

If the ideal measurement of RSRP is acquirable, then the test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Still, arranging the details on the sequence and test configuration may need further investigation. In essence, to our understanding, the UE is deemed to execute the RSRP measurement and prediction with same test configurations except for different SNR conditions.

Proposal 6: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Note 1: Test configurations are kept same in the steps except for different SNR conditions.

Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part.

A concern to the ideal measurement of RSRP from error distribution perspective is, since the ideal measurement of RSRP is believed to still have a residual error compared with the genie RSRP, the maximal offset between the predicted RSRP and ideal measurement RSRP may be

÷ predicted RSRP error÷ +÷ ideal measurement RSRP error÷

or

÷ predicted RSRP error - ideal measurement RSRP error÷

It depends on if the two errors are positive correlated or negative correlated or noncorrelated. The worst case is doubling the (residual) measurement error by using the formular:÷ predicted RSRP error÷ +÷ ideal measurement RSRP error÷.

Proposal 7: RAN4 to study the correlation between error (offset) between predicted RSRP and ideal measurement RSRP, and the correlation impact to the prediction metrics/KPIs.

Yet another issue is, at least one company in the last meeting commented that since the ideal measurement of RSRP and the predicted RSRP both are provided by the UE under test, the UE may deliberately modify the two values to be better aligned in the report. On the contrary, the test environment hasn’t any available approach to evaluate the authenticity of the measured and predicted values. We agree on the observation and the issue shall be solved.

A simple way for the test environment is adding a procedure manually changing the beam index between the prediction duration and measurement duration. By this way, the UE reports the results from the corresponding prediction and measurement with the corresponding beam indexes, but only the test environment has knowledge of the beam index mapping between each other.

By this way, the UE reports the results from the prediction and measurement with the corresponding different beam indexes for the same beam, while the UE doesn’t know the relationship between the beam indexes, only the test environment has the knowledge of beam index mapping between each other.

Proposal 8: How to avoid cheating in the reports on RSRP measurement and prediction is a valid issue to be studied. A simple way for the test environment is adding a procedure to manually switch the beam indexes between the prediction duration and the measurement duration.

Issue 2-5:	OTA Test setup for beam management

In this issue, we discuss the channels to be applied in training and inference.

For BM-case1, which predicts the best beams in Set A (different from Set A) with respect to the measurements on Set B, at the same time domain with respect to spatial correlation between Set B and Set A, two channels with same parameters but with some random distributions and variations are applied for the training and inference respectively.

Therefore, generating 2 channel instants, channel #1 for training and channel #2 with same parameters for inference is applicable to BM-case1, which is demonstrated in Figure 1.

Figure 1 Channel update for BM-case 1

For BM-case2, which harnesses the historical predictable non-random variation of Set B to anticipate Set A (same as Set B) for one or more future time instances with respect to temporal correlation between Set B and Set A, from test perspective, it needs some trackable changes, e.g., UE monotonous movements which reflect the temporal correlation as prerequisite of the temporal prediction.

In practice, the UE moving direction and UE velocity may determine temporal correlation in reality, but the temporal correlation in test must rely on a set of consequent changes of channel parameters. At the least, establishing a set of channel instants from channel #1 to channel #m for training and channel #m+1 to channel #n for training. These channels contain a set of parameters which update sequentially and gladfully from training to inference, as depicted in Figure 2. In which, the number of m and n is determined by the measurement and prediction windows in BM-case2.

Figure 2 Channel update for BM-case 2

Wherein, the parameters needing updates between channels may at least comprise:

- Departure angles (AOD, ZOD)

- Arrival angles (AOA, ZOA)

- UE position and/or rotation

- Channel coefficient

In an illuminative example, for BM-case 2, the updates of parameters may include the below:

- AOD in channel #1 is 10 degrees,

- AOD in channel #2 is 20 degrees,

- …,

- AOD in channel #k is (10*k) degree

in order to mimic the UE’s movement.

Proposal 9: Regarding channel generation in tests (for each test configuration, a test case may contain more than one test configuration):

- For BM-case1, generating 2 channel instants including channel #1 for training and generating channel #2 with same parameters for inference.

- For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

  - Generating a set of channel instants, from channel #1 to channel #m, for training, and

  - Generating a set of channel instants, from channel #m+1 to channel #n for inference

Wherein,

  - The channels instants comprise sequent and grade updates of a set of parameters.

  - The numbers of m and n depend on the measurement window and prediction window of BM-case2.

- The channel parameters may comprise one or more than one of

  - Departure angles (AOD, ZOD)

  - Arrival angles (AOA, ZOA)

  - UE position and/or rotation

  - Channel coefficient

Issue 2-6:	Datasets for training/testing

Agreement: Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training FFS on proposals to augment training data to avoid overfitting of UE models to the test environment RAN4 to strive to make the test conditions similar to field deployment conditions

The rationale behind the agreements on the issue is that the coverage of datasets for training/testing shall be broad adequately to meet or be close to the practical field deployment scenario as much as possible. In this manner, we suggest considering defining the test environment/conditions which can demonstrate the robustness of the AI/ML model.

Proposal 10: In order to avoid overfitting of UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions.

Since the training data are uniquely affected by the specific UE hardware and digital processing units, the error of training data could be left for implementation from each UE vendor, the effect to the predict accuracy may be covered by the overall measurement error.

Observation 1: the measurement error in Datasets for training/testing may be covered by the overall measurement error.

# Conclusion

Observation 1: the measurement error in Datasets for training/testing may be covered by the overall measurement error.

Observation 2: Different ratios between the number of Set A and Set B may demonstrate various AI/ML model characteristics.

Proposal 1: RAN4 to clarify the following parameters which simplifies the CDL channel:

- Number of AOA

- Number of AOD

Proposal 2: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training.

Proposal 3: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

Step 1: Test (inference) with the reference CDL channel.

Step 2: Test (inference) with the simplified CDL channel.

Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

Proposal 4: For BM-case1, the ratio between the numbers of Set A and Set B shall be a metric, at least two sets of numbers of Set A and Set B are provided as test configurations. Below are two examples:

- For verifying the prediction model computational power/complexity of AI/ML model, the number of set A = X and the number of set B = X/4.

- For verifying the prediction scaling capability of AI/ML model, the number of set A = X and the number of set B = X/16

Note: Different accuracy requirements may be applied for different sets of number of set A and set B.

Where X may be the maximum number of Set A that a UE supports mandatorily.

Proposal 5: RAN4 to evaluate the SNR condition for the ground truth by simulation.

Proposal 6: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Note 1: Test configurations are kept same in the steps except for different SNR conditions.

Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part.

Proposal 7: RAN4 to study the correlation between error (offset) between the predicted RSRP and ideal measurement RSRP, and the correlation impact to the prediction metrics/KPIs.

Proposal 8: How to avoid cheating in the reports on RSRP measurement and prediction is a valid issue to be studied. A simple way for the test environment is adding a procedure to manually switch the beam indexes between the prediction duration and the measurement duration.

Proposal 9: Regarding channel generation in tests (for each test configuration, a test case may contain more than one test configuration):

- For BM-case1, generating 2 channel instants including channel #1 for training and generating channel #2 with same parameters for inference.

- For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

  - Generating a set of channel instants, from channel #1 to channel #m, for training, and

  - Generating a set of channel instants, from channel #m+1 to channel #n for inference

Wherein,

  - The channels instants comprise sequent and grade updates of a set of parameters.

  - The numbers of m and n depend on the measurement window and prediction window of BM-case2.

- The channel parameters may comprise one or more than one of

  - Departure angles (AOD, ZOD)

  - Arrival angles (AOA, ZOA)

  - UE position and/or rotation

  - Channel coefficient

Proposal 10: In order to avoid overfitting of UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions.

# References

- R4-2414323 WF on AI/ML, Qualcomm