# 3GPP TSG-RAN WG4 Meeting #113	R4-2418901

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.17.4.1

Source: 	Ericsson

Title:	Simulation results of measurement error impact on beam prediction

Document for:	Information

- Introduction

In this contribution, the link level simulation (LLS) results for the L1-RSRP differences based on the simulation assumption from the R4-2417189 will be shown. Also, the impact of measurement error including both basedband errors and RF errors on the prediction accuracy is provided.

- Simulation results

In this section, we first simulated the LLS based on the Table 1-5 in R4-2417189 and obtained the absolute values for L1-RSRP differences between the ideal and practical channel estimation, as listed in Table 1.

From the table we can see that even though the RSRP differences can be up to 2.5 dB with a worse channel condition, within the suggested SNR range [-3,0,3], the L1-RSRP differences is between 0.18~0.33 dB which could have minor impact on the performance metric.

Table 1 The SSB L1-RSRP differences

| SNR (dB) | L1-RSRP differences (dB) |
| --- | --- |
| -15 | 2.53 |
| -12 | 1.25 |
| -9 | 0.6 |
| -6 | 0.42 |
| -3 | 0.33 |
| 0 | 0.28 |
| 3 | 0.18 |
| 6 | 0.13 |
| 9 | 0.09 |

Observation 1: The L1-RSRP differences of SSB in the SNR range [-3,0,3] is only a few tenths of dBs so its impact would be minor.

Then we studied the impact of measurement error for the BM case 1 for Set B is subset of Set A. According to the suggested simulation steps approach A, the measruement errors are applied only in the inference phase. In addition to take the +/-0.3 dB with uniform distribution as the baseband (BB) error, +/- 4dB and +/- 6.5 dB with uniform distribution are also applied as the RF errors. Then the impact of the measurement errors on the prediction accuracy for Top 1-5 is shown in Table 2. Top K/1, i.e. the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams” is considered as the prediction accuracy, and 1 dB margin is considered.

Table 2 the impact of measurement error on prediction accuracy (%)

|  | No measurement error | With BB errors +/- 0.3 dB | With RF errors +/- 4dB | With RF errors +/- 6.5dB |
| --- | --- | --- | --- | --- |
| Top-1 | 94.3 | 94.3 | 83.2 | 73.2 |
| Top-2/1 | 97.8 | 97.7 | 94.3 | 88.5 |
| Top-3/1 | 98.3 | 98.7 | 98.2 | 96.7 |
| Top-4/1 | 99.2 | 99 | 98.8 | 99.2 |
| Top-5/1 | 99.3 | 99.2 | 99.2 | 98.8 |

Observation 2: The measurement error impact is mainly from the RF errors.

Based on the initial results we provided, compared to the minor BB errors, the RF errors dominates the negative impact of measurement errors in most of the range, especially for the Top 1 and 2 beam prediction, up to 9% loss with 4 dB erros and up to 20 % loss with 6.5 dB errors.

Proposal 1: The negative impact of measurement error is mainly from the RF errors and cannot be ignored at least on Top 1 and 2 beam prediction.

Proposal 2: Whether tightening the current RF errors needs further discussison.

- Conclusion

Observation 1: The L1-RSRP differences of SSB in the SNR range [-3,0,3] is only a few tenths of dBs so its impact would be minor.

Observation 2: The measurement error impact is mainly from the RF errors.

Proposal 1: The negative impact of measurement error is mainly from the RF errors and cannot be ignored at least on Top 1 and 2 beam prediction.

Proposal 2: Whether tightening the current RF errors needs further discussison.

- Reference

[1]. R4-2417189, “Simulation assumptions for beam prediction”, vivo