3GPP TSG-RAN WG4 Meeting #113	R4-2418907

Orlando, US, November 18th-22nd, 2024

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Agenda Item:	7.15.2.2

Document for:	Discussion

# Introduction

In terms of  WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [2].

This contribution outlines our point of view on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC as follows.

# Discussion

## Topic #2-2: L3 measurement delay reduction by optimizing CSSF for UE not in multiple-Rx simultaneous reception mode

Issue 2-1: Clarification on the bullets in WID for this CSSF optimization

The WID RP-242380 agreed in the RANP#105 meeting is:

- FR2-1 SSB based L3 measurement delay reduction for connected mode - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

Regarding the agreement, our interpretation is a common solution of CSSF optimization shall be designed for a UE not operating with multiple-Rx based simultaneous L3 measurement and a UE not capable of multiple-Rx based simultaneous L3 measurement.

Proposal 1: The solution of CSSF optimization is same to the below cases:

- A UE not operating with multiple-Rx based simultaneous L3 measurement

- A UE not capable of multiple-Rx based simultaneous L3 measurement

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Agreement in RAN4 #112 meeting: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

Proposal 2: Regarding UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF, we recommend including aspects 1, 2, and 3 in the scope.

Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

Current requirements in TS38.133; Priority. ##### 9.1.5.1.1 EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps; Low. ##### 9.1.5.1.2 SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps; high. ##### 9.1.5.1.3 NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps; Low. ##### 9.1.5.1.4 NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps; Low. ##### 9.1.5.2.1 EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps; Low. ##### 9.1.5.2.2 SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps; high. ##### 9.1.5.2.3 NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps; Low. ##### 9.1.5.2.4 NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps; Low. ##### 9.1.5.2.5 SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps; high. ##### 9.1.5.2.6 NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps; Low. ##### 9.1.5.2.7 NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps; Low. #### 9.1.5.3 Monitoring of multiple layers within NCSG; Low. - Recommended WF - Discuss proposal from Xiaomi

Proposal 3: We suggest covering SA, EN-DC and NE-DC in the scope without deprioritization any of them.

Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF

Agreement in online session: On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers: - Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later.

The third searcher is derived from the case that a UE performs measurement within the gap in a FR and meanwhile the UE conducts measurement outside the gap in a different FR. The third searcher is the searcher for measurement within the gap. Given that, the gap searcher can be applied for measurements outside the gap when no gap is configured.

Proposal 4: RAN4 to check whether the 3rd searcher only can be used for CSSF optimization when no gap is configured.

Another concern is if 3 searchers is supported, in alignment with the below content about NW indicating the particular CC(s) to be measured, to gain more flexibility, the 3rd searcher may be configurable by the NW. In our opinion, it is more efficient than equally sharing among PCC/PSCC and/or SCCs.

Proposal 5: When 3 searchers is supported,

- 1 searcher for PCC,

- 1 searcher for other SCCs and frequency layers following legacy equalling sharing;

- 1 additional searcher is used for the specific PCC/PSCC/SCCs based on NW indication.

NW indicates the cell list which applied the measurement by the additional searcher.

RAN4 to discuss the use limitation on this additional searcher.

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Candidate options for further discussion: - Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B - Option 1A: Reuse the legacy principle for neighboring cell measurement. - Option 1B: Based on network indication - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band. - Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions For 3 searchers: In addition to option 1 and 2 - Option 3: consider to use 3 searchers for CSSF reduction. - Depending on issue 2-4.

Regarding Option 1:

First of all, a question to be addressed arises, as concerned by some companies in the last meeting, whether measuring only one CC out of multiple CCs that all are configured with MO is a valid use case?

A RRM requirement in Clause 9.2.3.2 of TS38.133, which is duplicated in the below, actually proves that a similar use case is valid, by defining the measurement rule on intra-frequency layer (not including the serving cell) of only one CC out of multiple CCs that all are configured with MO. Here, we just extend the existing use case for the intra-frequency layer measurement to the serving cell measurement.

Given that, we tend to agree that measuring only one CC out of multiple CCs which all are configured with MO is a valid use case.

Proposal 6: Measuring only one CC out of multiple CCs which all are configured with MO is a valid use case.

#### 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

Reusing the rule in Rel-15 for CSSF optimization allows a UE to measure the CCs with respect to the rules highlighted in blue and the CC(s) to be skipped from measurement by the CSSF optimization solution in this WI exactly is the CCs highlighted in yellow. From NW perspective, we observe a critical drawback that if MOs are configured for all CCs, then the NW cannot decide the CC(s) to be measured even the NW knows which CC is measured.

In this way, the NW couldn’t acquire the entire information of the overall channel quality on the whole band or all CCs in the band, if the UE doesn’t report SSBs on all intra-frequency layers (one particular example is only one frequency layer is to be measured) in the serving cells and the similarity of channel quality on the whole band isn’t valid.

Then, for the purpose of determining the CCs to be measured and acquiring the overall channel quality of the band, the NW has to reconfigure/update MOs and/or measurement report configurations one or more than one time for the UE. Apparently, these actions instead add more and unneglectable signaling load and delay, which is not expected by the WI.

Another advantage of the NW determining the CCs to be measured is that the UE cannot change the CCs to be measured once the rule is fixed, by contrast, the NW may update the CCs to be measured based on the reports from the UE, which improves the flexibility of the scheduling, network management etc.

In this sense, we prefer that the NW shall be in charge of measuring the CCs. Moreover, we consider it’s beneficial to give the UE of some flexibilities. For example, the UE may decide the CC(s) to be measured instead if the NW doesn’t offer any configuration or indication for identifying the CC(s) to be measured.

However, it shall be noted that some questions shall be clarified for the case of UE determining the CC (s) to be measured, e.g., shall there be any condition/rule enabling and disabling the UE to determine the CC(s) to be measured?

Proposal 7: NW indicates the particular CC(s) to be measured or the the particular CC(s) not to be measured, if multiple serving CCs are in the same band.

Proposal 8: In case of no NW configuration/indication, UE is allowed to determine the CC(s) to be measured, but the rules shall be studied further.

Among all SCCs, to reduce CSSF, the measurement on particular CC(s) may be skipped, and it’s reflected in  NSCC_SSB in CSSF definition table in TS 38.133 which could be reduced eventually. It’s noted that NSCC_SSB in that CSSF table is for accounting the SCC(s) on which neighbor cell measurement isn’t required, we may focus on such case.

Observation 1: Option 1(CSSF reduction by skipping measurement on particular CC(s)) covers the below case:

- CSSF for SCCs where neighbor cell measurement isn’t required, e.g., reducing NSCC_SSB.

And it doesn’t cover the below cases:

- CSSF on SCCs where neighbor cell measurement is required

- CSSF on SCCs for inter-frequency without gap.

Therefore, Option 1 enhancement only covers NSCC_SSB  number reduction.

Regarding Option 2:

The CSSF rule actually is based on 2 searchers assumption, and the CSSF rule implicitly indicates usages of searchers in the below:

- One searcher is dedicated for PCC,

- The other searcher is shared by all SCCs.

  - For DC, the cell search/L3-measurement engine is dedicated for PSCC with 50% of the overall time, and for the other 50%, the engine is equally shared by the rest of the CCs.

  - For CA, half of the other searcher (50%) is dedicated for SCC where neighbour cell measurement is required, which is prioritized than other SCC(s), the remaining half of the searcher is shared among FR1 SCCs and FR2 SCCs where neighbour cell measurement is not required.

It appears that the CSSF can be further optimized through (re-)arranging the measurement ratio among CCs. Compared with Option 1, Option 2 has wider coverage to deal with the CSSF issue, i.e., no only reducing the total number of NSCC_SSB , but re-arrange scaling factor.

From the carrier on which measurement ratio is to be adjusted perspective, there’re at least the below options:

- Option 1: Adjust measurement ratio between PCC/PSCC and SCC.

- Option 2: Adjust measurement ratio among SCCs.

- Option 3: Option 1 and Option 2 both are applied.

We understand that Option 1 and Option 2 both could be applied in principle. But reducing the measurement ratio of PCC/PSCC and consequently delaying the measurement on PCC/PSCC may leads to the network performance degradation. We must be aware of such risks.

Another concern is even though a CC, e.g., PCC/PSCC, shares 50% of its searcher to SCCs, a single SCC among them gains a very limited benefit if the shared part is shared by all SCCs, especially when the number of configured SCCs is more than 2. Below figures demonstrates the effects.

Where,

- Figure 1 is the reference as baseline.

- Figure 2 is the case of 2 SCCs equally sharing 50% searcher from PCC.

- Figure 3 is the case of 3 SCCs equally sharing 50% searcher from PCC.

From Figure 2 to Figure 3, it could be observed that when the number of SCCs is 3, the measurement enhancement on a single SCC introduced by the shared searcher from PCC is reduced significantly, which is demonstrated by the following numerical form of measurement ratios.

- The original measurement ratio of a SCC is ½=50%.

- The measurement ratio of a SCC with the shared searcher from PCC in case of two SCCs, is ¾ =75%, improved from the original 50%.

- The measurement ratio of a SCC with the shared searcher from PCC in case of three SCCs, is 3/8=37.5%, improved from the original 33.3%.

Figure 1 Searcher sharing between two SCCs as baseline

Figure 2 Meas. on SCCs, two SCCs equally apply the 50% searcher from PCC, in case of two SCCs.

Figure 3 Meas. on SCCs, three SCCs equally apply the 50% searcher from PCC, in case of three SCCs.

Observation 2: Even though a CC, e.g., PCC/PSCC shares 50% of its searcher to all SCCs, a single SCC among them gains a very limited benefit, if the shared part is shared by all SCCs, especially when the number of configured SCCs is more than 2. It eventually leads to an inefficient solution.

Furthermore, we don’t preclude measure ratio sharing between SCCs as well. For example, A SCC can share its 50% measurement ratio to another SCC or two CCs can share 25% measurement ratio respectively to the third SCC.

Taking a simplified example, in a case of 2 SCCs (e.g., the CCSF of a SCC = 2), after the 50% of searcher of SCC1 is shared to SCC2,

- The CSSF of the SCC 1 = 2*(1/ (1-1/2)) =4.

- The CSSF of the SCC 2 = 2*(1/ (1+1/2)) =4/3.

From the result, we observe that the SCC 2 gains significant measurement improvement.

Figure 4 Meas. on SCCs, SCC1 borrows 50% searcher of SCC2

On other hand, in real deployment, the NW may expect to know the measurement of certain CC(s) more urgently than other CCs, however, it takes a lot of time for the UE to measure all component carriers using the current CSSF, especially when accounting for FR2 undetected CC. Before obtaining measurement results on every CC including the intended CC(s), the NW might have to wait a lengthy time. In some other cases, the NW may expect to know the measurement of certain frequency range(s), e.g., FR1, more urgently than other frequency range(s), e.g., FR2. Such that it is expected the measurement on one or more than one CC can be faster than others based on the NW preference.

If we revisit Figure 3 but SCC1 applies all 50% searcher of PCC, the result is shown in Figure 5, the SCC1 can reach significantly higher measurement ratio.

Figure 5 Meas. on SCCs, SCC1 borrow 50% searcher of PCC, in case of three SCCs

Besides, we observe additional benefits from the NW indication solution, in one example, the NW can prioritize the CCs which is unknown; in another example, NW can prioritize the specific FR2/FR2 CCs based on different use cases.

Proposal 9: When two searchers are applied, NW to indicate the candidate PCC/PSCC/SCC(s) or the frequency range of the PCC/PSCC/SCC(s) which applies a specified percentage X% of the searcher resource, the other SCC(s) together with PCC/PSCC share the other (1-X)% of the searcher resource. If no NW indication, UE follows the legacy searcher sharing rules.

Regarding measurement ratio, even the above examples use 50% as a typical percentage of a searcher to be shared, we would like the number is configurable by the NW. As a reference, measurement gap sharing in MG can be a start point to design the measurement ratio.

Proposal 10: Measurement ratio is configurable by NW, the MeasGapSharingScheme in MG can be a start point.

Issue 2-6: feature capability of L3 measurement delay reduction by optimizing CSSF

- Option 1 (CATT): The feature capability depends on the solution: - For option 1 and option 2 in issue 2-5, a new UE capability is needed to indicate the UE support of new solution. - For option 3 in issue 2-5, the existing per-FR gap capability can be reused. - Option 2 (Apple, Nokia, CTC, Samsung): RAN4 to introduce a new individual capability for CSSF reduction in R19. - Option 3 (Apple, CTC, HW, Ericsson, Samsung): delay the capability discussion to the end of the core part. - Recommended WF - Moderator note: - Check if following option is agreeable: - Option 4(moderator): RAN4 to introduce a new individual capability for CSSF reduction in R19. - delay the detailed capability design to the end of the core part.

Proposal 11: Agree on Option 4, RAN4 to introduce a new individual capability for CSSF reduction in R19.

delay the detailed capability design to the end of the core part.

Issue 2-7: Other WID scope discussion

- Proposal 1(CMCC, ZTE, CTC): after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable. - Recommended WF - [Moderator note]: - Check if proposal 1 is agreeable?

Proposal 12: Agree on Proposal 1, after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable.

# Conclusion

Observation 1: Option 1(CSSF reduction by skipping measurement on particular CC(s)) covers the below case:

- CSSF for SCCs where neighbor cell measurement isn’t required, e.g., reducing NSCC_SSB.

And it doesn’t cover the below cases:

- CSSF on SCCs where neighbor cell measurement is required

- CSSF on SCCs for inter-frequency without gap.

Therefore, Option 1 enhancement only covers NSCC_SSB  number reduction.

Observation 2: Even though a CC, e.g., PCC/PSCC shares 50% of its searcher to all SCCs, a single SCC among them gains a very limited benefit, if the shared part is shared by all SCCs, especially when the number of configured SCCs is more than 2. It eventually leads to an inefficient solution.

Proposal 1: The solution of CSSF optimization is same to the below cases:

- A UE not operating with multiple-Rx based simultaneous L3 measurement

- A UE not capable of multiple-Rx based simultaneous L3 measurement

Proposal 2: Regarding UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF, we recommend including aspects 1, 2, and 3 in the scope.

Proposal 3: We suggest covering SA, EN-DC and NE-DC in the scope without deprioritization any of them.

Proposal 4: RAN4 to check whether the 3rd searcher only can be used for CSSF optimization when no gap is configured.

Proposal 5: When 3 searchers is supported,

- 1 searcher for PCC,

- 1 searcher for other SCCs and frequency layers following legacy equalling sharing;

- 1 additional searcher is used for the specific PCC/PSCC/SCCs based on NW indication.

NW indicates the cell list which applied the measurement by the additional searcher.

RAN4 to discuss the use limitation on this additional searcher.

Proposal 6: Measuring only one CC out of multiple CCs which all are configured with MO is a valid use case.

Proposal 7: NW indicates the particular CC(s) to be measured or the the particular CC(s) not to be measured, if multiple serving CCs are in the same band.

Proposal 8: In case of no NW configuration/indication, UE is allowed to determine the CC(s) to be measured, but the rules shall be studied further.

Proposal 9: When two searchers are applied, NW to indicate the candidate PCC/PSCC/SCC(s) or the frequency range of the PCC/PSCC/SCC(s) which applies a specified percentage X% of the searcher resource, the other SCC(s) together with PCC/PSCC share the other (1-X)% of the searcher resource. If no NW indication, UE follows the legacy searcher sharing rules.

Proposal 10: Measurement ratio is configurable by NW, the MeasGapSharingScheme in MG can be a start point.

Proposal 11: Agree on Option 4, RAN4 to introduce a new individual capability for CSSF reduction in R19.

delay the detailed capability design to the end of the core part.

Proposal 12: Agree on Proposal 1, after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2416853, WF for NR_RRM_Ph5, Apple