# 3GPP TSG-RAN WG4 Meeting #113	R4-2418909

Orlando, Florida, US, 18th – 22nd November, 2024

Source:				Ericsson

Title:					LS Reply to RAN2 on carrierBandwidth configuration for less-than-5MHz carriers

Agenda item:		5.8.1

Document for:		Approval

# Introduction

RAN2 sent a LS [4] to RAN4 highlighting RAN2 is discussing carrierBandwidth configuration in SIB1 and in RRC dedicated signaling. With this LS, RAN2 is asking for RAN4’s view on which value should be used to configure carrierBandwidth. This contribution is discussing this aspect, proposing a reply to RAN2 LS in annex.

# Discussion

## RAN2 LS

In the context of LessThan5MHzFR1, RAN2 sent a LS ([1]) to RAN4 with following description:

RAN2 seeks then RAN4’s opinion on the value that carrierBandwidth should get for 12 and 20RBs:

## Analysis

It should be noticed that 12 RBs and 20 RBs are exception cases, specified only for band n100 to help the Railway community to transition smoothly from GSM-R to FRMCS in that band. Those 2 specific cases have then been studied in that context only, based on the deployment constraints shared by UIC.

Also, to limit the impacts on RAN4 specifications, and considering those 2 cases are exception only, it was agreed to not specify any RF requirements for 12 and 20RBs (those are not configurations specified in the table 5.3.2-1 Maximum transmission bandwidth configuration in TS 38.101-1), assuming then they will always operate in respectively 3 and 5 MHz NR signal, i.e. the maximum transmission bandwidth configuration for 12 and 20RBs is still 15 and 25 RBs respectively.

Observation1: The maximum transmission bandwidth configuration for 12 and 20 RBs configuration is respectively 15 and 25 RBs.

Looking at RAN2 LS, the following statement is of importance:

According to TS 38.331, the carrierBandwidth shall be larger or equal to a “UE supported maximum transmission bandwidth”. Otherwise the cell would be considered as barred.

2> if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration

(see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which

- is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the

initial downlink BWP

…..

2> else:

3> consider the cell as barred in accordance with TS 38.304 [20]; and

If we don’t want the UE to be barred from the cell, the carrierBandwidth shall have a value greater or equal to the maximum transmission bandwidth configuration. This means that:

- For 12 RBs, the carrierBandwidth shall have value 15.

- For 20 RBs, the carrierBandwidth shall have value 25.

Observation2: According to TS 38.331, to not bar cell access, the carrierBandwidth shall have value 15 for 12 RBs and value 25 for 20 RBs.

This would then answer RAN2 LS and confirm their assumption:

RAN2 would like to understand from RAN4 perspective whether it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling, and if not what values should be configured.

Proposal1: Approve the LS reply to RAN2 in Annex.

# Conclusion

In this contribution, we discussed the RAN2 LS and proposed a reply in Annex.

Observation1: The maximum transmission bandwidth configuration for 12 and 20 RBs configuration is respectively 15 and 25 RBs.

Observation2: According to TS 38.331, to not bar cell access, the carrierBandwidth shall have value 15 for 12 RBs and value 25 for 20 RBs.

Proposal1: Approve the LS reply to RAN2 in Annex.

# Reference

- R2-2409409, LS on carrierBandwidth configuration for less-than-5MHz carriers, RAN2

# Annex: LS reply to RAN2 on carrierBandwidth configuration for less-than-5MHz carriers

# 3GPP TSG-RAN WG4 Meeting #113	R4-241xxxx

Orlando, Florida, US, 18th – 22nd November, 2024

Title:	LS Reply to RAN2 on carrierBandwidth configuration for less-than-5MHz carriers

Response to:	R2-2409409

Release:	Release 18

Work Item:	NR_FR1_lessthan_5MHz_BW

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:	TSG RAN WG1

Contact Person:

#### Name:		Dominique Everaere

- E-mail Address:	dominique.everaere@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN2 for their LS R2-2409409.

RAN4 further discussed which value should be considered for the carrierBandwidth, and this for the 2 specific cases 12 and 20 RBs transmission bandwidth configuration.

RAN4 concluded that carrierBandwdith shall be configured with the value 15 (for 12 RBs case) and with the value 25 (for the 20 RBs case) in SIB1 and in RRC dedicated signaling, this to not bar the cell access.

2. Actions:

To RAN WG2.

ACTION: 	RAN4 respectfully requests RAN2 to take into consideration the above information.

3. Date of Next TSG-RAN WG4 Meeting:

RAN4#114	from 2025-02-17	to 2025-02-21		Athens, Greece

RAN4#114bis	from 2025-04-07	to 2025-04-11		China