# 3GPP TSG-RAN WG4 Meeting #113	R4-2418912

Orlando, Florida, US, 18th – 22nd November, 2024

Source:			Ericsson

Title:			NTN Ku-band – VSAT RF

Agenda item:		7.9.4

Document for:		Discussion

# Introduction

In RAN#104, a new WI ([1]) has been approved to specify new NTN Ku band(s).

RAN4 had initial discussion in RAN4#112 and RAN4#112bis meetings and way forwards ([2] and ) has been agreed. This contribution discusses further the VSAT RF aspect, and more specifically the regulatory information for the agreed NTN KU-band for CEPT countries. Similar discussion should take place for the other countries where the Ku-bands will be defined, i.e. at least USA, Canada, Taiwan, Japan and Australia.

# Discussion on CEPT Regulation

In last RAN4#112 meeting, we shared our findings ([3]) about relevant ECC Decision (Table 1) that should be considered when specifying the new NTN Ku-band as agreed in last meeting ([4]).

The ECC Decision(18)05 should not be considered as this ECC Decision regulates ESIM operation with NGSO satellite, which is out of scope of this WI.

The ECC Decision(19)04 should only be considered when priority#2 ([1]) will be addressed.

Table 1: Relevant ECC Decisions to be considered for the NTN KU-band

| ECC Decision | Frequency Range | Title |
| --- | --- | --- |
| ECC Decision(18)04 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use, exemption from individual licensing and free circulation and use of land based Earth Stations In-Motion (ESIM) operating with GSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0- 14.5 GHz |
| ECC Decision(18)05 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use, exemption from individual licensing and free circulation and use of Earth Stations In-Motion (ESIM) operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz |
| ECC Decision(19)04 | 10.7 - 12.75 GHz (S-to-E) 12.75 – 13.25 GHz (E-to-S) | The harmonised use of spectrum, free circulation and use of earth stations on-board aircraft operating with GSO FSS networks and NGSO FSS systems in the frequency bands 12.75-13.25 GHz (Earth-to-space) and 10.7-12.75 GHz (space-to-Earth) |
| ECC Decision(05)10 | 10.7 – 11.7 GHz and 12.5 – 12.75 GHz (S-to-E) 14.0 - 14.5 GHz (E-to-S) | The free circulation and use of Earth Stations on board Vessels operating in fixed satellite service networks in the frequency bands 14-14.5 GHz |
| ECC Decision(17)04 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use and exemption from individual licensing of fixed earth stations operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz |
| ECC Decision(03)04 | 10.7 – 11.7 GHz (S-to-E) 14.25 – 14.5 GHz (E-to-S) | Exemption from Individual Licensing of Very Small Aperture Terminals (VSAT) operating in the frequency bands 14.25-14.50 GHz Earth-to-space and 10.70-11.70 GHz space-to-Earth |
| ECC Decision(06)03 | 10.7 – 12.75 GHz (S-to-E) 14.0 – 14.25 GHz (E-to-S) | Exemption from Individual Licensing of high e.i.r.p. satellite terminals (HEST) operating with geostationary satellites and in the frequency bands 10.70-12.75 GHz or 19.70-20.20 GHz space-to-Earth and 14.00-14.25 GHz or 29.50-30.00 GHz Earth-to-space |
| ERC Decision(00)08 | 10.7 – 12.5 GHz (S-to-E) | ERC Decision of 19 October 2000 on the use of the band 10.7 – 12.5 GHz by the fixed service and Earth stations of the broadcasting-satellite and fixed-satellite service (space-to-Earth) |

Analyzing the above ECC Decisions, the following technical requirements should be captured in TS 38.101-5 for the CEPT Ku-band priority#1:

- VSAT operating with GSO in the 14.25-14.5 GHz: 50dBW maximum EIRP, ECC Decision(03)04.

- Land ESIM operating in 14.25-14.5 GHz (when fixed services are deployed): maximum EIRP as below, , ECC Decision(18)04.

- Land ESIM operating in 14.47-14.5 GHz (when fixed services are deployed): maximum EIRP as below, , ECC Decision(18)04.

- Land ESIM, max EIRP of 54.5dBW, ECC Decision(18)04.

- Fixed Earth Station operating with NGSO in the 14.0-14.5 GHz: maximum EIRP of 60dBW, ECC Decision(17)04.

- Satellite terminals operating with GSO in the 14.0-14.25 GHz: max EIRP of 60dBW, ECC Decision(06)03.

- Earth station on board Vessels: minimum antenna size of 0.6m, ECC Decision(05)10.

The following Table 1 captures those regulatory technical requirements.

| NTN type | Frequency range | Technical requirement | Comment |
| --- | --- | --- | --- |
| NTN VSAT operating with GSO | 14.0 – 14.25 GHz | Max EIRP: 60 dBW |  |
| NTN VSAT operating with GSO | 14.25 – 14.5 GHz | Max EIRP: 50 dBW |  |
| NTN VSAT operating with GSO | 14.0 – 14.25 GHz | Max EIRP: 60 dBW |  |
| Mobile VSAT / Maritime ESIM | 14.0 – 14.5 GHz | Minimum antenna size: 0.6m |  |
| Mobile VSAT / Land ESIM | 14.0 – 14.5 GHz | Max EIRP: 54.5 dBW |  |
| Mobile VSAT / Land ESIM | 14.25-14.5 GHz |  | when fixed services are deployed |
| Mobile VSAT / Land ESIM | 14.47 – 14.5 GHz |  | when fixed services are deployed |

Table 1: Regulatory technical requirement in CEPT countries

Proposal: Agree to capture in TS 38.101-5 the requirements listed in Table 1 for the CEPT Ku-band.

Additional detailed analysis of the ETSI Harmonized Standards would also be needed to complete the regulatory and specifications context in CEPT countries.

Also, similar analysis should take place for the other countries where the Ku-bands will be defined, i.e. at least USA, Canada, Taiwan, Japan and Australia.

# Conclusion

In this contribution, we made an analysis of the ECC Decisions related to CEPT Ku-band. We made the following proposals and observations:

Proposal: Agree to capture in TS 38.101-5 the requirements listed in Table 1 for the CEPT Ku-band.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

- R4-2413525, Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF

- R4-2411190, NTN Ku-band – Regulatory aspects and band(s) definition, Ericsson

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General