3GPP TSG-RAN WG4 Meeting #113             	R4-2418939

Orlando, USA, 18th-22nd, November, 2024

Agenda item:	5.21.4.1

Source:	Samsung

Title:	Discussion on SNR requirement derivation for NTN enhancement

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the performance part of demodulation requirements for NTN enhancement was closed. The related requirement for each defined test cases were specified with [] in current specification.  However, TBD was kept for some of cases due to the ideal simulation results span among companies is larger than 2dB and there is no enough input after removing the outliers.

In this contribution, the view on how to derive SNR is provided for these cases.

# 2	Discussion

In the Rel-18 NTN enhancement WI, RAN4 has introduced requirement for PUSCH, PUCCH and PRACH requirement for above 10GHz in Ka-band, and also PUSCH with DMRS bundling requirement for coverage enhancement in NTN-FR1 S-band.

In the RAN#112 meeting, based on the latest simulation results summary in [1], the results for some of cases among companies are not well aligned. In order to derive the requirement, the rule to derive the requirement in Rel-15 was applied, where the results farthest from the AVER AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available.

After moving the outliers with at least 3 companies available, most of cases are aligned within 2 dB gap. Therefore, for those cases, the SNR with [] for requirement are captured in current specification.

Based on latest summary, only one case for PUCCH, after removing the outliers, there is no enough input.  Therefore, for this case, the TBD was kept for requirement in current specification, companies can still update the simulation results in the maintenance stage based on latest simulation assumption.

Therefore, RAN4 should discuss the rule for how to derive the requirement for those cases with ideal simulation results span large than 2dB, in case there is no enough input with well aligned and no update for the simulation results

For this case, we can discuss in case-by-case manner, either relaxing the span from 2dB to [2.5] dB, or additional marginal could be considered.

Based on current result, if removing the outliers, the span can be up to 2.2dB under the condition with at least 3 companies. Therefore, if relaxing the span to [2.5] dB, then, the requirement can be defined based on average impairment results provided by companies

If keeping the span value with 2dB, after removing the outliers, the results are divided into two camps and 2 companies are in each camp.  Then, additional margin can be considered, the half of difference of two camps average results

Proposal 1:  Discussion the rule for how to derive the requirement for those cases with ideal simulation results span large than 2dB, in case there is no enough input with well aligned. Either relaxing the span from 2dB to [2.5] dB, or additional marginal could be considered in case-by-case manner.

# 3	Conclusion

In this contribution, the view on SNR derivation of SAN demodulation requirements for NTN enhancement was provided.

Proposal 1:  Discussion the rule for how to derive the requirement for those cases with ideal simulation results span large than 2dB, in case there is no enough input with well aligned. Either relaxing the span from 2dB to [2.5] dB, or additional marginal could be considered in case-by-case manner.

# References

R4-2413566, Simulation results summary on demodulation requirements for NR NTN enhancement