3GPP TSG-RAN WG4 Meeting #113	R4-2418981

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	System parameters for LP-WUR

Agenda item:	7.23.2.1

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made

### Sub-topic 1-1 General Issue 1-1-1: draft skeleton for LP-WUS RF TR 38.774 Agreements: - The draft skeleton for TR 38.774 in R4-2415778 is endorsed. - Below is the TR drafting work splitting among interested companies: Section in TR; Title; Owner(s). General part; (scope, reference, definition, background); vivo. 5; System parameters; Ericsson, CATT. 6; Simulation assumptions and evaluation; CATT, CMCC, vivo. 7.1; UE RF; Huawei, Nokia, CMCC. 7.2; BS RF; ZTE, Ericsson. 8; Testability; Nokia, Samsung. Annex; Other aspects (if needed); vivo. Issue 1-1-2: Consideration on RAN1 LS R4-2414909 Agreements: - no specific RF work related to RAN1 LS. Issue 1-1-3: MDR value for RF requirements Agreements: - 1% MDR. Same for requirements and conformance testing. Issue 1-1-4: FAR assumption for MDR evaluation Agreements: - Use </=1% FAR as evaluation assumption for RF ### Sub-topic 1-2 System parameters Issue 1-2-1: Channel raster for LP-WUR Agreements: - Companies directly bring the specification impact if any and those will be discussed instead of whether a channel raster is required or not. Issue 1-2-2: considerations on LP-WUS allocation Agreements: - RAN4 to assume PRB grid alignment between LP-WUS and NR. - RAN4 to assume the LP-WUS RBs can be flexibly allocated within the wider NR carrier. Issue 1-2-3: number of RBs for LP-WUS with 15kHz SCS Agreements: - Follow RAN1 agreement for number of RBs for LP-WUS with 15KHz SCS, i.e., 11RBs.

Discussion

Channel raster for LP-WUR

In the last meeting, it was agreed that companies will directly bring the specification impact, if any, as there was no consensus on the applicability of channel raster for LP-WUR. Our understanding regarding LP-WUS and LP-SS is that it will be configured on common resource grid based on the SCS applied. Further, existing OFDM framework will be used to generate LP-WUS and LP-SS and they will be multiplexed with the existing NR transmission i.e., these signals will be transmitted on the common resource grid.

LP-WUS and LP-SS will be transmitted on common resource grid.

Currently, channel raster entries are defined per band. Bands which have channel raster of 100 kHz, an enhanced raster of 10 kHz is also defined. For bands with SCS based channel raster, no such enhanced raster is required. Proponents of new channel raster for LP-WUR have been proposing a new 10 kHz channel raster for NR bands with 100 kHz channel raster. This, we believe is already covered by the current specifications and LP-WUR can just reuse the same. So, in our opinion, nothing new must be done in the UE RF specification regarding channel raster.

No specification impact regarding channel raster.

Conclusion

The following Observations and Proposals were made:

Observation 1: LP-WUS and LP-SS will be transmitted on common resource grid.

Proposal 1: No specification impact regarding channel raster.

- References

- R4-2417112, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112bis, Hefei China, 14th – 28th Oct, 2024