3GPP TSG-RAN WG4 Meeting #113	R4-2418982

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Fundamental RX requirements for LP-WUR

Agenda item:	7.23.2.2

Document for:	Discussion

Introduction

In this document we present our views on open issues regarding REFSENS, ACS, and ASCS requirements. Following agreements were made in the last RAN4 meeting [1].

### Sub-topic 2-1 SNR simulation and values Issue 2-1-1: Alignment of FR1 SNR calculation/definition in simulation Agreements: - SNR is defined as the ratio of average energy of the LP-WUS signal per sub carrier over the average energy of Noise per sub carrier, over all the LP-WUS RBs. - Consider the sub-carriers allocated to LP-WUS excluding Guard RBs. Issue 2-1-2: FR1 SNR reference point Agreements: - same SNR reference point as MR Issue 2-1-3: Target FR1 SNR value for LP-WUS/WUR Agreements: - RAN4 can further discuss target SNR for OOK-based and OFDM-based receivers. Issue 2-1-5: Link level simulation assumption for FR2 LP-WUR Agreements: - RAN4 can further discuss simulation assumption for FR2 LP-WUS next meeting. ### Sub-topic 2-2 NF and REFSENS requirements Issue 2-2-1: Baseline architecture for FR1 OFDM-based LP-WUS Agreements: - use zero-IF receiver as a baseline RF architecture for OFDM based LP-WUR. Issue 2-2-3: REFSENS, NF, IM, SNR aspects for FR1 OOK-based and OFDM-based LR Agreements: - Collect input on REFSENS with values for each element（NF, IM, SNR） in the table next meeting. The value for each element can be discussed. - Encourage companies to also input the justification for each element. Issue 2-2-10: Baseline architecture for FR2 OOK-based and OFDM-based LP-WUR Agreements: - Companies to study baseline architecture for FR2 LP-WUS. ### Sub-topic 2-3 ASCS simulation and requirements Issue 2-3-1: ASCS requirements value Agreements: - Confirm ASCS RF requirements is needed for LP-WUS/WUR. Issue 2-3-2: Required number of guard RB for ASCS Agreements: - RAN4 further discuss required guard RBs for ASCS requirements. Issue 2-3-3: Whether Test case for ASCS should be defined in RAN4 Agreements: - Detailed test case for ASCS should be defined. ### 2.1.4 Sub-topic 2-4 ACS simulation and requirements Issue 2-4-1: Phase noise for simulation assumption Agreements: - Phase noise can be considered in LLS simulation analysis. - Phase noise used in R4-2415201, R4-2415780, and R4-2309204 could be considered as a reference. Other profiles are not precluded. - Number of guard RBs will be further discussed until Feb meeting. Encourage companies to provide simulation results with phase noise modelling. Issue 2-4-4: ACS requirements value Agreements: - FFS. Issue 2-4-5: Required guard RB for ACS requirements Agreements: - RAN4 further discuss the required number of guard RBs for ACS requirements Issue 2-4-7: Test parameters for LP-WUR ACS case Agreements: - Further discuss test parameters for LP-WUS ACS Issue 2-4-8: whether limit ADC bit for ACS requirements Agreements: - No limiting to a specific ADC bit for ACS requirements.

Discussion

NF value

A zero-IF receiver was agreed to be a baseline for both OOK and OFDM based LR. This architecture can have a high degree re-use of the NR radio like frontend, antennas, LNA, mixer, ADC (if it is configurable in frequency and sample depth) can be reused.

Figure 1 Basic UE LP-WUR architecture for utilizing a Zero-IF receiver.

Assumed values of the performance parameters of different components used in this architecture are shown in Table 1. These values are taken from published literature ([2]--[8]) and correspond to frequencies less than equal to 2.4GHz. As the direct conversion receiver folds the image spectrum, an image reject mixer is used. However, based on the frequency domain content of the signal, an acceptable solution can be designed without an image reject mixer. This however will work only for an envelope detector based receiver. An OFDM based detector will need an image reject mixer. Thus, further optimization can be done at the mixer. However, the overall NF level will only increase by around 0.1 dB. The NF of the whole receiver is being governed by the front-end filters and the LNA, which will be common for both OFDM based, and envelope-based receivers.

Table 1 Gain, and NF values for a reference Zero-IF based receiver for a sequence detector LP_WUR.

| Component | Gain (dB) | NF (dB) | Remarks |
| --- | --- | --- | --- |
| RF BPF/Duplex filter | -1.5 | 1.5 |  |
| LNA | 15 | 7 | Low power LNA with large NF. |
| Mixer | 3 | 10 |  |
| BB amplifier | 40 | 25 | BB amplifier providing the majority of required gain. |
| Total | 56.5 | 11.6 |  |

The NF of the receive chain is being governed by front-end components and LNA, which are common for both OFDM based, and envelope-based receivers.

Agree to use the estimated NF of 12dB as a baseline for both envelope and OFDM based LP_WUR for low and mid bands.

In the last meeting, arguments were raised regarding using different NF for different bands. It is true that NF performance degrades with increasing frequency, so a receiver operating at higher center frequency will have worse NF compared to LP-WUR operating on low and mid-bands. This phenomenon can be accommodated by adding a delta for bands with frequency greater than 2.4 GHz on top of the above agreed NF number.

Agree to increase the NF used for REFSENS calculation by a fixed delta value for bands with frequency greater than 2.4 GHz.

Implementation margin

For E-UTRA and NR in FR1, 2.5dB of IM had been used for the main receiver in case of 5 MHz channel bandwidth. However, for a LP-WUR there is no interference from its own transmitter as the transmitter should not be working because UE is in deep or ultra-deep sleep. Further, there is only a single receiver, thus no additional losses which can be attributed to multiple receivers.

Some companies have concerns regarding antenna sharing architecture and its impact on NF, and thus want to increase the IM. Our view is that it should be accounted under NF as that is the more appropriate place and not IM.

In our view, IM covers impairments related to BB artifacts, non-linearities, and channel estimation errors that are not accounted for anywhere else. Thus, we propose to tighten the IM to a value less than 2.5dB.

There is no self-desensitization because of own transmission for LP-WUR.

There are no multiple receivers associated losses for LP-WUR.

Agree to have implementation margin to be 1dB for both OOK and OFDM based receivers.

Additional impact on NF because of antenna sharing architecture should be accounted for in NF, not IM.

ASCS requirements

Link level simulations were performed with both AWGN and TDL-C 300 channel including the phase noise profile from Wi-Fi IEEE802.11-17/0326r0 [2] and R4-2415780 [3]. The simulated performance for OOK4 M=4 modulation in both AWGN channel and TCL-C300 with ideal LO and with phase noise profile are illustrated in Figure 2. Based on the simulations, there is an approximately 1 dB of degradation in sensitivity level at MDR=1% for both AWGN channel and for the TDL-C300 channel.

Figure 2 ASCS performance with ideal LO (two lower figures) and with Phase noise profile from IEEE802.11-17/0326r0.

The WiFi phase noise profile [2] degrade the ASCS performance with app. 1 dB.

ACS Requirements

ACS requirement value is linked with ACS guard RBs and there is a trade-off between the requirement and guard RBs. Further, the limited dynamic range of the LP-WUR will have an impact on the detection of LP-WUS in presence of an interferer.

In Figure 3 is the ACS performance is simulated with an ideal LO and one example phase noise profile from WiFi IEEE802.11-17/0326r0 (slide 7) [2]. The simulation is done with OOK4 M=4 and the adjacent carrier is 31.5dB higher compared to own signal. As it can be seen from the figures, there is more than 10 dB degradation in performance for both AWGN and TDL-C300 channel profile. However, this is just one reference profile and performance will be different with different profiles. So, to arrive at a set of results which can be compared, the group should agree to a common phase noise profile.

Figure 3 ACS performance with ideal LO (two lower figures) and with Phase noise profile from IEEE802.11-17/0326r0.

Agree to use common phase noise profile for simulations in order to agree derive the requirements.

Conclusion

The following Observations and Proposals were made:

Observation 1: The NF of the receive chain is being governed by front-end components and LNA, which are common for both OFDM based, and envelope-based receivers.

Proposal 1: Agree to use the estimated NF of 12dB as a baseline for both envelope and OFDM based LP_WUR for low and mid bands.

Proposal 2: Agree to increase the NF used for REFSENS calculation by a fixed delta value for bands with frequency greater than 2.4 GHz.

Observation 2: There is no self-desensitization because of own transmission for LP-WUR.

Observation 3: There are no multiple receivers associated losses for LP-WUR.

Proposal 3: Agree to have implementation margin to be 1dB for both OOK and OFDM based receivers.

Proposal 4: Additional impact on NF because of antenna sharing architecture should be accounted for in NF, not IM.

Observation 4: The WiFi phase noise profile [2] degrade the ASCS performance with app. 1 dB.

Proposal 5: Agree to use common phase noise profile for simulations in order to agree derive the requirements.

- References

- R4-2417112, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112bis, Hefei China, 14th – 28th Oct, 2024,

- IEEE802.11-17/0326r0, WUR Phase Noise Model Follow-Up.