3GPP TSG-RAN WG4 Meeting #113	R4-2418985

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	RedCap NTN UEs

Agenda item:	7.26.2.1

Document for:	Discussion

Introduction

In this paper, we provide our views on the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands. RedCap specific objectives mentioned in the WID [1] are copied below for ease of reading.

Objectives:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In the last RAN4 meeting, the following agreements were made [1].

## Issue 1-1: PC3 output power for HD-FDD (e)RedCap Agreement: Nominal power for PC3 is 23 dBm. ## Issue 1-2: HD-FDD refsens for 2 Rx Agreement: Agree n256 Refsens as below. Operating Band; SCS (kHz); 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm). 15; -100.0; -96.8; -95.0; -93.8. n256; 30; -97.1; -95.1; -94.0. 60; -97.5; -95.4; -94.2. ## Issue 1-4: Specification updates Agreement: - The maximum transmission bandwidth for NTN eRedCap with bandwidth reduction should be 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths. - REFSENS of NTN eRedCap with bandwidth reduction for wider channel BW in FR1-NTN bands, it should be defined as below: - The 5MHz REFSENS requirements are applicable for all the channel bandwidths including 5MHz, 10MHz, 15MHz, 20MHz channel bandwidth. - 25 contiguous RB placed in middle of channel BW both in UL and DL for both HD-FDD and FD-FDD, where both UL and DL allocations are following the 5MHz NTN RedCap REFSENS configurations and located at the center of the channel BW. - UL RB size follow the 5MHz UL configuration table - Reuse the same scaling factor of NTN (e)RedCap without bandwidth reduction for REFSENS for NTN eRedCap with bandwidth reduction with 2Rx HD-FDD, 1Rx HD-FDD and 1Rx FD-FDD. - The bandwidth reduction in TS 38.101-1 5.3I applies also for NTN (e)RedCap ## Issue 1-3: Simultaneous operation with GNSS Open items: - Strive towards common solution for (e)RedCap NR NTN and non-RedCap NR NTN - Consider asking RAN1/2 to ensure IDC framework allows appropriate configurations to ensure GNSS location acquisition and NR NTN can operate in parallel in bands with UL close to GNSS frequencies - Encourage further evaluations in RAN4 including but not limited to - Whether potential new solution(s) apply only from rel-19 or are early implementable - Whether UE speed needs to be considered

Discussion

HD-FDD REFSENS for two RX

In the last meeting, 0.5 dB tightening was agreed for n256, however, the tightening amount for n254 and n255 are still under discussion. Companies have proposed to use 0.5dB for n254 and 0dB for n255 based on the tightening rules that were used for TN RedCap HD-FDD REFSENS [3]. Table 1 illustrates the rules for reference.

Table 1 Proposed REFSENS tightening rules in TN.

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

The reasoning for 0.5 dB goes back to almost 10 years old limited survey on insertion loss difference between a SAW RF filter vs SAW duplex filter. As SAW RF filters have less insertion loss compared to SAW duplex filters and since HD-FDD implementations do not need duplex filters, REFSENS can be tightened by 0.5 dB. Further, the reasoning for 0dB was based on REFSENS of TDD bands.

Reasoning for 0.5 dB improvement goes back to almost 10 years old survey on insertion loss differences.

Reasoning for 0 dB tightening was related to REFSESN of TDD bands.

We did a limited survey of duplexers, RF filters, and TRX switches for n255 and n254, and noticed that the insertion loss difference ranges from 1.2 dB to 2 dB. This observation supports the 0.8 dB number that was proposed under more recent discussions done under Release 17 RedCap for HD-FDD case. Given that lot of improvement in device technology has happened over last 10 years, it is possible to leverage that improvement for improving the REFSENS.

Recent survey on RF components showed that insertion loss difference ranges from 1.2dB to 2dB.

Further, there are no TDD NTN bands, so utilizing TDD bands as an excuse for inaction hinders overall system efficiency and improvements.

There are no TDD bands in NTN, so sticking to 0dB tightening is unjustified.

Utilize 0.8dB of tightening for both n255 and n254.

Simultaneous operation with GNSS

In the previous meeting, a discussion was raised about the operation in parallel of GNSS with NR NTN, for bands n254 and n255. While the work item assumption is that a RedCap UE shall be capable to operate both concomitantly, there are concerns about some GNSS frequencies operating close to these bands and the absence of previous studies on this issue [2].  The issue is related to the following frequencies:

Table 2. Summary of frequencies potentially impacted by co-existence

| Channel | Band |
| --- | --- |
| n254 (uplink) | 1610-1626.5 MHz |
| n255 (downlink) | 1525 MHz – 1559 MHz |
| GPS Band L1 | 1567.5 MHz -1583 MHz |
| GLONASS Band G1 | 1589.0625 MHz - 1605.375 MHz. |
| BeiDou B1C | 1575.42 MHz |

It is clear that the impact is more stringent related to the GLONASS Band G1, which is very closely located to the UL of band n254, which might represent that the own transmission of the UE in UL might be seen as practical jammer to the reception of GLONASS on said frequency.

However, there are still GNSS bands available that are not suffering from this issue (for example GLONASS bands G2 and G3 and GPS band L1).  This means that depending on UE implementation, and the GNSS signals (and frequencies) available in a given location, not all UEs will be harmed by this collision all the time.

Not all UEs will be harmed by the potential interference between UL and GNSS frequencies.

Moreover, the issue is obviously more pronounced when the UE is actively performing UL transmissions. It is still unclear whether a short burst of UL transmissions is sufficient to jam the GNSS signal in such frequencies or whether long UL transmissions are necessary to cause the faulty condition.

The issue is related to NR NTN UL transmission and not DL reception on the UE side and might be dependent on the duration of the UL transmission.

The discussion above, with the respective observations is needed to clarify exactly what the problem is. The specification effort might strive to find a solution that can be applicable only for certain UEs (potentially UE triggered) and that do not interfere with DL receptions. In the absence of previous study in the specification efforts, the impact of short UL transmissions might also be left for UE to estimate and apply corrective action. In other words, we would favor a solution that allows for in-device coexistence instead of the introduction of a new measurement gap for GNSS (as needed for NB-IoT devices).  Besides, GNSS Measurement gaps have considerable impact on RRM cell measurements.

IDC solutions are favored over GNSS measurement gaps to address this issue.

Based on this discussion we are ready to comment on the solutions provided in the previous WF:

Issue 1-3: Simultaneous operation with GNSS Open items: - Strive towards common solution for (e)RedCap NR NTN and non-RedCap NR NTN - Consider asking RAN1/2 to ensure IDC framework allows appropriate configurations to ensure GNSS location acquisition and NR NTN can operate in parallel in bands with UL close to GNSS frequencies - Encourage further evaluations in RAN4 including but not limited to - Whether potential new solution(s) apply only from rel-19 or are early implementable - Whether UE speed needs to be considered

We agree that this problem is not related only to RedCap UEs, but also NR NTN UEs that operate in the same bands, and therefore we shall strive for a common harmonized solution. As a principle, we should discuss a solution that is applicable for RedCap UE but is not restricted for this type of UEs.  Regarding the applicability of said solution for previous releases (before Rel-19), we see it as potentially good, but not the end-goal of the solution to be designed. It is more important to have a good solution in place (if RAN1/2 agree that a new solution needs to be designed) than potentially have a “compromised, handicapped” solution due to concerns with backwards compatibility.

Agree to have a common harmonized solution applicable to both RedCap and non-RedCap NR NTN UEs.

Finally, for the issue on considering UE speeds for the final solution, we agree that UEs operating in higher velocity, need to update their GNSS position more frequently to avoid compromise the timing requirements. Even though in the past some companies have raised that static UEs do not need to update their position, they are still expected to do so.

Static UEs do not know they are static and are required to update their positions to establish whether they have moved or not and update timing and frequency requirements accordingly.

In this sense, the problem affects static and mobile UEs alike, with the consideration that a UE in high speed might need some more frequent GNSS updates. As a compromise, we might inform to RAN1/2 that their derived solutions might be applicable for both static and mobile UEs alike.

UE speed should not be a factor impacting the IDC solution.

Agreed IDC solution should be applicable to both static and mobile UEs equally.

Agree to the attached LS provided in the Annex.

Conclusion

The following Observations and Proposals were made:

Observation 1: Reasoning for 0.5 dB improvement goes back to almost 10 years old survey on insertion loss differences.

Observation 2: Reasoning for 0 dB tightening was related to REFSESN of TDD bands.

Observation 3: Recent survey on RF components showed that insertion loss difference ranges from 1.2dB to 2dB.

Observation 4: There are no TDD bands in NTN, so sticking to 0dB tightening is unjustified.

Proposal 1: Utilize 0.8dB of tightening for both n255 and n254.

Observation 5: Not all UEs will be harmed by the potential interference between UL and GNSS frequencies.

Observation 6: The issue is related to NR NTN UL transmission and not DL reception on the UE side and might be dependent on the duration of the UL transmission.

Observation 7: IDC solutions are favored over GNSS measurement gaps to address this issue.

Proposal 2: Agree to have a common harmonized solution applicable to both RedCap and non-RedCap NR NTN UEs.

Observation 8: Static UEs do not know they are static and are required to update their positions to establish whether they have moved or not and update timing and frequency requirements accordingly.

Proposal 3: UE speed should not be a factor impacting the IDC solution.

Proposal 4: Agreed IDC solution should be applicable to both static and mobile UEs equally.

Proposal 5: Agree to the attached LS provided in the Annex.

- References

- R4-2416586, Way Forward for [112][310] NR_NTN_Ph3_UE_RF, Qualcomm Inc., RAN4#112bis, Hefei, China, October 14 – October 18, 2024.

- R4-2416462, “UE RF requirements for NR NTN RedCap”, Qualcomm Inc., ., RAN4#112bis, Hefei, China, October 14 – October 18, 2024.

- Annex

Title:	draft LS on NR NTN co-existence with GNSS

Response to:

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN WG4

To:	RAN WG1, RAN WG2

Cc:	-

Contact person:

Name:	Alok Sethi

E-mail Address:	alok.sethi@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 has identified issues regarding simultaneous operation of NR RedCap UEs with GNSS. The issues in part stem from the frequency plan of NR bands n255 and n254, where their physical proximity to the some GNSS carriers can lead to a compromised performance. To avoid any field performance issues both for NR NTN RedCap as well as NR NTN non-RedCap UEs, a reliable in-device co-existence (IDC) solution needs to be supported in real deployments. It is RAN4 understanding that such solutions should equally apply to NR NTN RedCap/eRedCap, and NR NTN devices and be applicable at least from Rel19 onwards. The IDC solution should be valid for both mobile and static UEs.

RAN4 therefore respectfully asks RAN2 and RAN1 to ensure that NR NTN UEs can rely on an in-device co-existence solution for GNSS co-existence.

# Actions

ACTIONS:

To RAN WG2 and WG1: RAN4 respectfully asks RAN2 and RAN1 to ensure that NR NTN UEs can rely on an in-device co-existence solution for NR coexistence with GNSS, as described above.

# Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #114	Feb 17 - 21, 2025	Athens, Greece

TSG-RAN4 Meeting #114bis	Apr 7 - 11, 2025	China