3GPP TSG-RAN WG4 Meeting #112bis	R4-2418994

Hefei, China, October 14th-18th 2024

Agenda Item:	7.2.3

Source:	Ericsson

Title:	TP to TR 38.922 Updates to Clause 5

Document for:	pCR

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz.

In the previous 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, the estimated date for completion is latest by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, the estimated date for completion is latest by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

This text proposal provides editorial updates to Clause 5.

# 2	TP

<Start of TP>

# 5	7125 - 8400 MHz frequency range

#### 5.2.1.5	Maximum output power

The maximum base station output power/ sector (e.i.r.p.) was provided in the antenna parameter table of the LS [4], as extracted in Table 5.2.1.5-1. It was agreed to be aligned with antenna characteristics.

Table 5.2.1.5-1: Maximum BS output power in 7125 to 8400 MHz

|  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban |
| --- | --- | --- | --- |
| Maximum base station output power/sector (e.i.r.p.) (dBm) | 78.3 | 78.3 | 61.5 |

The Total Radiated Power (TRP) for two polarizations was agreed as shown in Table 5.2.1.5-2 below.

Table 5.2.1.5-2: Total Radiated Power

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 46 | 46 | 37 |

#### 5.2.1.6	Average output power

It was agreed the average output power won’t be mentioned in the reply LS in [5].

### 5.2.2	Receiver characteristics

#### 5.2.2.1	Noise figure

The BS typical noise figure is listed in Table 5.2.2.1-1.

Table 4.2.2.1-1: Noise figure

| BS class | Noise figure (dB) |
| --- | --- |
| Wide Area | 6 |
| Medium Range | 11 |
| Local Area | 14 |

<Next change>

## 5.3	UE parameters

### 5.3.1	Transmitter characteristics

#### 5.3.1.1	Power dynamic range

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. For existing FR1 bands, the minimum output power is -33 dBm for 100 MHz channel bandwidth. The minimum output power can be reused for 7.125 – 8.4 GHz, i.e. power dynamic range is 56 dB for 100 MHz channel bandwidth with power class 3 (i.e. 23 dBm maximum output power) UE.

<Next change>

#### 5.3.1.6	Average output power

This parameter was not mentioned in the previous response to ITU-R WP5D in [5].

<Next change>

## 5.4	Antenna characteristics

### 5.4.1	BS antenna characteristics

#### 5.4.1.1		Antenna model

The BS antenna model is described in subclause 7.1.

#### 5.4.1.2	Antenna parameters

|  | -- |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | dv,sub |  |  |  |  |

θetiltφescan

The BS antenna parameters are listed in Table 5.4.1.2-1.

Table 5.4.1.2-1: IMT parameters relevant for 7125 to 8400 MHz

| Parameter | Macro suburban | Macro urban | Micro urban |
| --- | --- | --- | --- |
| Am | 30 dB | 30 dB | 30 dB |
| SLAv | 30 dB | 30 dB | 30 dB |
| 3dB | 90 deg. | 90 deg. | 90 deg. |
| 3dB | 65 deg. | 65 deg. | 65 deg. |
| GE,max | 6.4 dBi | 6.4 dBi | 6.4 dBi |
| Msub | 3 | 3 | N/A |
| dv,sub | 0.7l m | 0.7l m | N/A |
| subtilt | 3 deg. | 3 deg. | N/A |
| M | 8 | 8 | 8 |
| N | 16 | 16 | 8 |
| dh | 0.5l m | 0.5l m | 0.5l m |
| dv | 2.1l m | 2.1l m | 0.7l m |
| etilt | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤120 deg. |
| escan | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. |
| r | 1 | 1 | 1 |
| Ptx | 46 dBm | 46 dBm | 37 dBm |
| mech | 6 deg. | 6 deg. | N/A |

### 5.4.2	UE antenna characteristics

The outcome of the RAN WG4 study in TR 38.820 [x] for collecting technical background information relevant for the frequency range 7 to 24 GHz indicated that the frequency range 7.125 - [10-13] GHz would have "FR1 like" requirements. Therefore, the UE is expected to have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming.

<End of TP>

# References

- R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

- R4-2405874, “Work Plan on IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz frequency ranges”, 3GPP RAN4#110-bis, Ericsson

- 3GPP TR 38.820 v16.1.0 (2021-03), “Study on 7-24 GHz frequency range (Release 16)”.