# 3GPP TSG-RAN WG4 Meeting #113	R4-2419001

# Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5152 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_MG_enh2-Core) CR on 38.133 MG enh2 on NCSGs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In RAN4 #111 meeting, UE performs deactivated SCell measurement within NSCG are agreed as following. - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association] - For UE configured with 2 NCSG, deactivated SCells are measured with NCSG - If the association is provided, deactivated SCells are measured with NCSG according to gap association. - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the MUSIM gap related requirement is based on NW configured MUSIM gaps. - Update the wording |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec. is unclear and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.13.2, 9.1.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Re-submit the CR R4-2412502. |  |  |  |  |  |  |  |  |

----------------------NEXT CHANGE---------------------------

#### 9.1.13.2	Requirements

If the UE requires concurrent measurement gaps and/or NCSG to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports [concurrentNCSGPerUE-OnlyMeasGapwithNCSG-r18] but does not support independent measurement gap patterns for different frequency ranges as specified in [14], in order for the requirements in the following clauses to apply, the network can provide one per-UE concurrent measurement gap and one per-UE NCSG or at most two per-UE NCSGs for monitoring of all frequency layers.

If the UE requires concurrent measurement gaps and/or NCSG to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports [concurrentNCSGPerUE-PerFRCombMeasGapwithNCSG-r18] as specified in [14], in order for the requirements defined for concurrent measurement gaps with NCSG to apply, the network can provide the concurrent measurement gap with NCSG combinations configurations specified in Table 9.1.13-1 for monitoring of all frequency layers.

Table 9.1.13-1: The number of Gap Combination Configurations by UE supporting both concurrent measurement gap with NCSG patterns, per-FR NCSG patterns and independent measurement gap patterns

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 [measurement gap] | Per-FR2 [measurement gap] | Per-UE [measurement gap] |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| 6 | 2 | 0 | 0 |
| 7 | 0 | 2 | 0 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap with NCSG is concurrent MG (and cannot be NCSG) is associated to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP measurement defined in TS 38.215 [4], and when the per-FR measurement gap with NCSG in an FR is NCSG. [Note 2: In Gap Combination Configuration Id #0, #1, #6, #7, one per-FR measurement gap in an FR (and cannot be NCSG) can be associated to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP measurement defined in TS 38.215 [4] provided that UE supports independentGapConfigPRS-r17.] Note 3: In Gap Combination Configuration Id #0, #1, #2, #6, #7, one FR can be configured with up to 2 NCSGs, regardless they are per-UE or per-FR configured. Otherwise, the gaps can only be configured as Gap(s) configured via GapConfig without suffix or Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17. |  |  |  |

For UE configured in the SA operation mode, when monitoring of multiple inter-RAT E-UTRAN carrier frequency layers and inter-frequency NR carrier frequency layers as configured by PCell using gaps, each monitored carrier frequency layer, including following measurement types:

-	a measurement object with SSB based measurement,

-	a measurement object with CSI-RS based measurement,

-	E-UTRA inter-RAT measurement object,

can be associated to either one concurrent measurement gap pattern or one NCSG pattern, while the following measurement types:

-	E-UTRAN inter-RAT RSTD measurement,

-	NR PRS-based positioning measurement,

can be only associated to one measurement gap pattern. Requirements for [concurrent measurement gaps with NCSG] apply provided that each frequency layer is only associated with one concurrent measurement gap or one NCSG, and at least one of the gaps is NCSG. There can be one or more frequency layers associated with each concurrent measurement gap or each NCSG. Furthermore, if the UE is not capable of [concurrentMeasGapEUTRA-r17][2], all E-UTRAN measurement objects shall be associated with a single concurrent measurement gap or NCSG for the requirement to apply.

When UE supports concurrent measurement gap with NCSG, where at least one of the concurrent gaps is NCSG, supported concurrent measurement gap patterns are listed in Table 9.1.2-1 based on the applicability specified in table 9.1.2-3, while supported NCSG patterns are listed in Table 9.1.9.3-1 based on the applicability specified in table 9.1.9.3-2.

The requirements in clause 9.1.2 are also applicable for the UE capable of and configured with multiple [concurrent measurement gap with NCSG] patterns within each concurrent measurement gap pattern. The requirements in clause 9.1.9 are also applicable for the UE capable of and configured with multiple [concurrent measurement gap with NCSG] patterns within each NCSG pattern.

When the measurement of the measurement object is related to a deactivated SCell, and

when NW configures the concurrent measurement gap and NCSG,

-	UE is expected to conduct the measurement of this measurement object based on the configured association if the deactivated SCell is activated;

-	UE is expected to conduct the measurement of this measurement object within the NCSG if the measurement object corresponding to a deactivated Scell.

when NW configures two NCSGs, UE is expected to conduct the measurement of this measurement object based on the configured association. If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case.

The requirements in clause 9.1.2 are also applicable for the UE capable of and configured with multiple [concurrent measurement gap with NCSG] patterns within each concurrent measurement gap pattern. The requirements in clause 9.1.9 are also applicable for the UE capable of and configured with multiple [concurrent measurement gap with NCSG] patterns within each NCSG pattern.

#### 9.1.13.3	Collision involving NCSGs

Collisions between occasions of concurrent measurement gap and NCSG or of two NCSGs may occur as specified in this clause if the two occasions are

-	two per-UE NCSGs, or

-	two per-FR NCSGs in the same FR, or

-	one per-UE NCSG and one per-UE measurement gap, or

-	one per-FR NCSG and one per-UE measurement gap, or

-    one per-UE NCSG and one per-FR measurement gap, or

-	one per-FR NCSG and one per-FR measurement gap in the same FR.

and if the gap collision condition specified in clause 9.1.8.3 is met then the gap collision rule applies.

When the first occasion is NCSG, the ending point of the first occasion is the end of VIL2 and/or when the second occasion is NCSG, the starting point of the second occasion is the start of VIL1.The requirements with [concurrent measurement gaps with NCSG] apply provided that two gaps (at least one of the gaps is NCSG) colliding with each other are configured with different priorities.

--------------------END OF CHANGES--------------------------