# 3GPP TSG-RAN WG4 Meeting #113	R4-2419008

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.22.2.1

Source: 	Ericsson

Title:	Discussion on OD-SSB in Rel-19 NES

Document for:	Discussion

# Introduction

In RAN Penlary #102, enhancements of network energy savings for NRError! Reference source not found. are introduced. In last RP meeting, the WID (mainly in Obj. 2) is updated with the following objectives.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we continue to discuss on the topic on OD-SSB operation of Rel-19 NES WI from RAN4 perspective.

- On-demand SSB(OD-SSB)

  - General

RAN1 LS

In last meeting, RAN4 had some discussions on OD-SSB transmission and UE processing timeline in RAN1 LS, but no conclusion is achieved. In last RAN1 meeting, some latest updates related to the OD-SSB tansmission is captured as agreement below.

RAN1 has made the following working assumption that the value of T_min for MAC-CE based OD-SSB indication is the same value as UE’s MAC-CE processing time for SCell activation command. RAN1 would like to ask RAN4 if the working assumption of T_min=. can be confirmed to agreement. RAN1 #118bis Agreement (Note: Delete the redundant wording) The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s)is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index]ofwithinthe first “possible” on-demand SSB burst which is at leastT slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min.

In legacy SCell activation, RAN4 followed the same principle of T_min to handle the SCell activation requirement as RAN1.

### 8.3.2 SCell Activation Delay Requirement for Deactivated SCell … Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. … TFirstSSB: is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length.

Observation 1: In legacy SCell activation, RAN4 followed the same RAN1 principle to apply SCell activation requirement based on slot n + THARQ+3msNR slot length.

The procedure for UE to activate the OD-SSB based SCell activation is similar as legacy SCell activation. UE will first parse the SCell activation and OD-SSB MAC-CE command and transmit the HARQ feedback. After that, UE will try to receive the first OD-SSB to complete the OD-SSB based SCell activation. Thus, RAN4 can confirm RAN1’s working assumption that T_min equals to m+3Nslotsubframe,μ+1.

Observation 2: The first steps of OD-SSB based SCell activation procedure is similar as legacy SCell activation.

Proposal 1: RAN4 to send LS to confirm that T_min equals to m+3Nslotsubframe,μ+1.

In last meeting, UE vendors had concern on additional UE processing time for OD-SSB. We think such UE processing time can be further discussed in RAN4 internally since RAN1’s question is about when NW can transmit the OD-SSB other than UE processing such OD-SSB.

Proposal 2: RAN4 can further discuss the possible UE processing time for OD-SSB reception within RAN4 internally.

L1 measurement for deactivated Scell

In last meeting, there is an open issue about how to understand RAN1 agreement for L1 measurement.

Issue 1-1-4: L1 measurement for deactivated SCell - Proposals - Option 1: Xiaomi, Samsung, China Telecom, Intel - RAN4 shall define both L1 and L3 measurement requirements based on OD-SSB. - Option 2: Qualcomm, OPPO, Nokia - L1 measurements are not supported for deactivated SCell - Option 3: Nokia - Wait further RAN1 clarification on the applicable scenario - Recommended WF - Notes: - From moderator’s understanding, RAN1’s agreement is applied to the L1 measurement after receiving SCell activation command other than before SCell activation. - RAN4 only needs to clarify the L1 related requirement can also applied to OD-SSB in SCell. - Suggest to further discuss the scenarios below. The proponent companies need to clarify which of the following scenarios are supported by your proposal. - L1 measurement before SCell activation: - L1 measurement during SCell activation: - L1 measurement after SCell activation:

In our understanding, RAN1’s agreement is applied to the L1 measurement after receiving SCell activation command other than before SCell activation. There is no such L1 measurement mechanism for deactivated Scell today. Thus, we don’t support such L1 measurement for deactivated Scell, but it’s fine to define OD-SSB based L1 measurement during unknown Scell activation and after the Scell activated.

Proposal 3: L1 measurements are not supported for deactivated Scell. Instead, RAN4 can define OD-SSB based L1 measurement during unknown Scell activation or after the Scell activated.

OD-SSB measurement accuracy requirement

There is another open issue about OD-SSB measurement accuracy. In our understanding, OD-SSB is a specific type of SSB. It doesn’t change any SSB physical design, such as the sequence, componenets. Thus, we don’t think any new measurement accuracy requirement is needed for such OD-SSB measurement.

Issue 1-1-6: OD-SSB accuracy - Proposals - Option 1: Qualcomm - No new measurement accuracy requirements should be defined. - Recommended WF - Moderator suggests the group to check whether the following proposal can be agreed. - No new measurement accuracy requirements should be defined.

Proposal 4: Legacy measurement accuracy requirements should also apply for OD-SSB.

  - OD-SSB based deactivated SCell measurement

    - Cell identification componenet

In last meeting, there is an open issue about whether UE needs to always perform cell identification to obtain coarse timing before deactivated Scell activation. The motivation to always require additional cell indetification is due to OD-SSB is not always transmitted.

Issue 2-1-6: OD-SSB based deactivated SCell measurement components - Proposals - Option 1: vivo - RAN4 to clarify the deactivated SCell measurement components includes both cell identification and measurement.

However, we think at least for the following cases UE doesn’t need to always perform cell identification. Firstly, for always-on SSB transmitted in case 2 means UE has performed the cell identification based on always-on SSB and doesn’t need to perform cell identification when OD-SSB is indicated. Secondly, in Case 1, if the OD-SSB SCell being measured is contiguous to an active serving cell in the same band, UE can refer on the active serving cell’s timing as legacy. No cell identification is needed. Thirdly, if UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is less than Xs and then the OD-SSB is transmitted again, cell identification is also unnecessary.

Proposal 5: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

- Case 1: No always-on SSB on the cell

  - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

  - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [X]s and then the OD-SSB is transmitted again with the same spatial reception parameter

- Case 2: Always-on SSB on the cell

    - Case 1: No always-on SSB on the cell

In last meeting, RAN4 made the following agreements about OD-SSB based deactivated SCell measurement in Case 1. The OD-SSB based fast L3 measurement framework has agreed, but the details are still FFS.

Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

OD-SSB based fast L3 measurement

The first issue is about the fast L3 measurement reqruiement. To maximize the NES gain, NW is expected to extend the muting duration before SCell activation as much as possible. When NW indicates the OD-SSB transmission for deactivated Scell measurement, UE shall follow the OD-SSB periodicity to perform measurement. At the same time, considering the measurement scheduling is a semi-static algorithm in UE side, it’s fine to allow UE to follow the same CSSF as legacy and not further speed up for multiple cells’ scenario. The OD-SSB based deactivated Scell measurement delay is Ceil(5 x Kp) x OD-SSB periodicity x CSSF.

Proposal 6: The OD-SSB based fast L3 measurement for deactivated Scell should be as follow.

- The measurement is based on OD-SSB periodicity

- CSSF doesn’t need to be changed

- The delay requriement is Ceil(5 x Kp) x OD-SSB periodicity x CSSF

Time window

In our understanding, when UE finished the first deactivated Scell measurement and reporting, UE is allowed to fallback to the legacy deactivated measurement considering the UE power issue. If NW decides to activate the Scell fast, NW can indicate the Scell activation command immediately when NW receives the deactivated measurement report. The time window can be defined from UE performing OD-SSB based fast measurement until the first time UE reporting the target OD-SSB based de-activated Scell measurement results. Once the NW deactivates the OD-SSB before the time window expires, UE is not required to measure the deactivated Scell.

Proposal 7: The time window for fast L3 measurement for deactivated SCell is defined starting from UE receiving OD-SSB indication during deactivated Scell stage until the first time UE reports the target OD-SSB based de-activated Scell measurement result.

UE fallback slow mode

After UE finishes OD-SSB based fast L3 measurement for deactivated Scell, UE is allowed to fallback to legacy deactivated Scell measurement which can be believed as the same as legacy procedure until NW deactivates the OD-SSB.

Proposal 8: UE is assumed to fallback to legacy deactivated Scell measurement based on measCycleSCell until NW deactivates the OD-SSB.

    - Case 2: With always-on SSB on the cell

In last meeting, RAN4 sent an LS to RAN1 to clarify the sceanarios between always-on SSB and OD-SSB.

Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation - Background From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible based on RAN1’s current agreements. - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different frequencies. Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

At the same time, RAN4 discussed the following issue without any conclusion.

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreements: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE) - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO) - Option 3: RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE) - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm) - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

In Case 2, NW transmits the legacy always-on SSB together with the OD-SSB as follows: (Green SSB is legacy always-on SSB, Orange SSB is the OD-SSB). Before NW indicating the OD-SSB transmission, UE is assumed to use legacy always-on SSB to perform deactivated SCell measurement. UE can combine both OD-SSB and always-on SSB to perform deactivated SCell measurement once UE receiving the OD-SSB transmission indication.

Figure 1. OD-SSB based deactivated SCell measurement and reporting(Case 2)

Observation 3: UE can combine the measurement results if the always-on SSB and OD-SSB are on the same frequency with the same spatial relation.

Proposal 9: In Case 2, the measurement requirement is defined assuming combining the legacy SSB and OD-SSB when OD-SSB transmission is indicated if the always-on SSB and OD-SSB are on the same frequency with the same spatial relation.

  - OD-SSB based SCell activation

In last meeting, RAN4 agreed to follow the legacy known/unknown Scell activation princinples. Based on current known condition in SCell activation, the UE needs to send a valid L3 measurement report (MR) for the SCell being activated. Thus, during the time period where the Scell is deactivated, UE needs to perform deactivated SCell measurement. After NW receiving the L3 MR, the SCell can be believed as a known SCell. When NW indicates the SCell activation command, UE is assumed to perform SCell activation based on known SCell requirement. In other words, the SCell is an unknown SCell if no L3 MR is transmitted to NW.

Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded.

RAN4 needs to discuss the impacts due to OD-SSB for these Scell activation requriements.

Aspect 1: which SSB will be used?

In last meeting, the issue related to aspect 1: which SSB will be used in Scell activation has been agreed, UE is required to always use OD-SSB for Scell activation. There are still other aspects will impact the Scell activation delay.

Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

Aspect 2: OD-SSB uncertainty

Currently, in legacy requirement, TFirstSSB, TFirstSSB_MAX are introduced to define the uncertainty to receive the first complete SSB burst after HARQ processing and MAC-CE parsing time. We think the similar uncertainty shall also be introduced in OD-SSB based SCell activation. UE needs to wait for the first complete OD-SSB burst after HARQ processing and MAC-CE parsing time.

Proposal 10: RAN4 to define the SCell activation requriement based on OD-SSB periodicity.

Aspect 3: whether L1-RSRP measurement is needed?

In legacy SCell activation procedure, if UE performs L3 measuremernt and reporting the quality within a valid time(such as 5s), the SCell is believed as a known cell. UE can perform SCell activation without L1-RSRP measurement. However, when UE performs the deactivated SCell measurement by always-on SSB or OD-SSB and NW activates the SCell by another SSB. UE may not know the TCI information of such OD-SSB if OD-SSBs and/or always-on SSB use different spatial filters. The issue is still being discussed in RAN1. RAN4 can wait RAN1’s further progress to clarify the spatial relation within OD-SSBs and/or with always-on SSB.

Proposal 11: RAN4 to wait RAN1’s further progress on spatial relation between OD-SSBs and always-on SSB.

Aspect 4: SSB-less SCell scenario

RAN4 also introduced the SSB-less SCell activation requirement for intra-band and inter-band collocated scenario . UE follows the SCell activation command to activate the SCell without SSB. In our understanding, UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation phase. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Proposal 12: UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Aspect 5: known cell condition

In last meeting, there is an agreement about the known cell condition as follow.

Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded.

Currently, the known cell condition is as follow.

FR1 SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. FR2 For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Currently, the known cell condition is assumed only one SSB is measured and used for Scell activation. Considering OD-SSB, the known cell condition shall be updated based on the OD-SSB and always-on SSB. For example, the reported SSB is based on always-on SSB, but the Scell activation is based on OD-SSB. On the contrary, the NW can indicate the OD-SSB for deactivated Scell measuremnet, but deactivated OD-SSB and only use the always-on SSB for Scell activation.

Proposal 13: In Case 2, RAN4 to discuss how to update the known cell condition since the measured SSB may be not the same SSB used in Scell activation.

- Summary

In this contribution, we have discussed the OD-SSB requirements. Based on the discussions, we have made following proposals and observations:

Observation 1: In legacy SCell activation, RAN4 followed the same RAN1 principle to apply SCell activation requirement based on slot n + THARQ+3msNR slot length.

Observation 2: The first steps of OD-SSB based SCell activation procedure is similar as legacy SCell activation.

Observation 3: UE can combine the measurement results if the always-on SSB and OD-SSB are on the same frequency with the same spatial relation.

Proposal 1: RAN4 to send LS to confirm that T_min equals to m+3Nslotsubframe,μ+1.

Proposal 2: RAN4 can further discuss the possible UE processing time for OD-SSB reception within RAN4 internally.

Proposal 3: L1 measurements are not supported for deactivated Scell. Instead, RAN4 can define OD-SSB based L1 measurement during unknown Scell activation or after the Scell activated.

Proposal 4: Legacy measurement accuracy requirements should also apply for OD-SSB.

Proposal 5: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

- Case 1: No always-on SSB on the cell

  - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

  - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [5s] and then the OD-SSB is transmitted again with the same spatial reception parameter

- Case 2: Always-on SSB on the cell

Proposal 6: The OD-SSB based fast L3 measurement for deactivated Scell should be as follow.

- The measurement is based on OD-SSB periodicity

- CSSF doesn’t need to be changed

- The delay requriement is Ceil(5 x Kp) x OD-SSB periodicity x CSSF

Proposal 7: The time window for fast L3 measurement for deactivated SCell is defined starting from UE receiving OD-SSB indication during deactivated Scell stage until the first time UE reports the target OD-SSB based de-activated Scell measurement result.

Proposal 8: UE is assumed to fallback to legacy deactivated Scell measurement based on measCycleSCell until NW deactivates the OD-SSB.

Proposal 9: In Case 2, the measurement requirement is defined assuming combining the legacy SSB and OD-SSB when OD-SSB transmission is indicated if the always-on SSB and OD-SSB are on the same frequency with the same spatial relation.

Proposal 10: RAN4 to define the SCell activation requriement based on OD-SSB periodicity.

Proposal 11: RAN4 to wait RAN1’s further progress on spatial relation between OD-SSBs and always-on SSB.

Proposal 12: UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Proposal 13: In Case 2, RAN4 to discuss how to update the known cell condition since the measured SSB may be not the same SSB used in Scell activation.

- References

- RP-242354, “Revised WID - Enhancements of network energy savings for NR”, Ericsson

- R4-2416916, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson

- 3GPP TR 38.864 V18.1.0, “Study on network energy savings for NR”

- Backup

The summary of SCell activation requirement impact due to the OD-SSB.

FR1

| # Case | # To-be-activated target SCell | # Rules | # Legacy Requirement | # OD-SSB impact |
| --- | --- | --- | --- | --- |
| # #1 | # FR1 known Scell with meas. period <= 2400ms | # Known, measurement period | # TFirstSSB | # Aspect 2 |
| # #2 | # FR1 known Scell with meas. period > 2400ms |  | # TFirstSSB_MAX+ Trs |  |
| # #3 | # FR1 unknown Scell with active serving cell(s) on the same band | # Unknown, with ssb-PositionInBurst, TCI indication | # TFirstSSB_MAX + TSMTC_MAX+ Trs | # Aspect 2 |
| # #4 | # FR1 unknown Scell without active serving cell(s) on the same band |  | # TFirstSSB_MAX + TSMTC_MAX+ 2Trs |  |
| # #5 | # FR1 unknown Scell without TCI indication in MAC-CE, and with SP CSI-RS for CSI reporting | # Purely Unknown | # TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) | # Aspect 2, 3 |
| # #6 | # FR1 unknown Scell without TCI indication in MAC-CE, and with P CSI-RS for CSI reporting |  | # 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay) |  |
| # #7 | # FR1 with active serving cell(s) on the same band | # SSB-less | # 3ms | # Aspect 4 |

FR2

| # Case | # To-be-activated target SCell | # Rules | # Legacy Requirement | # OD-SSB impact |
| --- | --- | --- | --- | --- |
| # #8 | # FR2 with active serving cell(s) on the same band with configured SMTC | # Active serving cell in the same band(SSB-less) | # TFirstSSB+ 5ms | # Aspect 2 |
| # #9 | # FR2 with active serving cell(s) on the same band but without configured SMTC, and the RS(s) of Scell being activated is QCL-TypeD with RS of one active serving cell on that FR2 band |  | # 3ms | # Aspect 4 |
| # #10 | # FR2 known Scell without active serving cell on the same band, and with SP-CSI-RS for CSI reporting | # First active FR2 Scell # P1: Known and unknown # P2: SP-CSI-RS, P-CSI-RS | # 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) | # Aspect 2 |
| # #11 | # FR2 known Scell without active serving cell on the same band, and with P-CSI-RS for CSI reporting |  | # max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ) |  |
| # #12 | # FR2 unknown Scell without active serving cell on the same band, and with SP-CSI-RS for CSI reporting |  | # 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) | # Aspect 2, 3 |
| # #13 | # FR2 unknown Scell without active serving cell on the same band, and with P-CSI-RS for CSI reporting |  | # 6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) |  |