# 3GPP TSG-RAN WG4 Meeting #113	R4-2419013
Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.6.2

Source: 	OPPO

Title: 	Discussion on reducing the number of UE Rx chains

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, how to reduce the number of UE Rx chains for fragmented carriers were discussed and some agreements were approved in WF [1]:

Agreements: Sub-topic 1-2: Study scope clarification - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below: - Any update of scope needs be decided by RAN Merge issue 2-0/2-1 discussion: - The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823) Issue 2-2: Study on power spectral density difference between carriers of co-located adjacent channel operators - To evaluate acceptable [PSD] difference from UE side between wanted signal and un-wanted signal based on the deployment where BSs from different operators are collocated but their radio units (RUs) are not shared. - The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD - Start evaluation with wanted signal level currently existing ACS requirements as baseline and to see the performance difference Open issues: Issue 2-3: Signaling related Issue 2-4: Discussion on semi-statically switch hardware resources

This contribution further discussed the application of fragmented carriers.

- Discussion

In last meeting, the concepts of fully shared Rx chain and partially shared RF chain were clarified and the related reference architectures were given in the WF. It’s now clear that the existing DL intra-band NC CA were implemented based on the partially-shared Rx architecture, and to reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers.

Currently, in NR Spec, the non-redcap UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for some bands where the UE is required to be equipped with a minimum of four Rx antenna ports. That is, the non-redcap UE will have one main receiver and one or more diversity receivers. In my understanding, the diversity receivers will reduce multi-Rx chains to one Rx chain when the main receiver adopt fully shared Rx chain to reduce the number of RX chains for fragmented carriers in the same band.

Proposal 1:  Fully shared Rx chain architecture to reduce the number of RX chains for fragmented carriers in the same band should be adopted by both of the main receiver and the diversity receiver(s).

In current Spec, there is no clear specification about how many Rx should be implemented for DL intra-band NC CA. and the Spec just clarify that the 2Rx and 4Rx requirements for the band shall applies when the band is used as a standalone band or as part of a band combination. Whether the fragmented carriers in the same band can only supporting 2Rx for the bands which are required 4Rx mandatorily is out of this SI scope, RAN4 doesn’t need discuss.

Proposal 2: Whether it is allowed to reduce the number of diversity receivers (reduce 4Rx to 2Rx) for the fragmented carriers is out of this SI scope, RAN4 doesn’t need discuss.

- Conclusion

This paper analyzed the application of fragmented carriers and proposed:

Proposal 1:  Fully shared Rx chain architecture to reduce the number of RX chains for fragmented carriers in the same band should be adopted by both of the main receiver and the diversity receiver(s).

Proposal 2: Whether it is allowed to reduce the number of diversity receivers (reduce 4Rx to 2Rx) for the fragmented carriers is out of this SI scope, RAN4 doesn’t need discuss.

Reference

[1] R4-2417201, WF on fragmented DL, MediaTek Inc., RAN4#112bis 2024-10