3GPP TSG-RAN WG4 Meeting #113                                         R4-2419033

Orlando, US, November 18 - 22, 2024

Source: 	Huawei, HiSilicon

Title: 	On general aspects for SBFD

Agenda Item:	7.20.1

Document for:	Approval

Introduction

During RAN4#112bis, general aspects (including RAN4 aspects for SBFD system parameters) for SBFD have been discussed. Following conclusions are reproduced from [1].

##### Minimum channel bandwidth for SBFD (X MHz) Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = 40 as the default value - For band n39, X = 20MHz is used instead. ##### Specification implementation for band/channel bandwidth support for SBFD Agreement: - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD. - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD - FFS the band for NR-U shall be supported ##### Transmission bandwidth configuration for SBFD Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues. ##### DL/UL subband configurations support for SBFD Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case. ##### BS RF Specification structure for SBFD requirements Way Forward: - FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable. - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements - Option 3: A new specification for SBFD BS RF requirements

In this contribution, we would like to share our views regarding this topic.

Discussion

To be further discussed, one of the hot topics in last meeting is subband configuration for SBFD. In addition, we would like to share our preference on specification structure regarding how to capture the SBFD feature.

The guard band and subband configuration for SBFD

To specify DL/UL subband configuration in BS RF requirements, it was agreed to further study whether additional restrictions can be introduced to Option 2b.

Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

Per our understanding, the Alternative 1 listed in the WF is trying to constrain manufacture implementation badly so eventually only one DL subband size for an indicated channel bandwidth per band is allowed. Besides, the proposed ratio for UL subband size over channel bandwidth will restrict the implementation choice further.

During the Rel-18 SBFD SI, extensive analysis and discussion regarding the interference caused by simultaneous Tx/Rx at gNB side have been conducted. To guarantee the feasibility of SBFD, multiple techniques in RF domain have been proposed. When it comes to Rel-19, parallel discussion on the RF requirements which can guarantee the performance of SBFD deployment is ongoing.

Another point is if RAN4 adopts a ratio e.g. 20% for UL subband size over channel bandwidth, then for 100MHz channel bandwidth the 20MHz UL subband size could be supported. But for the BS that is capable of SBFD and also multi-carrier operation (SBFD operation only for one carrier), such ratio will rule out the support of 20MHz size when the RF bandwidth is e.g. 200MHz.

Thus we think from technical perspective there is no reason to have such restriction also because the support of SBFD is anyway optional.

Last but not the least, operator demand is another important factor that cannot be ignored. In conclusion we think the Alternative 2 is a preferable direction to move forward, while we can offer a compromise which is only introducing some of the bandwidth configurations from Table 5.3.2-1 for UL, e.g. taking 10MHz as a granularity then 10MHz, 20MHz, 30MHz, 40MHz, 50MHz, 60MHz, 70MHz, 80MHz, 90MHz are selected.

Proposal 1: The number of the declared DL subband sizes should not be limited to one.

Proposal 2: As a compromise, only introduce the following bandwidth from Table 5.3.2-1 for SBFD UL subband configuration:

- 10MHz, 20MHz, 30MHz, 40MHz, 50MHz, 60MHz 70MHz, 80MHz and 90MHz.

BS RF Specification structure for SBFD requirements

According to the WF, RAN4 will further study how to capture the SBFD feature in the TS 38.104. Like we have proposed before, we prefer to embed the corresponding SBFD RF requirements in the related section of TS 38.104 as much as possible (e.g. RAN4 already agreed to embed the supporting band of SBFD in clause 5) and consider new clauses or new suffix for SBFD for the RF requirements that are necessary.

Proposal 3: Embed the corresponding SBFD RF requirements in the corresponding section of TS 38.104 as much as possible (e.g. RAN4 already agreed to embed the supporting band of SBFD in clause 5) and consider new clauses or new suffix for SBFD for the RF requirements that are necessary.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following proposals:

Proposal 1: The number of the declared DL subband sizes should not be limited to one.

Proposal 2: As a compromise, only introduce the following bandwidth from Table 5.3.2-1 for SBFD UL subband configuration:

- 10MHz, 20MHz, 30MHz, 40MHz, 50MHz, 60MHz 70MHz, 80MHz and 90MHz.

Proposal 3: Embed the corresponding SBFD RF requirements in the corresponding section of TS 38.104 as much as possible (e.g. RAN4 already agreed to embed the supporting band of SBFD in clause 5) and consider new clauses or new suffix for SBFD for the RF requirements that are necessary.

- References

- R4-2416584, “Way Forward for [112bis][306] NR_duplex_evo_General”, Samsung, 3GPP TSG-RAN4 Meeting #112bis, Hefei, China, October 14-18, 2024.